| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0248-v1 |
| --- | --- |

| Title: | TemporalId restrictions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

A basic syntax and semantics for supporting temporal layers was adopted at the 11th meeting in Ljubljana. However, the proponents of this contribution claim that the VVC draft can be improved by adding some additional definitions and restrictions. It is proposed to add text from HEVC adapted for the VVC high-level syntax.

The following elements are proposed to be added to the VVC draft:

- Definition of sub-bitstream extraction process

- The following restriction to the semantics of slice_pic_parameter_set_id: “It is a requirement of bitstream conformance that the value of TemporalId of the PPS that has pps_pic_parameter_set_id equal to slice_pic_parameter_set_id shall be less than or equal to the value of TemporalId of the current picture.”

- The sub-bitstream extraction process with restrictions as shown in italics below

Inputs to this process are a bitstream and a target highest TemporalId value tIdTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance that any output sub-bitstream that is the output of the process specified in this clause with tIdTarget equal to any value in the range of 0 to 6, inclusive, shall be a conforming bitstream and fulfill the following:

- The output sub-bitstream shall contain at least one VCL NAL unit.

- The decoded sample values of a picture shall be identical for any value of tIdTarget in the range of 0 to tId, where tId is the TemporalId of the picture.

The output sub-bitstream is derived as follows:

–	Remove all NAL units with TemporalId greater than tIdTarget.

# Introduction

A basic syntax and semantics for supporting temporal layers was adopted at the 11th meeting in Ljubljana. However, the proponents of this contribution claim that the VVC draft can be improved by adding some additional definitions and restrictions. It is proposed to add text from HEVC adapted for the VVC high-level syntax.

The following elements are proposed to be added to the VVC draft:

- Definition of sub-bitstream extraction process

- The following restriction to the semantics of slice_pic_parameter_set_id: “It is a requirement of bitstream conformance that the value of TemporalId of the PPS that has pps_pic_parameter_set_id equal to slice_pic_parameter_set_id shall be less than or equal to the value of TemporalId of the current picture.”

- A sub-bitstream extraction process with restrictions

# Definitions, sub-layers and bitstream extraction

The main purpose of temporal layers is to assign coded pictures to temporal layers and support layer extraction. For this to work, we propose to add some definitions and additional constraints from HEVC. Also, we propose to include a simplified HEVC sub-bitstream extraction process into the VVC draft.

## Proposed definition to add to VVC

  - sub-bitstream extraction process: A specified process by which NAL units in a bitstream, determined by a target highest TemporalId, are removed from the bitstream, with the output sub-bitstream consisting of the NAL units in the bitstream having a TemporalID equal to or lower than the target highest TemporalID.

## Proposed constraint for slice_pic_parameter_set_id

Since PPS NAL units may be larger than 0, we propose to restrict that the TemporalId of a slice must be equal to or higher than the TemporalId of the PPS it refers to.

We propose to add the following text to slice_pic_parameter_set_id:

slice_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use. The value of slice_pic_parameter_set_id shall be in the range of 0 to 63, inclusive. It is a requirement of bitstream conformance that the value of TemporalId of the PPS that has pps_pic_parameter_set_id equal to slice_pic_parameter_set_id shall be less than or equal to the value of TemporalId of the current picture.

For information, the corresponding HEVC version 5 text looks as follows:

“Let activePPS be the PPS that has pps_pic_parameter_set_id equal to slice_pic_parameter_set_id, and let activeSPS be the SPS that has sps_seq_parameter_set_id equal to pps_seq_parameter_set_id of activePPS. It is a requirement of bitstream conformance that the following constraints apply:

- The value of TemporalId of activePPS shall be less than or equal to the value of TemporalId of the current picture.

- The value of nuh_layer_id of activePPS shall be less than or equal to the value of nuh_layer_id of the current picture.

- The value of nuh_layer_id of activeSPS shall be less than or equal to the value of nuh_layer_id of the current picture.”

## A sub-bitstream extraction process with proposed constraint

In order to make temporal layers work, a rule that a picture can only refer to pictures of lower TemporalId is needed. In this contribution we propose to address that by including a sub-bitstream extraction process and express the constraints there. The main reason for that is that the VVC specification currently does not contain any text for reference picture handling, such as RPS, where restrictions may be expressed.

We propose to add the following text:

# 10 Sub-bitstream extraction process

Inputs to this process are a bitstream and a target highest TemporalId value tIdTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance that any output sub-bitstream that is the output of the process specified in this clause with tIdTarget equal to any value in the range of 0 to 6, inclusive, shall be a conforming bitstream and fulfill the following:

–	The output sub-bitstream shall contain at least one VCL NAL unit.

–	The decoded sample values of a picture shall be identical for any value of tIdTarget in the range of 0 to tId, where tId is the TemporalId of the picture.

The output sub-bitstream is derived as follows:

–	Remove all NAL units with TemporalId greater than tIdTarget.

For information, the corresponding HEVC version 5 text looks as follows:

# “10 Sub-bitstream extraction process

Inputs to this process are a bitstream, a target highest TemporalId value tIdTarget and a target layer identifier list layerIdListTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitsteam, tIdTarget equal to any value in the range of 0 to 6, inclusive, and layerIdListTarget either equal to the layer identifier list associated with a layer set specified in the active VPS or consisting of all the nuh_layer_id values of the VCL NAL units present in the input bitstream as inputs, and that satisfies both of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in layerIdListTarget.

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE 1 – A bitstream conforming to a profile specified in Annex A contains one or more coded slice segment NAL units with nuh_layer_id equal to 0.

NOTE 2 – A conforming bitstream contains one or more coded slice segment NAL units with TemporalId equal to 0.

The output sub-bitstream is derived as follows:

–	When one or more of the following two conditions are true, remove all SEI NAL units that have nuh_layer_id equal to 0 and that contain a non-scalable-nested buffering period SEI message, a non-scalable-nested picture timing SEI message, or a non-scalable-nested decoding unit information SEI message:

–	layerIdListTarget does not include all the values of nuh_layer_id in all NAL units in the bitstream.

–	tIdTarget is less than the greatest TemporalId in all NAL units in the bitstream.

NOTE 3 – A "smart" bitstream extractor may include appropriate non-scalable-nested buffering picture SEI messages, non-scalable-nested picture timing SEI messages and non-scalable-nested decoding unit information SEI messages in the extracted sub-bitstream, provided that the SEI messages applicable to the sub-bitstream were present as scalable-nested SEI messages in the original bitstream.

–	Remove all NAL units with TemporalId greater than tIdTarget or nuh_layer_id not among the values included in layerIdListTarget.”

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).