| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0250r1 |
| --- | --- |

| Title: | CE3: 6 MPM with truncated binary code for non-MPM and CTU-row constraint (Test 6.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Ted Hsieh Nan Hu Vadim Seregin Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com geertv@qti.qualcomm.com thsieh@qti.qualcomm.com nanh@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports the results of the sub-test 6.1.1 of CE3 Intra Prediction and Mode coding as described in JVET-K1023. The features tested include 6 most probable modes (MPMs), with MPMs coded with truncated unary binarization and the first three bins being context coded, and non-MPM modes coded with truncated binary code; in addition, a CTU-row constraint is enabled whereby modes of above neighbour modes (above, above-left and above-right) are not used in the generation of the MPM when the above neighbouring modes belong to a different CTU row than the current block. The RD checks are the same as that of the anchor. It is reported that the tested method has a BD-rate performance of (−0.42%, −0.40%, −0.35%) for (Y, Cb, Cr) with runtimes of 102% (Enc) and 102% (Dec) for the AI configuration and (−0.17%, −0.13%, −0.16%) for (Y, Cb, Cr) for the RA configuration with runtimes of 100% (Enc) and 99% (Dec).

In revision 1 of this document, a table containing some complexity measures of the algorithm are added.

# Introduction

VTM-2.0.1 uses 67 intra modes and 3 MPMs in the intra coding of a block. The MPMs are derived from a combination of the left and above neighbouring blocks (when available), default modes (e.g., planar, DC, etc.) and modes derived from the neighbouring blocks (e.g., offset by 1). The 3 MPMs are coded by truncated unary coding (one or two bits) and the remaining 64 modes are coded using bypass coding with 6 bins. The number of RD checks is determined by the block size and whether the left and above neighbouring intra directions are equal.

# Proposed test

The proposed test increases the number of MPMs to 6, and the derivation of the MPMs is a combination of intra mode used by five neighbouring blocks (left, above, below-left, above-right and above-left), default modes (e.g., planar, DC, etc.) and angular modes derived from neighbouring blocks using offset of +1 and −1. Truncated unary binarization is used for the MPM index, and the first three bins are context coded; the context of the i-th bin, i = 0, 1, or 2, depends on the i-th MPM mode; a total of three contexts are used. The remaining modes (61 in number) are coded using truncated binary coding. The full-RD checks are the same as that of the anchor. Above, above-left and above-right neighbouring blocks that belong to a different CTU row than the current block are considered unavailable for MPM derivation.

# Results

The simulations according to the CE3 test conditions show that the proposed method has a BD-rate performance of (−0.42%, −0.40%, −0.35%) for (Y, Cb, Cr) with runtimes of 102% (Enc) and 102% (Dec) for the AI configuration and (−0.17%, −0.13%, −0.16%) for (Y, Cb, Cr) for the RA configuration with runtimes of 100% (Enc) and 99% (Dec).

Table 1: Results for CE3 sub-test 6.1.1 w.r.t. BMS-2.0.1 in VTM configuration

The following table gives complexity measures of the MPM-generation algorithm described in this document.

| Test | Max number of neighbors to access | Line buffer required? | Max layers of if conditions | Max number of comparison operator | Max number of logical operators | Max number of assignment operators | Max number of increment | Max number of bit operation | Parsing dependency? | Number of Context modeling for MPM coding | number of full RDO checks | Has LUT? | LUT size | Number of condition check for remaining modes | Non-MPM coding |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1. | 5 | N | 4 | 33 | 11 | 35 | 6 | 0 | Y | 4 | 1 or 2 | Y | 68 | 1 | TB |

# Additional results and discussion

The gain of the proposed algorithm comes from several features: addition of 3 MPMs (i.e., extending 3 MPMs to 6 MPMs), the MPM derivation algorithm, additional neighbouring modes, use of context coding for the bins of the MPM index syntax element, and using truncated binary coding for the remaining modes.

To understand the impact of one of these factors, the addition of 3 MPMs, we tested a simple extension of the anchor (which contains 3 MPMs) by appending 3 additional MPMs. The first 3 MPMs do not change from the anchor. The MPM index is coded using truncated unary coding (no contexts) and the binarization of the remaining modes is unchanged (6-bit bypass) w.r.t. the anchor, thereby only showing the gain a simple extension from 3 to 6 MPMs. The gain obtained is as follows:

Around 0.30% comes from the combination of the modified MPM derivation algorithm, additional neighbouring modes, use of context coding for the bins and the use of truncated binary coding (instead of 6-bit bypass coding) for the remaining modes.

# Acknowledgements

We would like to thank Canon for cross-checking the CE test.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).