| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0252 |
| --- | --- |

| Title: | CE10.2.2: Overlapped block motion compensation (OBMC) early termination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment (CE) 10.2.2 on early termination for overlapped block motion compensation (OBMC). In this CE test, the OBMC design in the joint exploration model (JEM)-7.0 is used as the base. Additionally, one early termination method is applied to skip some motion compensation processes based on the difference between the motion vectors (MVs) of the current block/sub-block and its spatial neighbors. Simulation results show that compared to the VTM-2.0.1 anchors, with the proposed early termination method, the average {Y, U, V} BD-rate savings of {1.04%, 1.95%, 2.13%} and {1.43%, 2.27%, 2.23%} for RA and LD, respectively. The average encoding and decoding time are 106% and 108% for RA, and 108% and 111% for LD.

# Introduction

Overlapped block motion compensation (OBMC) is one coding tool in joint exploration model (JEM)-7.0 [1], which reduce the blocking artifacts at the motion compensation stage by combining multiple prediction signals using the motion vectors (MVs) of spatial neighboring blocks of the current coding unit (CU). Although it can improve the coding efficiency, the OBMC also introduces complexity increase to both encoder and decoder. To reduce the OBMC complexity, one early termination method was proposed in [2] that use the similarity between the MV of one current block and the MV of its spatial neighboring block to bypass some motion compensations during the OBMC. The method was included in Core Experiment (CE) 10.2.2 for further evaluation [3]. This document reports the simulation results of the CE test.

# Technical description

Some motion compensations are skipped during the OBMC process based on the similarity between the MVs of one current block and one of its spatial neighboring block. Specifically, each time before motion compensation is invoked using the motion information from a given neighboring block, the neighboring MV(s) is compared to the current block’s MV(s) based on the following three conditions:

- If all the prediction lists (i.e., either L0/L1 in uni-prediction or both L0 and L1 in bi-prediction) that are used by the neighboring block are also used for the prediction of the current block;

- If the same reference picture(s) are used by the neighboring MV(s) and the current MV(s);

- If MV difference between the neighboring MV(s) and the current MV(s) is no larger than pre-defined MV difference threshold (both L0 and L1 MVs are checked if bi-pred is used).

If all the 3 conditions are met, then motion compensation is not performed (this is applicable to luma and chroma) using that given neighboring motion. Additionally, to achieve a better performance/complexity trade-off, two different threshold values i.e., 1.5-pel and 1-pel, are applied when the neighboring block is uni-predicted or bi-predicted.

# Simulation results

The proposed method was implemented based on the BMS-2.0.1 reference software [4] and tested using the VTM configuration. The test condition and evaluation criteria specified in CE10 [3] are used for the simulations.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -1.10% | -1.32% | 104% | 104% |
| Class A2 | -1.26% | -2.18% | -2.23% | 106% | 111% |
| Class B | -0.84% | -2.00% | -2.06% | 107% | 107% |
| Class C | -1.55% | -2.36% | -2.74% | 107% | 112% |
| Class E |  |  |  |  |  |
| Overall | -1.04% | -1.95% | -2.13% | 106% | 108% |
| Class D | -1.39% | -2.40% | -2.36% | 108% | 117% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.09% | -1.99% | -2.11% | 108% | 109% |
| Class C | -1.79% | -2.41% | -2.87% | 109% | 114% |
| Class E | -1.51% | -2.56% | -1.59% | 107% | 110% |
| Overall | -1.43% | -2.27% | -2.23% | 108% | 111% |
| Class D | -1.81% | -2.24% | -2.85% | 110% | 118% |

# References

J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of joint exploration test mode (JEM7)”, JVET-G1001, August. 2017, Torino, Italy.

X. Xiu, Y. He, Y. Ye, “CE10.2.2: Complexity reduction for over-lapped block motion compensation (OBMC)”, JVET-K0345, July 2018, Ljubljana, Slovenia.

C.-W. Hsu, M. Winken, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction”, JVET-K1030, July 2018, Ljubljana, Slovenia.

BMS-2.0.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).