| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0277 |
| --- | --- |

| Title: | CE3: Simplifications for chroma intra coding (Test 2.3.1 and 2.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of CE3-2.3 on simplifications for chroma intra coding, including test CE3-2.3.1 and CE3-2.3.2.

Introduction

In VTM-2.0.1 [1], there are 6 chroma intra coding modes, including LM, DM and 4 additional angular intra prediction modes. When the chroma intra prediction mode is LM, the reconstructed pixels in the co-located luma component is used to generate the prediction samples of current chroma block. When the chroma intra prediction mode is DM, the intra prediction mode of the co-located luma component is used. Otherwise, the default modes, including Planar, Vertical, Horizontal and DC, are added. In case of DM being equal to any of the default modes, that default mode is replaced by VDIA_IDX (mode 34).

Proposed method

In JVET-K0293 [2], it was proposed to only use DM and LM modes for certain block sizes. In test CE3-2.3.1, only DM and LM modes are used for 2xN or Nx2 block sizes. In test CE3-2.3.2, only DM and LM modes are used for all block sizes.

Experimental results

The proposed method is implemented on top of BMS2.0.1 [1] software. The simulations are performed on Linux cluster with VTM AI and RA configurations under the common test condition described in JVET-K1010 [3].

Table 1: Coding performance of test CE3-2.3.1 over VTM-2.0.1

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.00% | 96% | 93% | -0.02% | 0.06% | 0.16% | 96% | 96% |
| Class A2 | 0.01% | 0.34% | 0.17% | 93% | 93% | 0.03% | 0.32% | 0.14% | 95% | 96% |
| Class B | 0.00% | 0.23% | 0.33% | 95% | 94% | 0.04% | 0.09% | 0.29% | 96% | 99% |
| Class C | -0.01% | 0.39% | 0.76% | 94% | 96% | 0.04% | 0.27% | 0.63% | 92% | 87% |
| Class E | 0.02% | 0.08% | 0.17% | 96% | 96% |  |  |  |  |  |
| Overall | 0.00% | 0.23% | 0.32% | 95% | 95% | 0.02% | 0.18% | 0.32% | 94% | 94% |
| Class D | 0.00% | 0.77% | 0.61% | 91% | 81% | 0.06% | 0.58% | 0.75% | 92% | 88% |
| Class F | 0.01% | 0.26% | 0.48% | 94% | 95% | 0.01% | 0.18% | 0.45% | 93% | 91% |

Table 2: Coding performance of test CE3-2.3.2 over VTM-2.0.1

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.00% | 0.67% | 92% | 92% | -0.02% | 0.57% | 0.41% | 93% | 96% |
| Class A2 | 0.10% | 1.44% | 1.10% | 88% | 94% | 0.05% | 1.11% | 0.72% | 94% | 95% |
| Class B | 0.05% | 1.20% | 1.44% | 91% | 94% | 0.03% | 0.77% | 1.16% | 94% | 98% |
| Class C | 0.04% | 1.13% | 1.73% | 90% | 97% | 0.05% | 0.82% | 1.28% | 90% | 87% |
| Class E | 0.21% | 1.56% | 2.29% | 91% | 96% |  |  |  |  |  |
| Overall | 0.08% | 1.25% | 1.46% | 91% | 95% | 0.03% | 0.81% | 0.95% | 92% | 94% |
| Class D | 0.05% | 1.48% | 1.34% | 88% | 82% | 0.02% | 0.79% | 1.03% | 90% | 87% |
| Class F | 0.11% | 1.03% | 1.31% | 89% | 95% | 0.09% | 0.81% | 0.99% | 90% | 91% |

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS

- L. Zhao, X. Zhao, X. Li, and S. Liu “CE3 related: Simplifications for chroma intra coding,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K293, Ljubljana, SI, 10–18 Jul. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1010, Ljubljana, SI, 10–18 Jul. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).