| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 08–12 Oct. 2018 | Document: JVET-L0282 |
| --- | --- |

| Title: | CE4-related: Merge List Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email: | 1-858-635-5300 seethal.paluri@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

Abstract

Merge list construction relies on examining a variety of candidates including up to 5 spatial, 2 temporal, 1 sub-PU, 12 combined Bi-prediction and zero motion candidates. Due to the complexity of evaluating the validity of each type of candidate before being inserted into the Merge list, simplification methods to reduce computational complexity are highly desirable. In this proposal, we present two forms of Merge list simplification. First, we suggest to limit the number of motion data comparisons for spatial candidates and temporal candidate, and secondly, we limit the number of combined Bi-prediction candidates from 12 to 4. Therefore, our proposed simplifications reduce the complexity. Experiments performed on RA and LDB under common test conditions shows no noticeable change in coding performance. The simplification to reduce the number of comparisons results in (0.01/0.06/-0.01)% and (0.02/-0.11/-0.11)% for RA and LDB respectively, while the simplification for limiting the number of combined bi-prediction candidates results in (0.03/0.06/0.02)% and (0.03/0.14/-0.06)% under the same test conditions.

# Background

## Motion data comparisons

Merge list construction in VTM 2.0.1 allows for 6 Merge candidates. These candidates can be chosen from a combination of 5 spatial candidates, 2 temporal, 1 sub-PU, 12 Bi-pred and zero motion candidates. Each type of candidate brings coding value to the motion compensation process at the expense of increased computational complexity. This is specifically true for the temporal, sub-PU and bi-pred candidates. Motion data comparisons are performed during the list construction to ensure that no candidate with redundant motion data enters the list. If M is the number of spatial candidates, a complete redundancy check would consist of (M(M-1))2 motion data comparisons. In case of 5 spatial candidates, in the worst case, 10 motion data comparisons are needed to ensure all the candidates in the list have distinct motion.

## Bi-Prediction Candidates

In bi-predictive slices, additional candidates can be generated based on the existing ones by combining the reference picture list 0 motion data of one candidate with the list 1 motion data of another one. This is done by copying Δx0, Δy0, Δt0 from one candidate and Δx1, Δy1, Δt1 from another. The different combinations are predefined and shown in the table below [2].

Table 1.0: Shows the Bi-Prediction candidate order.

Combined Bi-prediction candidates are typically computationally complex as each list has to be scanned for the available candidate combinations in both the lists before it is used.

# Proposed simplification

## Limiting the number of motion data comparisons

In this proposal we suggest to impose a limit to the number of comparisons for 5 spatial candidates during the Merge list construction. As mentioned in Section 1.1, a full redundancy check for 5 spatial candidates would require 10 motion data comparisons. However, the proposed scheme would only require 5 motion data comparisons. This can be achieved by disabling the macros HM_JEM_MERGE_CAND in TypeDef.h. Enabling this macro utilizes the full redundancy check in the worst case scenario. However, disabling this macro limits the number of comparisons from 10 to 5 as in HEVC, i.e. for spatial candidate order of {A1, B1, B0, A0, and B2}, B1 checks A1, while B0 only checks B1, A0 only checks A1, and B2 only checks A1 and B1. By disabling the macro HM_JEM_MERGE_CAND, the redundant check for the temporal candidate is also removed.

Fig. 1 Positions of spatial candidates

## Restricting the number of Combined Bi-prediction candidates

In another simplification to the Merge list, we propose that the number of combined Bi-prediction candidates be limited to 4. Data analysis of the frequency distribution of the different Bi-prediction combination orders show that the combination orders; 0, 1, 2, and 3 occur most often with the frequencies of 33%, 31%, 19% and 9% respectively. The remaining combination orders constitute of the 8%. Therefore, we propose to limit the number of Bi-prediction candidates to 4.

# Experimental Results

Each of the proposed method of simplification has been implemented on VTM 2.0.1 reference software and evaluated experimentally according to common test conditions defined in JVET-J1010 [2]. The results were generated for the entire sequence.

Table 2.0: Experimental results for random access test condition for disabling the HM_JEM_MERGE_CAND macros; anchor is VTM 2.0.1.

| RA | Over VTM 2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01 | 0.08 | 0.06 | 100 | 100 |
| Class A2 | 0.01 | -0.06 | -0.07 | 100 | 100 |
| Class B | 0.01 | 0.11 | -0.02 | 100 | 100 |
| Class C | 0.01 | 0.09 | 0.00 | 101 | 100 |
| Class E |  |  |  |  |  |
| Overall | 0.01 | 0.06 | -0.01 | 100 | 100 |
| Class D | 0.03 | -0.04 | 0.04 | 100 | 99 |

Table 3.0: Experimental results for low delay B test condition for disabling the HM_JEM_MERGE_CAND macros; anchor is VTM 2.0.1.

| LD | Over VTM 2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04 | -0.06 | -0.06 | 95 | 96 |
| Class C | 0.05 | 0.05 | -0.08 | 97 | 99 |
| Class E |  |  |  |  |  |
| Overall | 0.02 | -0.11 | -0.11 | 97 | 99 |
| Class D | 0.04 | 0.48 | -0.27 | 101 | 100 |

Table 4.0: Experimental results for random access test condition for restricting the number of Bi-prediction candidates to 4; anchor is VTM 2.0.1.

| RA | Over VTM 2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00 | 0.06 | -0.07 | 99 | 96 |
| Class A2 | 0.07 | 0.01 | 0.02 | 98 | 95 |
| Class B | 0.02 | 0.05 | 0.00 | 100 | 97 |
| Class C | 0.04 | 0.09 | 0.11 | 99 | 94 |
| Class E |  |  |  |  |  |
| Overall | 0.03 | 0.06 | 0.02 | 99 | 96 |
| Class D | 0.06 | 0.00 | 0.07 | 98 | 81 |

Table 5.0: Experimental results for low delay B test condition for restricting the number of Bi-prediction candidates to 4; anchor is VTM 2.0.1.

| LD | Over VTM 2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06 | 0.23 | -0.06 | 99 | 97 |
| Class C | 0.06 | 0.09 | -0.06 | 98 | 93 |
| Class E | -0.05 | 0.05 | -0.07 | 100 | 97 |
| Overall | 0.03 | 0.14 | -0.06 | 99 | 96 |
| Class D | 0.07 | 0.41 | 0.12 | 98 | 86 |

# Conclusion

We proposed 2 simplifications for Merge list construction. We observed no noticeable loss in the coding performance, even when the worst case number of comparisons were reduced by 50%. Similarly, we proposed that the number of Bi-prediction candidates be restricted to 4 without any noticeable loss in coding performance.

# Reference

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

- V. Sze, M. Budagavi, G. J. Sullivan, “High Efficiency Video Coding (HEVC)”, 2014.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).