| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0288 |
| --- | --- |

| Title: | CE6: Coupled primary and secondary transform (Test 6.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-3.2 on coupled primary and secondary transform. Instead of signaling primary and secondary transform independently, with the proposed method, primary and secondary transform are coupled and signaled jointly by one transform index. It is reportedly shown that, on top of VTM-2.0.1, average -0.50%, -0.24% and -0.17% luma BD-Rate is achieved under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively, with almost no run-time impact.

# Introduction

In BMS-2.0.1, the primary (MTS) and secondary transforms (NSST) are signaled and used independently. Since both MTS and NSST allow multiple candidates to be selected at the encoder side, checking all possible combinations of MTS and NSST significantly increases encoder run-time.

# Proposed method

In CE6-3.2, it is proposed to couple MTS and NSST, so that the usage of the two tools is indicated by a single transform index at the encoder. The proposed method is illustrated in Figure 1.

Figure 1: Proposed coupled primary and secondary transform (CPST).

The transform selection is indicated by the intra prediction mode and signaled transform index (mts_idx). The method for selecting transform type is unchanged and kept as same as the original design of MTS in BMS-2.0.1. When transform index (mts_idx) is 0, only DST-7 is applied as both horizontal and vertical transform, and secondary transform is not applied, otherwise, a secondary transform set is indicated by the intra prediction mode and the NSST core is further identified by the NSST index. The proposed method can be applied on luma only, or both luma and chroma. When the proposed method is applied on chroma and LM mode is being used, DCT-2 is used as the primary transform and secondary transforms are selected from the transform set associated with Planar mode and the transform index is implicitly derived using the mts_idx value of co-located luma block.

# Results

The proposed scheme is implemented on top of BMS-2.0.1 software, and simulations are performed using VTM configurations under common test condition defined in [2]. Three tests are defined in CE6 document, as described below.

Table 1: Descriptions on CE6-3.2 tests.

| Test # | Description |
| --- | --- |
| 6.3.2-a | Coupled primary and secondary transform for luma only, and secondary transform based on 4x4 NSST |
| 6.3.2-b | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on 4x4 NSST |
| 6.3.2-c | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on 4x4 NSST, with reduced transform sets based on grouping intra prediction modes |

Detailed test results are reported in the following tables.

Table 2: Coding performance of CE6-3.2-a over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.14% | -0.07% | 100% | 102% | -0.10% | 0.07% | 0.05% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.23% | -0.11% | -0.18% | 100% | 102% | -0.13% | 0.02% | -0.03% | 100% | 100% |  |  |  |  |  |
| Class B | -0.41% | -0.36% | -0.39% | 100% | 101% | -0.23% | -0.15% | -0.11% | 100% | 100% | -0.07% | -0.16% | 0.01% | 100% | 103% |
| Class C | -0.81% | -0.77% | -0.62% | 97% | 100% | -0.42% | -0.25% | -0.23% | 100% | 100% | -0.12% | -0.11% | -0.38% | 100% | 99% |
| Class E | -0.56% | -0.47% | -0.50% | 101% | 101% |  |  |  |  |  | -0.25% | 0.04% | -1.81% | 99% | 98% |
| Overall | -0.46% | -0.39% | -0.37% | 100% | 101% | -0.23% | -0.10% | -0.10% | 100% | 100% | -0.13% | -0.09% | -0.57% | 99% | 100% |
| Class D | -0.79% | -0.54% | -0.81% | 98% | 101% | -0.34% | -0.58% | -0.05% | 100% | 99% | -0.07% | 0.49% | 0.05% | 101% | 101% |
| Class F | -0.64% | -0.78% | -0.69% | 98% | 102% | -0.40% | -0.24% | -0.20% | 100% | 101% | -0.21% | -0.06% | -0.41% | 101% | 100% |

Table 2: Coding performance of CE6-3.2-b over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.17% | -0.19% | 101% | 98% | -0.16% | -0.01% | -0.15% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.27% | -0.30% | -0.29% | 101% | 100% | -0.12% | -0.17% | -0.24% | 100% | 100% |  |  |  |  |  |
| Class B | -0.46% | -0.73% | -0.78% | 101% | 100% | -0.24% | -0.77% | -0.90% | 100% | 100% | -0.09% | -0.61% | -0.63% | 100% | 103% |
| Class C | -0.85% | -1.16% | -1.13% | 98% | 102% | -0.41% | -0.59% | -0.58% | 100% | 99% | -0.14% | -0.37% | -0.40% | 100% | 99% |
| Class E | -0.61% | -0.64% | -0.82% | 102% | 100% |  |  |  |  |  | -0.34% | -0.57% | -1.84% | 100% | 99% |
| Overall | -0.50% | -0.65% | -0.68% | 101% | 100% | -0.24% | -0.45% | -0.53% | 100% | 100% | -0.17% | -0.52% | -0.86% | 100% | 101% |
| Class D | -0.86% | -0.85% | -1.19% | 99% | 104% | -0.35% | -0.59% | -0.73% | 100% | 102% | -0.08% | 0.22% | 0.09% | 101% | 100% |
| Class F | -0.66% | -0.91% | -0.77% | 99% | 102% | -0.45% | -0.49% | -0.52% | 101% | 100% | -0.18% | -0.41% | -0.06% | 100% | 100% |

Table 2: Coding performance of CE6-3.2-c over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Complexity Analysis

## Memory requirements for storing transform cores

Comparisons on the memory requirement for storing secondary transform cores are described in Table 6.

Table 6: Memory requirements for storing secondary transform cores

| CE test | Memory cost for storing NSST cores |
| --- | --- |
| CE6-3.2-a | 35*3*256 = 26.88 KB |
| CE6-3.2-b | 35*3*256 = 26.88 KB |
| CE6-3.2-c | 13*3*256 = 9.984 KB |

## Other metrics

| Metrics | Reported results |
| --- | --- |
| Minimum bit-precision | Unchanged |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output | No |
| Other operations and memory requirements relevant to the proposed tool | No |
| Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided | Reported in Section 3 |
| List of all combinations of transforms block size and transform type used for secondary transformation. | Same with BMS-2.0.1 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms | Proposed method applies matrix multiplication based NSST |
| If the proposal requires additional computations at the encoder or decoder | No |

# Conclusions

This proposal provides a method of coupled primary and secondary transform signaling. The experimental results show average -0.50%, -0.24% and -0.17% luma BD-Rate is achieved under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively. It is recommended to further study this proposed scheme in CE.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).