| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0290-v1 |
| --- | --- |

| Title: | CE8: CPR mode with dual-tree support (Test CE8.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the CE test results for CE8.2. Specifically, in CE8.2, results of the difference between CPR mode with and without dual-tree support are reported. It is reported that with dual-tree support turned off, the BD rate changes over VTM2 are:

- In AI, 1.17%/-16.62%/-53.19% for CTC/Class F/SCC 1080p classes, separately.

- In RA, 0.02%/-14.44%/-35.61% for CTC/Class F/SCC 1080p classes, separately.

- In LDB, 0.08%/-8.52%/-25.25% for CTC/Class F/SCC 1080p classes, separately.

# Introduction

In last JVET meeting, CPR mode [1] was adopted into BMS software for further evaluation. At the same time, separate coding trees for luma and chroma in I slice was adopted in to VTM. The interaction of CPR mode and dual-tree structure is reported in CE8.2. When dual-tree is enabled for a P slice where the current picture is the only reference picture, the following applies to chroma coding units:

- Full coverage: for a current chroma coding unit, if all its corresponding luma sub-blocks are coded in CPR mode, the current chroma CU can choose to be coded in CPR mode. For each chroma sub-block, its block vector is derived from the corresponding luma block’s block vector. Or this chroma CU can choose from other available modes (intra, PCM, etc.). Note that all the derived block vectors for chroma blocks should be valid.

- No coverage or partial coverage: for a current chroma coding unit, if at least one corresponding luma sub-blocks is not coded in CPR mode, or the derived block vector is not valid, the current chroma CU will be coded in one of the available modes (intra mode, PCM, etc.) but not CPR mode.

This document reports coding efficiency of the CE8.2 (dual-tree support turned off) in comparison to VTM-2 and the CPR in BMS (BMS-CPR, dual-tree support turned on).

# Simulation results

The simulation is performed using CE8-2 software (based on BMS-2.1rc1), under VTM configuration. CPR mode is turned on. Common test conditions in CE8 [2] are followed. The following test is performed:

- Test 1: CE8.2 (dual-tree support turned off)

Results are shown in the following table:

Table I: coding performance comparison of test 1 on top of VTM-2.0.1 and BMS-CPR (dual-tree support turned on)

# Conclusion

In this contribution, the effect of CPR mode with dual-tree support is investigated. Results of the difference between CPR mode with and without dual-tree support are reported.

# References

[1] Xiaozhong Xu, Xiang Li and Shan Liu, “CE8-2.2: Current picture referencing using reference index signaling”, JVET-K0076, Ljubljana, SL, July 2018

[2] X. Xu, K. Müller, L. Wang, “Description of Core Experiment 8 (CE8): Current Picture Referencing”, JVET-K1028, Ljubljana, SL, July 2018

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).