| woint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0299-v1 |
| --- | --- |

| Title: | CE8-related: CPR mode with merge mode improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li, Min Gao, Jing Ye and Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, the CPR mode is combined with some improvements from merge mode, i.e., non-adjacent and history-based merge candidates (CE4.4.2, CE4.4.7). Combination of the two are tested.

With the improved merge mode CE4.4.2, the reported BD rate changes from the base method (BMS-CPR) are as follows:

- In AI, -0.09%/-0.33%/-1.71% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.58%/-0.61%/-2.06% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.37%/-0.63%/-1.93% for CTC/Class F/SCC 1080p classes, separately.

With the improved merge mode CE4.4.2, the reported BD rate changes from the base method (CPR with 1 CTU search range, CE8.3.1b) are as follows:

- In AI, -0.06%/-0.21%/-0.95% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.57%/-0.56%/-1.69% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.43%/-0.50%/-2.12% for CTC/Class F/SCC 1080p classes, separately.

With the improved merge mode CE4.4.2+CE4.4.7, the reported BD rate changes from the base method (BMS-CPR) are as follows:

- In AI, -0.23%/-0.78%/-2.69% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.87%/-1.14%/-2.99% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.52%/-0.94%/-2.87% for CTC/Class F/SCC 1080p classes, separately.

With the improved merge mode CE4.4.2+CE4.4.7, the reported BD rate changes from the base method (CPR with 1 CTU search range, CE8.3.1b) are as follows:

- In AI, -0.16%/-0.57%/-1.83% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.84%/-1.00%/-2.56% for CTC/Class F/SCC 1080p classes, separately.

- In LB, -0.56%/-0.94%/-2.74% for CTC/Class F/SCC 1080p classes, separately.

# Introduction

In CE4, a few methods are proposed to improve the inter prediction coding efficiency by adding more merge mode candidates into the list. Among them, two methods are of interest. One is CE4.4.2, non-adjacent merge candidates, whose positions are directly extended from spatial adjacent merge candidates in VVC [1]; the other one is CE4.4.7, history-based candidates, which are from previously coded blocks’ motion vectors in coding order [2]. When these two are combined in [3], the coding efficiency is higher than any individual ones.

The tests performed in this contribution show that when the merge mode improvement is utilized in CPR mode, the coding efficiency improvement can be achieved as well. For CPR mode, the following variation is tested:

- CPR with full version, as in BMS-2.1rc1

- CPR with 1 CTU size search range, as in CE8.3.1b

For merge mode extension, the following variations are tested:

- CE4.4.2: non-adjacent merge candidates

- CE4.4.2+CE4.4.7: combination of non-adjacent merge candidates and history-based merge candidates

# Simulation results

The simulation is performed using CE8-3 software (based on BMS-2.1rc1), under VTM configuration. CPR mode is turned on. Common test conditions in CE8 [4] are followed. The following two tests are performed:

- Test 1: BMS-CPR + CE4.4.2

- Test 2: CE8.3.1b + CE4.4.2

- Test 3: BMS-CPR + CE4.4.2+CE4.4.7

- Test 4: CE8.3.1b + CE4.4.2+CE4.4.7

Results are shown in the following two tables:

Table I: coding performance comparison of test 2 on top of VTM-2.0.1 and BMS-CPR (full frame)

Table II: coding performance comparison of test 1 on top of VTM-2.0.1 and CE8-3-1b (1 CTU, no chroma interpolation)

Table III: coding performance comparison of test 2 on top of VTM-2.0.1 and BMS-CPR (full frame)

Table IV: coding performance comparison of test 3 on top of VTM-2.0.1 and CE8-3-1b (1 CTU, no chroma interpolation)

# Conclusion

In this contribution, combinations of CPR mode variations (CE8.3.1b and full frame version), some of the merge mode improvement methods are tested. It is reported that by adding the improved merge mode, the coding performances of the tested CPR modes (CE8.3.1b and full frame version) are improved further.

# References

[1] M. Gao, J. Ye, X. Li, X. Xu, S. Liu, “CE4.4.2: Long distance merge candidates”, JVET-L0323, Macao, CN, Oct. 2018

[2] L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, “CE4: History-based Motion Vector Prediction (Test 4.4.7)”, JVET-L0266, Macao, CN, Oct. 2018

[3] M. Gao, J. Ye, X. Li, X. Xu, S. Liu, L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, “CE4: Combined test of CE4.4.2 and CE4.4.7”, JVET-L0321, Macao, CN, Oct. 2018

[4] X. Xu, K. Müller, L. Wang, “Description of Core Experiment 8 (CE8): Current Picture Referencing”, JVET-K1028, Ljubljana, SL, July 2018

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).