| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 03–12 Oct. 2018 | Document: JVET-L0300 |
| --- | --- |

| Title: | CE4-related: Generic Vector Coding of Motion Vector Difference |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Mehdi Salehifar Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email: | 1-858-635-5300 seethal.paluri@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

Abstract

Current MVD coding uses 4 context coded bins in the worst case scenario. Furthermore, these context bins are used in conjunction with sign coding. This proposal presents, a Generic Vector Coding technique for motion vector difference (MVD) that can be applied to any x-y MVD combination with the goal to decrease the complexity by reducing the number of context coded bins. The proposed method jointly exploits the frequency distribution capturing the similarity between the MVD (x-y) combinations, thereby enabling combined MVD coding instead of independently coding MVDx and MVDy. This scheme has an advantage over the conventional technique as it uses 50% fewer context coded bins in the worst case scenario. Additionally, the proposed method offers another unique advantage of eliminating the need for sign coding by using a Layer and Index concept to represent the MVD values. It is reported that the coding performance is unchanged and the results for RA and LDB are (-0.01/0.02/-0.04) and (-0.01/-0.17/-0.08) respectively.

# Background

The existing coding of MVD x-y components rely on coding the x and y components independently. Decoding of each MVD component is done by first checking if the received MVD component is greater than zero and if true, if it is greater than one. It should be noted that these two flags are context coded bins. Following this, the MVD component’s absolute value is decoded and the sign information is also decoded. The process is then repeated for the second MVD component. Therefore, in the worst case scenario, the decoded block uses 4 context coded bins during decoding, which can increase the computational complexity. Further to this the sign bit needs to also be coded.

# Proposed method

## Layer and index concept

Figure 1.0 below shows the results achieved by grouping MVD(x,y) values with similar frequency distributions. The center cell represents a MVD(x,y) combination of (0,0), i.e., MVDx = 0 and MVDy = 0. The center point (0,0) also corresponds to the most frequently occuring MVD combination in the data set. Further to this, the blocks adjacent to the center point correspond to either an increment or decrement of the MVD values along the horizontal or vertical axes. Frequency analysis of the data suggests, that certain combinations of MVD values occur with similar probabilities such that it can be combined into groups. Consequently, we define a Layer to represent MVD combinations that occur with similar probabilities. The figure below can also be expanded to incorporate several other layers. We define groups of MVD combinations that occur with similar likelihood as a Layer. Figure 1.0 illustrates a rhombus like structure with five layers. As the Layers grow, so too does the magnitude of the MVDs. Additionally, within each Layer, an index identifies the MVD combination being considered. The proposed scheme codes, the Layer and subsequently its index to indicate the combination of the MVD(x,y) being used. This proposal has advantages over the tradition MVD coding technique, as the proposed scheme (i) codes both the MVD x and y components together, (ii) sign information is not sent separately as it is deciphered using the Layer and index, and (iii) uses fewer context bins.

Figure 1.0: Shows the MVD(x,y) Layers.

Figures 2.0 (a) below shows Layers 0 and 1 and Figure 2.0 (b) shows the characteristics of MVD in each quadrant. It should be noted that while the horizontal axis represents MVD x and the vertical axis represents the MVD y components, the proposed scheme uses a flipped vertical axis to lower the computational complexity of the indexing mechanism.

Figures 2.0 (a –b): Show ths specifics of the MVD characteristics.

## Proposed Coding Structure

Figure 3.0 below illustrates the proposed decoder structure. First, a flag checking if the Layer is  greater than zero (GRT0) is parsed. Following this, the Layer GRT0 flag is checked if it is true or false. If GRT0 flag is 0 (i.e., false), then MVDx and MVDy values are both zero and the process is exited. If Layer GRT0 flag is true, then another flag, i.e., is Layer greater than one (GRT1) flag is parsed. If the Layer GRT1 is false, then index to be decoded lies within Layer 1. In such a case, the index is then parsed and the MVD(x,y) is deciphered. If the Layer GRT1 flag is true, then the minimum Layer that can be decoded is 2. Therefore, the Layer Id is determined by parsing the remaining Layer information (i.e., Rem_Layer) using Exponential Golomb Code of order one. Here, the (Absolute Layer – 2) value is decoded. Furthermore, the index within the Layer is also parsed using Truncated Binary Code. Once, the index is parsed, the combination of MVD value is deciphered.

Figure 3.0 Shows the decoder structure.

Once the Layer and index information is deciphered, the decoder interprets the actual MVD (x,y) values in the following way. For simplification of notation, Layer is represented as Lyr and index is represented as idx in the equations shown below:

It should be noted that if Layer equals to 0, then MVD(x,y) = (0,0).

If Layer equals 1:

MVDx=(idx≤Lyr ? idx :Lyr≪1-idx)

MVDy=(idx≤Lyr ? !idx :(idx==2 ? -1 :0)                     Eq. (1-2)

If Layer is greater than 1:

Then MVDx and MVDy are represented by Eq. (3-4)

MVDx= (idx≤Lyr ?idx:(idx≤Lyr≪1+Lyr?Lyr≪1-idx :idx-Lyr≪2)

MVDy= idx≤(Lyr≪1 ? Lyr-idx :idx-(Lyr≪1+Lyr))                       Eq. (3-4)

# 3	Experimental results

The proposed method has been implemented on VTM 2.0.1 reference software and evaluated experimentally according to common test conditions defined in JVET-J1010 [2]. The results were generated for all the sequences.

Table 1: Experimental results for random access test condition; anchor is VTM 2.0.1.

|  | Over VTM 2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01 | -0.01 | 0.00 | 100 | 99 |
| Class A2 | 0.03 | 0.04 | -0.01 | 99 | 98 |
| Class B | -0.01 | 0.05 | -0.07 | 100 | 97 |
| Class C | -0.03 | -0.03 | -0.05 | 99 | 96 |
| Class E |  |  |  |  |  |
| Overall | -0.01 | 0.02 | -0.04 | 100 | 97 |
| Class D | 0.03 | -0.21 | -0.14 | 98 | 99 |

Table 2: Experimental results for low delay B test condition; anchor is VTM 2.0.1.

|  | Over VTM 2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00 | 0.09 | -0.02 | 99 | 94 |
| Class C | 0.00 | -0.26 | -0.05 | 99 | 95 |
| Class E |  |  |  |  |  |
| Overall | -0.01 | -0.17 | -0.08 | 99 | 95 |
| Class D | 0.04 | -0.01 | -0.28 | 99 | 97 |

# Conclusion

We observed that the proposed method is a simplified and logical way to combine the MVD coding of x and y components and it reduces the number of context coded bins used by 50% in the worst case scenario. Also, the sign information need not be coded. We recommend that the proposed Generic Vector Coding for Motion Vector Difference be adopted.

# Reference

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-J1010, 10th Meeting: San Diego, CA, 10–20 April 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).