| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0309 |
| --- | --- |

| Title: | CE4-related: Simplification to HMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seethal Paluri Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This proposal presents methods to reduce the worst case number of pruning checks for CE4.4.7  − History based Motion Vector prediction (HMVP). In CE4.4.7, history-based MV candidate for merge and AMVP is proposed, and pruning check is performed when inserting HMVP candidate into the merge list or AMVP list. To reduce the worst case number of pruning checks, the following two methods are proposed:

- Limit the number of existing merge candidates to check,

- Limit the number of history candidates to check

In both methods, the worst case of number of pruning checks is reduced by half from 24 in existing CE4.4.7 software to 12. Experimental results show that the loss is very small or no loss. For method (1), comparing to HMVP without such limitation, there is 0.04% and xx%  loss for RA and LD respectively. For method 2), there is no loss.

# Introduction

In CE4.4.7 History-based Motion Vector Prediction (HMVP) by ByteDance [1], a table of HMVP candidates is maintained and updated after coding a non-affine inter-coded block. The table is updated by adding the associated motion information as a new HMVP candidate to the last entry of the table. A First-In-First-Out (FIFO) or constraint FIFO rule is applied to remove and add entries to the table.  The HMVP candidates is applied to either merge candidate list or AMVP candidate list. When adding HMVP candidates to the merge candidates list, a pruning check is performed to avoid adding redundant candidate into the list. For a full redundant check, it needs to compare the new HMVP candidate with all existing merge candidates in the list. In the case of merge list size of 6, and HMVP table size of 6, the worst case number of pruning checks needed to insert HMVP candidate to merge list would be 30.

In the released CE4.4.7 software, the default HMVP table size is 6, and the following condition is present to reduce the worst case number of pruning checks.

if (numSpatTMVP >= 5) num_allowedCand = std::min(num_allowedCand, 8 - numSpatTMVP);

where numSpatTMVP is the number of existing merge candidates except ATMVP, and num_allowedCand is the number of history MV to check. Given this limitation, and depending on the existing number of candidate, the worst case number of checks to insert a history MV would be 24. The breakdown for different number combinations of existing candidates and history MV candidates is as below.

| # of existing candidates | # of existing candidate to check | Max # of history MV to check | Worst case # of redundant checks |
| --- | --- | --- | --- |
| 1 | 1 | 6 | 6 |
| 2 | 2 | 6 | 12 |
| 3 | 3 | 6 | 18 |
| 4 | 4 | 6 | 24 |
| 5 | 5 | 3 | 15 |

# Proposed methods

## Limit the number of Existing Candidates to Check

In this method, the number of existing candidates to check is limited to 2, and the worst case number of redundant checks is reduced to 12. The breakdown is as below.

| # of existing candidates | # of existing candidate to check | Max # of history MV to check | Worst case # of redundant checks |
| --- | --- | --- | --- |
| 1 | 1 | 6 | 6 |
| 2 | 2 | 6 | 12 |
| 3 | 2 | 6 | 12 |
| 4 | 2 | 6 | 12 |
| 5 | 2 | 6 | 12 |

## Limit the number of History MVs Candidates to Check

In this method, the number of history MV candidates to check is dependent on the number of existing merge candidate. The concept is similar to what is already present in the CE software but with different threshold values. The new limitation on the number of history MV to check is”

if (numSpatTMVP >= 3) num_allowedCand = std::min(num_allowedCand, 7 - numSpatTMVP);

Given this restriction, the worst case number of redundant checks is also reduced to 12.

| # of existing candidates | # of Existing candidate to check | Max # of history MV to check | Worst case # of redundant checks |
| --- | --- | --- | --- |
| 1 | 1 | 6 | 6 |
| 2 | 2 | 6 | 12 |
| 3 | 3 | 4 | 12 |
| 4 | 4 | 3 | 12 |
| 5 | 5 | 2 | 10 |

# Experimental results

The proposed method has been implemented into CE4.4.7 software and macros  APPLIED_AMVP , ADD_PRUNING_4TABLE_UPDATE are turned off. So HMVP is only applied to merge list generation, and without constraints during table update. This result is shown in Table 1 and used as anchor.

Table 2 shows result of (1) limiting existing merge candidates to 2, comparing to Table 1, there is a small loss of 0.05% in RA condition.

Table 3 shows result of (2) limiting number of history MV candidates. Comparing the Table 1, there is no loss.

Table 1: Result of HMVP with ADD_PRUNING_4TABLE_UPDATE=0

|  | Over VTM 2.0.1 (RA) |  |  |  |  | Over VTM 2.0.1 (LD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.38% | -0.50% | 99% | 100% |  |  |  |  |  |
| Class A2 | -0.36% | -0.66% | -0.64% | 98% | 99% |  |  |  |  |  |
| Class B | -0.52% | -0.61% | -0.68% | 98% | 100% | -0.32% | -0.38% | -0.62% | 97% | 98% |
| Class C | -0.33% | -0.37% | -0.57% | 99% | 96% | -0.04% | -0.09% | -0.21% | 98% | 97% |
| Class E |  |  |  |  |  | 0.15% | 0.23% | -0.03% | 99% | 98% |
| Overall | -0.39% | -0.51% | -0.60% | 98% | 99% | -0.10% | -0.13% | -0.34% | 98% | 97% |
| Class D | -0.12% | -0.29% | -0.28% | 98% | 84% | 0.03% | 0.64% | -0.04% | 97% | 89% |

Table 2: Results of limiting  existing candidates to check to 2 during HMVP pruning

|  | Over VTM 2.0.1 (RA) |  |  |  |  | Over VTM 2.0.1 (LD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.27% | -0.45% | 99% | 99% |  |  |  |  |  |
| Class A2 | -0.32% | -0.60% | -0.54% | 98% | 98% |  |  |  |  |  |
| Class B | -0.47% | -0.55% | -0.57% | 98% | 98% | -0.25% | -0.37% | -0.82% | 97% | 97% |
| Class C | -0.27% | -0.30% | -0.44% | 98% | 94% | -0.04% | -0.03% | 0.00% | 98% | 96% |
| Class E |  |  |  |  |  | 0.03% | 0.21% | -0.04% | 99% | 96% |
| Overall | -0.34% | -0.44% | -0.51% | 98% | 97% | -0.11% | -0.11% | -0.35% | 98% | 96% |
| Class D | -0.08% | -0.22% | -0.24% | 97% | 81% | -0.03% | 0.44% | 0.01% | 98% | 88% |

Table 3: Results of limiting  number of HMVP candidates during pruning

|  | Over VTM 2.0.1 |  |  |  |  | Over VTM 2.0.1 (LD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.24% | -0.49% | 99% | 100% |  |  |  |  |  |
| Class A2 | -0.33% | -0.48% | -0.58% | 97% | 99% |  |  |  |  |  |
| Class B | -0.53% | -0.64% | -0.74% | 98% | 99% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.31% | -0.35% | -0.36% | 98% | 95% | -0.08% | -0.15% | -0.16% | #NUM! | #NUM! |
| Class E |  |  |  |  |  | 0.09% | 0.00% | -0.01% | #NUM! | #NUM! |
| Overall | -0.39% | -0.45% | -0.56% | 98% | 98% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.11% | -0.45% | -0.29% | 97% | 82% | 0.04% | 0.22% | -0.04% |  |  |

# Conclusion

This proposal presents two methods to reduce the worst case number of pruning checks for history based motion vector prediction (HMVP). It reduces the number of pruning checks to half with little or no loss comparing to existing CE4.4.7 software. It is proposed to consider these methods together with the CE4.4.7 HMVP method.

.

# Reference

- L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Dong, “CE4-related: History-based Motion Vector Prediction,” JVET-K0104, Ljubljana, SI, July 2018.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVT-K1024, Ljubljana, SI, July 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-K1010 Ljubljana, SI, July 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).