| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0310 |
| --- | --- |

| Title: | CE1-1.3.1: Partial CU for picture boundary handling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

As part of the CE1 sub test 1, partial CU is tested for picture boundary handling. It is reported that 0.00%, 0.02%, 0.08% and 0.04% luma BD-rate reduction is achieved on average over VTM-2.0.1, for AI, RA, LDB and LDP configurations, respectively.

# Introduction

In VTM-2, when picture width or height is not power of 2, at the picture boundary a CU may not fit the picture boundary and is forced to split into smaller coding blocks, so that each CU will have its area entirely inside the picture boundary. It is not efficient to always force split into smaller CUs.

# Proposed method

In this contribution, partial CU is proposed. A partial CU is defined as a CU which contains both areas inside and outside a picture. For partial CU, only the area inside the picture boundary is considered valid and coded, as shown in Figure 1.

Figure 1 a partial CU with both valid and invalid regions

Therefore, with partial CU, it is not necessary to force split until all smaller CUs fit the picture boundary. When a CU is coded as non-split and its valid part has non-power-of-2 width or height, it is called a partial CU. The residual is not coded for a partial CU, and skip flag is inferred as true for the valid CU area. In current implementation, partial CU is disabled for intra slices.

The syntax modifications are highlighted in the syntax table.

| coding_quadtree( x0, y0, log2CbSize, cqtDepth, treeType ) { | Descriptor |
| --- | --- |
| if( ( ( ( ( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples ) ? 1 : 0 ) + ( ( y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples ) ? 1 : 0 ) + ( ( ( 1 << log2CbSize ) <= MaxBtSizeY ) ? 1 : 0 ) ) >= 2 ¦ ¦ allowPartialCu ) && log2CbSize > MinQtLog2SizeY ) |  |
| qt_split_cu_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( qt_split_cu_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| y1 = y0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| coding_quadtree( x0, y0, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree( x1, y0, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x0, y1, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x1, y1, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| } else |  |
| multi_type_tree( x0, y0, 1 << log2CbSize, 1 << log2CbSize, 0, 0, 0, treeType, allowPartialCu ) |  |
| } |  |

| multi_type_tree( x0, y0, cbWidth, cbHeight, mttDepth, depthOffset, partIdx, treeType, allowPartialCu ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ¦ ¦ allowPartiaCu ) && ( x0 + cbWidth <= pic_width_in_luma_samples ¦ ¦ allowPartialCu ) && (y0 + cbHeight <= pic_height_in_luma_samples ¦ ¦ allowPartialCu ) ) |  |
| mtt_split_cu_flag | ae(v) |
| if( mtt_split_cu_flag ) { |  |
| … |  |
| } else { |  |
| cbWidth = min( cbWidth, pic_width_in_luma_samples – x0 ) |  |
| cbHeight = min( cbHeight, pic_height_in_luma_samples – y0 ) |  |
| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| if( is_power_of_2( cbWidth ) && is_power_of_2( cbHeight ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| … |  |
| } |  |

The variable allowPartialCu is derived as follows.

allowPartialCu = slice_type != I && ( ( ( x0 + ( 1  <<  log2CbSize ) > pic_width_in_luma_samples ) && is_non_power_of_2( pic_width_in_luma_samples – x0 ) ) | | ( ( x0 + ( 1  <<  log2CbSize ) > pic_height_in_luma_samples ) && is_non_power_of_2( pic_height_in_luma_samples – y0 ) ) )

# Simulation results

The proposed method is implemented on VTM-2.0.1. The VTM RA and LDB configurations are used for the simulation.

Reference: VTM-2.0.1

Test: VTM-2.0.1 + proposed method

# Conclusions

In this contribution, partial CU is proposed for picture boundary handling. It is reported that 0.00%, 0.02%, 0.08% and 0.04% luma BD-rate reduction is achieved on average over VTM-2.0.1, for AI, RA, LDB and LDP configurations, respectively.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations for SDR video,” F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010.

- “Description of Core Experiment 1 (CE1): Partitioning,” J. Ma, F. Le Léannec, M. W. Park, JVET-K1021.

- “CE1-related: Partial CU for picture boundary,” M. Xu, X. Li, S. Liu, JVET-K0366.

- “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, JVET-K1001.