| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0311 |
| --- | --- |

| Title: | CE9.1.7: Constrained Decoder Side Motion Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In CE9.1.7, it is tested that the refined MV from immediate previous CU cannot be used as MVP in current block, which is studied in CE9. In addition, it is required in CE9 that the refined MV from left CTU and top-right CTU cannot be used as MVP in current block. In this contribution, a constraint region is signaled to include the current CTU, the left CTU, and the top-right CTU. When the MVP comes from the constraint region, the refined MV cannot be used as MVP for current CU. Otherwise, the refined MV can be used as MVP for current block. In addition, the refined MV in previous CU cannot be used as MVP. It is reported the proposed method achieves -1.80% and -0.90% average luma BD-rate change with the MV in previous CU marked as available and unavailable, respectively.

# Introduction

Decoder side motion vector derivation (DMVD) techniques were proposed to derive motion information at decoder side after parsing stage so that a lot of bits can be saved for motion vectors (MV). These methods, such as FRUC and DMVR, showed good coding performance and were studied in CE-9 [2]. Such methods introduce dependency between the initial MV of the current block and the final MV of previously coded blocks, which leads to pipeline latency issue in hardware design. In BMS-2.1-rc1 [5], such dependency was removed by prohibiting using the refined MVP from spatial neighbors. However, such constraint may degrade the coding performance.

# Proposed method

It is proposed in JVET-K0295 [3] the refined MV from immediate previous CU cannot be used as MVP in current block, which is studied in CE9. In addition, it is required in CE9 that the refined MV from left CTU and top-right CTU cannot be used as MVP in current block.

Conforming to the CE9 requirements, a constraint region is signaled. The refined MV in the constraint region cannot be used as spatial MVP for the neighboring blocks. The refined MV outside of the constraint region may be used as spatial MVP. In this contribution, the constraint region includes the current CTU, the left CTU, and the top-right CTU. In addition, the refined MV from immediate previous CU cannot be used as MVP in current block. It is either marked as unavailable or the unrefined MV (MV_init) is used as MVP for the current block.

In BMS-2.1-rc1, DMVR mode is a special merge candidate and there is no specific signaling for DMVR. Moreover, it is possible that the final MV of a block is unchanged after MV refinement. Unfortunately, it is difficult to distinguish whether MV_final would be different from MV_init without actual MV refinement. Therefore, a neighboring block CU_neib is regarded as in DMVD mode if all the following conditions are true in the current implementation.

- CU_neib is in merge mode

- CU_neib is not affine

- CU_neib has two valid but different reference frames

# Simulation results

The proposed method is implemented on top of BMS-2.1-rc1, using VTM configurations with DMVR tool on.

## Test with MV in previous CU not marked as unavailable

First reference: BMS-2.1-rc1 (VTM configuration)

Second reference: BMS-2.1-rc1 (VTM configuration) + DMVR

Test: BMS-2.1-rc1 (VTM configuration) + DMVR + MV in previous CU not marked as unavailable

When the MV in previous CU is not marked as unavailabe, it is observed from the results that the proposed method shows -1.80% average luma BD-rate difference compared with BMS-2.1-rc1 with VTM configurations, and -0.17% difference compared with the BMS-2.1-rc1 with VTM configuration and DMVR tool on. The gain over BMS-2.1-rc1 with DMVR comes from using the refined MV from outside of the constraint region.

## Test with MV in previous CU marked as unavailable

First reference: BMS-2.1-rc1 (VTM configuration)

Second reference: BMS-2.1-rc1 (VTM configuration) + DMVR

Test: BMS-2.1-rc1 (VTM configuration) + DMVR + MV in previous CU is marked as unavailable

When the MV in previous CU is marked as unavailabe, it is observed from the results that the proposed method shows -0.87% average luma BD-rate difference compared with BMS-2.1-rc1 with VTM configurations, and 0.77% difference compared with the BMS-2.1-rc1 with VTM configuration and DMVR tool on.

# Conclusions

A constraint region is defined as conforming to CE9 requirement. The availability of the refined MV depends on whether it is within the constraint region. In addition, the refined MV from immediate previous CU cannot be used as MVP in current block. It is reported that the proposed method achieves -1.80% and -0.90% average luma BD-rate change with the MV in previous CU marked as available and unavailable, respectively. Moreover, the signaling of constrained region provides more flexibility and higher coding efficiency. It is proposed to adopt the method with the MV in previous CU not marked as unavailable.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations for SDR video,” F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010.

- “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation,” S. Esenlik, Y.-W. Chen, F. Chen, JVET-K1029.

- “CE9-related: Constrained Decoder Side Motion Vector Derivation,” M. Xu, X. Li, S. Liu, JVET-K0295.

- “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, JVET-K1001.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS