| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0315 |
| --- | --- |

| Title: | CE4 - Extension of merge and AMVP candidates for inter prediction (Test CE4.4.10) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, and Shan Liu 661 Bryant St, Palo Alto, CA 94301 USA | Tel: Email: | +1-650-798-3300 guichunli@tencent.com xiaozhongxu@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

# Abstract

This contribution reports the test results of Extension of merge and AMVP candidates for inter prediction, which was proposed in JVET-K0301. This CE has 4 sub-tests. In test CE4.4.10.a, it was reported that there are 0.03% and 0.06% BD-rate gain on random access and low delay B configuration, respectively. In test CE4.4.10.b, it was reported that there are 0.02% and 0.07% BD-rate gain on random access and low delay B configuration, respectively. In test CE4.4.10.c, it was reported that there are 0.05% and 0.10% BD-rate gain on random access and low delay B configuration, respectively. A combined test of CE4.4.10.c and CE4.4.2.b was also performed as test CE4.4.10.e, and it was reported that there are 0.73% and 0.51% BD-rate gain on random access and low delay B configuration, respectively.

# Introduction

In current Versatile Video Coding Test Model (VTM-2.0.1), the positions of spatial motion vector predictors for inter merge candidates and AMVP candidates are as depicted in Figure 1.

Figure 1 - Merge Candidates Currently used in VTM-2.0.1

A0 is the left-bottom candidate, A1 is the left candidate, B0 is the above-right candidate, B1 is the above candidate, and B2 is the above-left candidate.

In inter merge mode, the order of spatial merge candidates is {A1, B1, B0, A0, B2}.

The following pruning process is performed in current VTM software is shown in Table 1.

Table 1-Pruning Process of Merge Candidates in VTM

| Candidate | Pruning |
| --- | --- |
| A1 | If A1 available |
| B1 | If B1 available, prune against A1 |
| B0 | If B0 available, prune against B1, A1 |
| A0 | If A0 available, prune against A1, B1, B0 |
| B2 | If B2 available, prune against A1, B1, A0, B0 |

In AMVP mode, the order of derivation of spatial motion vector predictor candidates on left side is

{A0, A1, Scaled A0, Scaled A1};

and the order of derivation on above side is

{B0, B1, B2, Scaled B0, Scaled B1, Scaled B2}.

# Proposed Method in JVET-K0301

Two new spatial candidate positions are included in the proposed method. As depicted in Figure 2, the new candidates are A2, left-middle candidate, and B3, above-middle candidate. These two candidates are added into both merge candidate list and AMVP candidate list. For both list, the number of allowed candidates, as well as the number of full RD evaluations, is kept unchanged.

Figure 2 - Merge Candidates used in Proposed Method

## Test CE4.4.10.a: Candidate extension in Inter Merge Mode

In inter merge mode, the order of spatial merge candidate derivation in the proposed method is {A2, B3, A1, B1, B0, A0, B2}. Pruning is applied to remove redundant merge candidates. And the derivation process stops when maximum number of merge candidates is met.

The derivation and pruning process is shown in order in the following Table 2. Additional pruning is applied only for B3, A1, and B1 to reduce the impact on complexity.

Table 2-Pruning Process of Merge Candidates in Proposed Method

| Candidate | Pruning |
| --- | --- |
| A2 | If A2 available |
| B3 | If B3 available, prune against A2 |
| A1 | If A1 available, prune against A2, B3 |
| B1 | If B1 available, prune against A1, A2, B3 |
| B0 | If B0 available, prune against B1, A1 |
| A0 | If A0 available, prune against A1, B1, B0 |
| B2 | If B2 available, prune against A1, B1, A0, B0 |

Compare to the original method, there are only 2 additional potential candidates introduced.

The following complexity analysis includes spatial, temporal, combined bi-prediction candidates, and ATMVP is excluded as it doesn’t prune against regular non-sub-block candidates.

Table 3- Complexity analysis for CE4.4.10.a

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | 8 | 25 | 0 | 1 | 0 | 8 |  |

## Test CE4.4.10.b: Candidate extension in AMVP mode

In AMVP mode, the order of derivation of spatial motion vector predictor candidates on left side is

{A2, A0, A1, Scaled A2, Scaled A0, Scaled A1};

and the order of derivation on above side is

{B3, B0, B1, B2, Scaled B3, Scaled B0, Scaled B1, Scaled B2}.

## Test CE4.4.10.c: Candidate extension for both Merge and AMVP mode

Test CE4.4.10.c is a combination of CE4.4.10.a and CE4.4.10.b, which applies extended candidates in middle positions to both merge and AMVP candidate derivation processes.

## Combined Test with CE4.4.2.b

Non-adjacent spatial candidates are proposed in CE4.4.2, which doesn’t include candidates from middle positions. In this combined test, middle-left and above-middle candidates are added to each round of search based on CE4.4.2.b. And similar to CE4.4.10, all the extended spatial candidates are used for both merge candidates and AMVP candidates.

In each round of spatial merge candidate search, the corresponding middle-left and above-middle candidates are checked first, then followed by other spatial positions in the order as in CE4.4.2. An example of the non-adjacent search with extended middle candidates is depicted in figure-3.

Figure 3 - extended non-adjacent candidates with middle positions

In AMVP mode, the order of derivation of spatial motion vector predictor candidates on left or above side is following the same order as in 2.2.

For example, let R denote the R-th round of search, candidate positions are as depicted in Figure 3, the order of derivation in the R-th round of search on the left side is:

{AR, FR, CR, Scaled AR, Scaled FR, Scaled CR};

and the order of derivation in the R-th round of search on above side is:

{BR, ER, DR, GR, Scaled BR, Scaled ER, Scaled DR, Scaled GR}.

In this test, the number of additional search rounds is set to be 2. Max number of candidate comparison is set to 30. Maximum number of candidates for the merge list is set to 8.

The following complexity analysis includes spatial, temporal, combined bi-prediction candidates, and ATMVP is excluded as it doesn’t prune against regular non-sub-block candidates.

Table 4 - Complexity analysis for non-adjacent merge of CE4.4.10.e

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 22 | 30 | 0 | 1 | 0 | 22 |  |

# Simulation Results

In this core experiment, VTM-2.0.1 with common test condition is used as anchor.

There are 4 tests performed, as listed below. Originally proposed CE4.4.10.d was withdrawn.

- CE4.4.10.a: Add merge candidates at middle position for inter merge mode;

- CE4.4.10.b: Add new AMVP predictor candidates from middle position;

- CE4.4.10.c: CE4.4.10.a + CE4.4.10.b;

- CE4.4.10.e: CE4.4.10.c + CE4.4.2;

## Simulation result of test CE4.4.10.a

In test CE4.4.10.a, it was observed that there are 0.03% and 0.06% BD-rate gain on random access and low delay B configuration, respectively.

Table 5 - Test result of CE4.4.10.a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.07% | 0.09% | 99% | 99% |
| Class A2 | 0.02% | -0.01% | -0.05% | 100% | 98% |
| Class B | -0.04% | -0.06% | -0.10% | 102% | 100% |
| Class C | -0.05% | 0.00% | -0.10% | 105% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.01% | -0.05% | 102% | 101% |
| Class D | -0.04% | -0.17% | -0.13% | 104% | 108% |
| Class F (optional) | -0.02% | -0.01% | -0.02% | 103% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.04% | -0.21% | 97% | 94% |
| Class C | -0.01% | -0.18% | -0.29% | 100% | 98% |
| Class E | -0.18% | 0.16% | -0.44% | 101% | 98% |
| Overall | -0.06% | -0.04% | -0.29% | 99% | 96% |
| Class D | -0.02% | 0.08% | -0.75% | 104% | 101% |
| Class F (optional) | -0.06% | 0.50% | -0.20% | 102% | 100% |

## Simulation result of test CE4.4.10.b

In test CE4.4.10.b, it was observed that there are 0.02% and 0.07% BD-rate gain on random access and low delay B configuration, respectively.

Table 6 - Test result of CE4.4.10.b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | 0.01% | 98% | 98% |
| Class A2 | -0.03% | -0.01% | -0.08% | 99% | 97% |
| Class B | 0.00% | -0.06% | -0.06% | 101% | 100% |
| Class C | -0.01% | 0.03% | -0.08% | 105% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | -0.06% | 101% | 101% |
| Class D | -0.02% | -0.10% | -0.10% | 104% | 107% |
| Class F (optional) | -0.03% | -0.05% | -0.06% | 103% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.07% | -0.33% | 98% | 97% |
| Class C | -0.02% | -0.04% | -0.16% | 100% | 98% |
| Class E | -0.22% | -0.51% | -0.50% | 100% | 99% |
| Overall | -0.07% | -0.17% | -0.31% | 99% | 98% |
| Class D | 0.00% | 0.07% | -0.12% | 105% | 105% |
| Class F (optional) | -0.11% | 0.42% | 0.54% | 102% | 99% |

## Simulation result of test CE4.4.10.c

In test CE4.4.10.c, it was observed that there are 0.05% and 0.10% BD-rate gain on random access and low delay B configuration, respectively.

Table 7 - Test result of CE4.4.10.c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.05% | 0.01% | 98% | 98% |
| Class A2 | -0.02% | 0.02% | -0.10% | 99% | 98% |
| Class B | -0.05% | -0.06% | -0.14% | 102% | 99% |
| Class C | -0.09% | 0.00% | -0.10% | 105% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.02% | -0.09% | 101% | 101% |
| Class D | -0.06% | -0.24% | -0.06% | 104% | 109% |
| Class F (optional) | -0.03% | -0.06% | -0.08% | 103% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.02% | -0.28% | 98% | 96% |
| Class C | -0.03% | -0.16% | -0.16% | 101% | 99% |
| Class E | -0.28% | -0.17% | -0.35% | 102% | 102% |
| Overall | -0.10% | -0.09% | -0.26% | 100% | 98% |
| Class D | -0.09% | 0.44% | -0.16% | 104% | 104% |
| Class F (optional) | -0.15% | 0.53% | 0.12% | 102% | 101% |

## Simulation result of test CE4.4.10.e

In test CE4.4.10.e, it was observed that there are 0.73% and 0.51% BD-rate gain on random access and low delay B configuration, respectively.

Table 8 - Test result of CE4.4.10.e

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.46% | -0.70% | 100% | 100% |
| Class A2 | -0.78% | -1.12% | -1.29% | 100% | 100% |
| Class B | -0.93% | -0.96% | -1.08% | 100% | 99% |
| Class C | -0.64% | -0.73% | -0.92% | 107% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.73% | -0.83% | -1.00% | 102% | 102% |
| Class D | -0.42% | -0.46% | -0.65% | 107% | 112% |
| Class F (optional) | -0.52% | -0.56% | -0.53% | 104% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.71% | -0.52% | -0.79% | 102% | 100% |
| Class C | -0.35% | -0.47% | -0.42% | 103% | 102% |
| Class E | -0.37% | -0.14% | -0.41% | 103% | 102% |
| Overall | -0.51% | -0.41% | -0.57% | 103% | 101% |
| Class D | -0.24% | 0.19% | -0.58% | 105% | 99% |
| Class F (optional) | -0.97% | -0.53% | -0.33% | 104% | 102% |

The test result of CE4.4.2.b with the same configuration is provided in table-9 as a second reference.

Table 9 - Test result of CE4.4.2.b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.44% | -0.61% | 95% | 98% |
| Class A2 | -0.57% | -0.91% | -1.06% | 95% | 98% |
| Class B | -0.75% | -0.80% | -0.87% | 95% | 92% |
| Class C | -0.46% | -0.42% | -0.55% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.56% | -0.65% | -0.77% | 96% | 96% |
| Class D | -0.32% | -0.26% | -0.36% | 100% | 99% |
| Class F (optional) | -0.38% | -0.38% | -0.40% | 98% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.60% | -0.66% | -0.66% | 98% | 101% |
| Class C | -0.31% | -0.48% | -0.46% | 95% | 91% |
| Class E | -0.23% | -0.51% | -0.48% | 95% | 91% |
| Overall | -0.41% | -0.56% | -0.55% | 96% | 95% |
| Class D | -0.21% | 0.22% | -0.32% | 99% | 91% |
| Class F (optional) | -0.58% | -0.16% | -0.62% | 98% | 94% |

# Conclusion

It was observed that the proposed extension of spatial candidates on middle positions provide consistent gain in both adjacent merge/AMVP and non-adjacent merge/AMVP, with negligible impact on encoding/decoding time. It’s proposed to adopt the proposed method in VVC and VTM software.

# References

[1] G. Li, X. Xu, X. Li, S. Liu, “CE4-related: extension of merge and AMVP candidates for inter prediction”, JVET-K0301, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[4] VTM-2.0.1 software: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-2.0.1

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).