| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0319-v1 |
| --- | --- |

| Title: | CE4-related: Sub-block MV clipping in planar motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Gao, Xiang Li, Meng Xu, and Shan Liu | Tel: Email: | maxwellgao@tencent.com xlxiangli@tencent.com mengxxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

Planar motion vector prediction is achieved by averaging a horizontal and vertical linear interpolation on 4x4 block basis, while the per-pixel memory bandwidth in 4x4 inter prediction block is quite high, especially when 4x4 bi-directional inter prediction is enabled. This contribution is proposed to reduce the memory bandwidth for planar motion vector prediction. Two methods are proposed to reduce the memory bandwidth. In the first method, the MVs of all 4x4 sub-blocks inside a 4x16, 16x4 or 8x8 block are constrained such that the max difference between integer parts of all 4x4 sub-blocks MV is no more than 1 pixel. The second method is implemented on the top of the first method and the MVs of all 4x4 sub-blocks inside a 4x16 and 16x4 block are constrained to make the integer parts of all 4x4 sub-blocks MV are the same. It is reported that the worst-case memory bandwidth requirement by planar motion vector prediction is reduced. Compared with CE4.3.1 with PLANAR_SUBPU_LOG2SIZE = 2, it is reported that the worst-case memory bandwidth requirement by planar motion vector prediction is reduced for the two methods at the cost of minor BD-rate loss.

Introduction

Planar motion vector prediction was proposed in [1] to generate a smooth fine granularity motion field, which is achieved by averaging a horizontal and vertical linear interpolation on 4x4 block basis. But the uni-directional inter prediction for 4x4 block needs to load up to (4+7)*(4+7)=121 luma samples due to the interpolation requirement. That means the per-pixel memory bandwidth requirement for 4x4 uni-directional inter prediction is 7.6 sample per pixel. If the 4x4 bi-directional inter prediction is allowed, the requirement is up to 15.125 sample/pixel. Therefore, the memory bandwidth requirement in planar motion vector prediction is quite high.

Sub-block MV clipping in planar motion vector prediction

The size of the minimal block coded in planar motion vector is 4x16, 16x4 or 8x8 (PLANAR_SUBPU_LOG2SIZE=2). To reduce the memory bandwidth requirement in planar motion vector prediction, the MVs for all 4x4 sub-blocks inside the 4x16, 16x4 or 8x8 blocks are constrained such that the max difference between integer parts of these sub-blocks is no more than a given threshold.

In the first method, the threshold is set as 1 pixel for the sub-blocks inside the 4x16, 16x4 and 8x8 blocks. This means that the max difference between the integer parts of all 4x4 sub-blocks MV inside the 4x16, 16x4 and 8x8 blocks is no more than 1 pixel. The second method is on the top of the first method, where the threshold is set to 0 pixel for 4x16 and 16x4 blocks along vertical and horizontal directions, respectively. Smaller threshold is put on one direction is because there is only one sub-block along the direction. Accordingly, the MV difference among sub-blocks along the direction is not big.

The sub-block MV constrained process applied to 4x16, 16x4 or 8x8 blocks is described as follows:

- The minimal and maximal values for the MV components, MVminx, MVminy, MVmaxx, MVmaxy, are firstly determined for each 4x16, 16x4 or 8x8 block as follows:

  - Get the minimal MV components among all 4x4 sub-blocks inside 4x16, 16x4 or 8x8 block

(MVminx, MVminy) = (min{MVxi}, min{MVyi})

  - Use the integer part of MVminx and MVminy as the minimal MV component

MVminx = MVminx >> MV_precision << MV_precision

MVminy = MVminy >> MV_precision << MV_precision

  - The maximal MV component is calculated as follows:

MVmaxx = MVminx + ((1 + threshold) << MV_precision) – 1

MVmaxy = MVminy + ((1 + threshold) << MV_precision) – 1

- The MV components of each 4x4 sub-block inside 4x16, 16x4 or 8x8 block are clipped as follows:

MVxi = max(MVminx, min(MVmaxx, MVxi))

MVyi = max(MVminy, min(MVmaxy, MVyi))

where (MVxi, MVyi) is the MV of ith sub-block within 4x16, 16x4 or 8x8 block; MV_precision is equal to 4 corresponding to 1/16 motion vector fraction accuracy; threshold is set 1 for 4x16, 16x4 and 8x8 blocks in the first method and in the second method, threshold is set 0 for 4x16 and 16x4 blocks and 1 for 8x8 blocks.

Simulation results

The proposed methods are implemented on the top of CE4.3.1 planar motion vector prediction. The common test conditions defined in [2] is used in the simulations. Table 1 shows the BD-Rate of the first method relative to VTM2.0.1 and CE4.3.1 with PLANAR_SUBPU_LOG2SIZE = 2. Table 2 shows the BD-Rate of the second method relative to VTM2.0.1 and CE4.3.1 with PLANAR_SUBPU_LOG2SIZE = 2.

Table. 1. The BD-Rate of the first method.

Table. 2. The BD-Rate of the second method.

Conclusion

This contribution proposes two methods to constrain the MVs of 4x4 sub-blocks for planar motion vector prediction to reduce memory bandwidth requirement. It is reported that the worst-case memory bandwidth requirement by planar motion vector prediction is reduced at the cost of minor BD-rate loss.

References

[1] N. Zhang, Y. Lin, J. Zheng, JVET-K0135, “CE4.2.14:Planar Motion Vector Prediction”, July. 2018.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, July. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).