| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0320 |
| --- | --- |

| Title: | CE4-related: affine merge mode with prediction offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, and Shan Liu 661 Bryant St, Palo Alto, CA 94301 USA | Tel: Email: | +1-650-798-3300 guichunli@tencent.com xiaozhongxu@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

# Abstract

In this contribution, a method of affine merge mode with prediction offsets is proposed. The proposed method is a refinement of control point motion vectors based on an existing affine merge candidate. The refinement is performed by selecting one of the predefined offsets around the base affine control point motion vector. It is reported that with the proposed method implemented on top of VTM-2.0.1 affine merge common base, 0.32% and 0.09% BD-rate improvement on random access and low delay B configurations are observed, respectively.

# Introduction

Affine merge mode is used in both current Versatile Video Coding Test Model (VTM-2.0.1) and VTM-2.0.1 with affine merge common base software provided in CE4.2.1.

In VTM-2.0.1, there are 5 positions of spatial candidate for model based affine merge candidates, as depicted in Figure 1. If one of the five exists, the first available spatial candidate is used to derive the affine model and the motion vectors of control points of the current block.

Figure 1 - Model-based Affine Merge Candidates

Figure 2 - Control Point based Affine Merge Candidate

In VTM-2.0.1 with affine merge common base, all available spatial affine merge candidates are used to construct an affine merge list. And the list is further extended to include constructed control-point based candidates, if available. Each merge candidate contains predicted motion vector values of each control point of the current block.

Similar to applying prediction offsets to regular inter merge candidates, as is in JVET-K0115, it is proposed in this contribution to enable affine merge mode with prediction offsets.

# Proposed Method

The proposed method selects the first available affine merge candidate as a base predictor. Then it applies a motion vector offset to each control point’s motion vector value from the base predictor. If there’s no affine merge candidate available, this proposed method will not be used.

The selected base predictor’s inter prediction direction, and the reference index of each direction is used without change.

In the current implementation, the current block’s affine model is assumed to be a 4-parameter model, only 2 control points need to be derived. Thus, only the first 2 control points of the base predictor will be used as control point predictors.

For each control point, a zero_MVD flag is used to indicate whether the control point of current block has the same MV value as the corresponding control point predictor. If zero_MVD flag is true, there’s no other signaling needed for the control point. Otherwise, a distance index and an offset direction index is signaled for the control point.

A distance offset table with size of 5 is used as shown in the table below. Distance index is signaled to indicate which distance offset to use. The mapping of distance index and distance offset values is shown in Figure 3.

Table 1 - Distance offset table

| Distance IDX | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Distance-offset | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel |

Figure 3 - Distance index and distance offset mapping

The direction index can represent four directions as shown below, where only x or y direction may have an MV difference, but not in both directions.

| Offset Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-dir-factor | +1 | –1 | 0 | 0 |
| y-dir-factor | 0 | 0 | +1 | –1 |

If the inter prediction is uni-directional, the signaled distance offset is applied on the offset direction for each control point predictor. Results will be the MV value of each control point.

For example, when base predictor is uni-directional, and the motion vector values of a control point is MVP (vpx, vpy). When distance offset and direction index are signaled, the motion vectors of current block’s corresponding control points will be calculated as below.

MV(vx, vy) = MVP (vpx, vpy) + MV(x-dir-factor * distance-offset, y-dir-factor * distance-offset);

If the inter prediction is bi-directional, the signaled distance offset is applied on the signaled offset direction for control point predictor’s L0 motion vector; and the same distance offset with opposite direction is applied for control point predictor’s L1 motion vector. Results will be the MV values of each control point, on each inter prediction direction.

For example, when base predictor is uni-directional, and the motion vector values of a control point on L0 is MVPL0 (v0px, v0py), and the motion vector of that control point on L1 is MVPL1 (v1px, v1py). When distance offset and direction index are signaled, the motion vectors of current block’s corresponding control points will be calculated as below.

MVL0(v0x, v0y) = MVPL0 (v0px, v0py) + MV( x-dir-factor * distance-offset, y-dir-factor * distance-offset );

MVL1(v0x, v0y) = MVPL1 (v0px, v0py) + MV( - x-dir-factor * distance-offset, - y-dir-factor * distance-offset );

# Simulation Results

The proposed method is implemented on top of VTM-2.0.1 plus affine merge common base software, as provided in CE4.2.1 [1][4]. Simulation tests were performed under common test condition in [2].

VTM-2.0.1 affine merge common base software with common test condition is used as anchor. VTM-2.0.1 software [3] with common test condition is used as a second reference.

## Simulation result

Simulation results are provided in the following tables. It was reported that there are 0.32% and 0.09% BD-rate gain on random access and low delay B configuration, respectively.

Table 2 -  Simulation results of Random Access Configuration

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 Affine Merge Common Base |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.37% | -0.32% | 134% | 104% | -1.37% | -0.95% | -1.21% | 139% | 111% |
| Class A2 | -0.47% | -0.46% | -0.52% | 144% | 104% | -1.47% | -1.44% | -1.65% | 152% | 111% |
| Class B | -0.33% | -0.21% | -0.27% | 140% | 103% | -1.00% | -0.77% | -0.97% | 153% | 112% |
| Class C | -0.12% | -0.12% | -0.16% | 138% | 101% | -0.60% | -0.57% | -0.70% | 157% | 123% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.32% | -0.27% | -0.30% | 139% | 103% | -1.06% | -0.89% | -1.08% | 151% | 114% |
| Class D | -0.07% | 0.03% | 0.09% | 143% | 104% | -0.75% | -0.64% | -0.69% | 162% | 135% |
| Class F (optional) | -0.08% | -0.04% | -0.14% | 129% | 101% | -0.22% | -0.22% | -0.28% | 142% | 114% |

Table 3 - Simulation results of Low Delay B Configuration

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 Affine Merge Common Base |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.19% | 0.16% | 145% | 100% | -0.11% | -0.19% | 0.16% | 145% | 100% |
| Class C | -0.03% | 0.02% | -0.24% | 142% | 105% | -0.03% | 0.02% | -0.24% | 142% | 105% |
| Class E | -0.12% | -0.34% | -1.02% | 156% | 102% | -0.12% | -0.34% | -1.02% | 156% | 102% |
| Overall | -0.09% | -0.16% | -0.27% | 147% | 102% | -0.09% | -0.16% | -0.27% | 147% | 102% |
| Class D | 0.11% | 0.58% | -0.27% | 140% | 100% | 0.11% | 0.58% | -0.27% | 140% | 100% |
| Class F (optional) | -0.14% | -0.31% | -0.09% | 133% | 102% | -0.14% | -0.31% | -0.09% | 133% | 102% |

# Conclusion

It is reported that the proposed affine merge mode with prediction offset method has 0.32% and 0.09% BD-rate improvement compared with VTM-2.0.1 affine merge common base. It is proposed to adopt the proposed method into VTM or study in core experiments.

# References

[1] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[3] VTM-2.0.1 software: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-2.0.1

[4] VTM-2.0.1 with Affine Merge Common Base: https://vcgit.hhi.fraunhofer.de/JVET-K-CE4/VVCSoftware_VTM/tree/CE4.2.1

[5] S. Jeong, M. W. Park, C. Kim (Samsung), “CE4 Ultimate motion vector expression in JVET-J0024 (Test 4.2.9)”, JVET-K0115, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).