| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0325 |
| --- | --- |

| Title: | CE7-related: High throughput coefficient coding depending on the sub-block size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung-Ah Choi Jin Heo Sunmi Yoo Jangwon Choi Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 jungah.choi@lge.com jin78.heo@lge.com sunmi.yoo@lge.com jangwon84.choi@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution targets on improving throughput of the arithmetic coding engine by reducing the number of context coded bins for coefficients level coding. Specifically, in the proposed method, the number of worst case context coded bins is dependent on the sub-block size because current VTM 2.0.1 allows two sub-block sizes (16 and 4). The proposed methods are tested on top of CE7.1.3a and CE7.1.3b where the order of par_level_flag and rem_abs_gt1_flag is swapped.

Experiment results of the proposed methods are summarized as follows.

Method I + CE7.1.3a
- Test #1: AI  0.05%, 0.13%, 0.18% (Y, Cb, Cr) at 109% encoder and 99% decoder run time;
        RA  0.04%, 0.15%, 0.15% (Y, Cb, Cr) at 103% encoder and 96% decoder run time;
- Test #2: AI  0.07%, 0.14%, 0.20% (Y, Cb, Cr) at 110% encoder and 99% decoder run time;
        RA  0.05%, 0.24%, 0.24% (Y, Cb, Cr) at 103% encoder and 96% decoder run time;
- Test #3: AI  0.14%, 0.78%, 0.86% (Y, Cb, Cr) at 110% encoder and 98% decoder run time;
        RA  0.14%, 1.37%, 1.41% (Y, Cb, Cr) at 103% encoder and 99% decoder run time.

Method II + CE7.1.3b
- Test #4: AI  -0.17%, -0.03%, -0.01% (Y, Cb, Cr) at 108% encoder and 98% decoder run time;
        RA  -0.07%, 0.02%, -0.02% (Y, Cb, Cr) at 113% encoder and 102% decoder run time;
- Test #5: AI  -0.14%, 0.01%, 0.09% (Y, Cb, Cr) at 108% encoder and 98% decoder run time;
        RA  -0.06%, 0.17%, 0.23% (Y, Cb, Cr) at 113% encoder and 102% decoder run time;
- Test #6: AI  -0.07%, 0.63%, 0.74% (Y, Cb, Cr) at 108% encoder and 99% decoder run time;
        RA  0.03%, 1.20%, 1.29% (Y, Cb, Cr) at 102% encoder and 96% decoder run time.

It is further reported that all the modules and processes of current arithmetic coding engine are kept unchanged and no additional module is needed in the proposed approaches.

Introduction

In the current VTM2.0.1 software, the transform coefficient levels of a sub-block are coded in four passes over the scan positions [1]:

- pass 1: the following flags are transmitted for each scan position using the regular mode: 
sig_coeff_flag and, when sig_coeff_flag is equal to 1, par_level_flag and rem_abs_gt1_flag;

- pass 2: for all scan positions with rem_abs_gt1_flag equal to 1, rem_abs_gt2_flag is coded using the regular mode of the arithmetic coding engine;

- pass 3: for all scan positions with rem_abs_gt2_flag equal to 1, the non-binary syntax element abs_remainder is coded in the bypass mode of the arithmetic coding engine;

- pass 4: for all scan positions with sig_coeff_flag equal to 1, a sign_flag is coded in the bypass mode of the arithmetic coding engine.

Among the six syntax elements, the bins of sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, and rem_abs_gt2_flag are encoded with adaptive context models (regular mode); binarized bins of abs_remainder and sign_flag are encoded through bypass mode with fixed equal probability model. With the current design, three or more bins are encoded with adaptive context model for a coefficient larger than zero. It is well known that context-based bin coding is one of the main bottlenecks of the throughput of the arithmetic coding engine.

In residual coding, a transform unit is partitioned into several subblocks. A 4x4 transform unit consists of exactly one subblock. However, for chroma blocks, transform units of size 4x2, 2x4, 2x2 are partitioned into nonoverlapping 2x2 subblocks. Figure 1 shows an example of quantized coefficients in a 2×2 chroma sub-block, and the encoded symbols for these coefficients in inverse scan order are summarized.

Figure 1. Data dependency on parallel processing

The coefficients level information is coded in inverse scan order, which means the high frequency components of transformed coefficients are scanned first. In such design, the amplitude of the first few coefficients in the reverse scan order tends to be small. Furthermore if they are significant, with high probability, the absolute level is 1 or 2. For these coefficients, it is beneficial to use pass 1 (sig_coeff_flag, par_level_flag, rem_abs_gt1_flag) and pass 2(rem_abs_gt2_flag) to reduce the length of binarized bins and the flags can be efficiently encoded with well adapted context model. However, after encountering a certain number of greater than 1 coefficients, due to reverse scanning, it is much likely that the remaining significant coefficients also have high magnitudes. In such a case, using pass 1 and pass 2 is not likely to improve the compression performance while increasing the number of context-coded bins.

This proposal aims to improve the throughput performance of the arithmetic coding engine by reducing the number of context-coded bins.

Proposed method

Method I

In this contribution, we propose to encode pass 1 and pass 2 only for a few starting non-zero coefficients in inverse scan order.

- Encode pass 1 (sig_coeff_flag, par_level_flag, rem_abs_gt1_flag) until the number of encoded context-coded bins are smaller or equal than M in a sub-block

- Encode pass 2 (rem_abs_gt2_flag) for the first N coefficients with rem_abs_gt1_flag being 1 in a sub-block

We propose three M and N combinations offering different trade-offs of throughput and compression efficiency. Note that M and N are dependent on the sub-block size. We tested different combinations of M (from 1 to 8) and N (from 1 to 8) to confirm that the proposed methods provide the best trade-off of performance and throughput improvement. The proposed method is implemented on top of CE7.1.3.a [2].

Table 1. Number of context coded bins for Method I

| Subblock | Test #1 |  | Test #2 |  | Test #3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | M | N | M | N | M | N |
| 4x4 | 34 | 2 | 34 | 2 | 34 | 2 |
| 2x2 | 7 | 1 | 4 | 0 | 0 | 0 |

Table 2 shows the symbols to be coded for the example 2x2 sub-block shown in Figure 1 with Test #2 (M = 4, N = 0). Compared with the coded symbols in Figure 1 (the current VTM2.0.1 method), 9 context coded bins at the positions marked as ‘X’ are removed. The value of abs_remainder is adjusted appropriately.

Table 2. Coded symbols for the coefficients in an chroma 2x2 sub-block with Method I (M = 4, N = 0)

| scan_pos | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- |
| coefficients | 0 | -3 | -5 | 6 |
| sig_coeff_flag | 0 | 1 | X | X |
| par_level_flag |  | 0 | X | X |
| rem_abs_gt1_flag |  | 1 | X | X |
| rem_abs_gt2_flag |  | X | X | X |
| abs_remainder |  | 0 | 5 | 6 |
| coeff_sign_flag |  | 1 | 1 | 0 |

Method II

CE7.1.3b [2] proposes several schemes to enhance the throughput of the arithmetic coding engine. One of their proposed schemes is to swap the order of par_level_flag and rem_abs_gt1_flag. In this contribution, par_level_flag is only present if rem_abs_gt1_flag is equal to 1. By enabling CE7.1.3b, combinations in Table 3 are the appropriate trade-offs.

In VTM2.0.1, sig_coeff_flag, rem_abs_gt1_flag, and par_level_flag are coded in one pass. As shown in Table 4, if (M – the number of encoded context coded bins) < 3, the encoding process of pass 1 is stopped. Accordingly, coding pass 1 process is stopped, the encoding process of pass 2 is also stopped.

Table 3. Number of context coded bins for Method II

| Subblock | Test #4 |  | Test #5 |  | Test #6 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | M | N | M | N | M | N |
| 4x4 | 28 | 4 | 28 | 4 | 28 | 4 |
| 2x2 | 6 | 2 | 3 | 1 | 0 | 0 |

Table 4. Coded symbols for the coefficients in an chroma 2x2 sub-block with Method II (M = 6, N = 2)

| scan_pos | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- |
| coefficients | 0 | -3 | -5 | 6 |
| sig_coeff_flag | 0 | 1 | X | X |
| rem_abs_gt1_flag |  | 1 | X | X |
| par_level_flag |  | 1 | X | X |
| rem_abs_gt2_flag |  | 0 | X | X |
| abs_remainder |  |  | 5 | 6 |
| coeff_sign_flag |  | 1 | 1 | 0 |

Experimental results

The proposed method has been implemented on the VTM2.0.1 reference software. The simulations were performed following the common test conditions defined in JVET-K1010 [3]. In every case, the anchor is the VTM2.0.1 software.

Method I

Table 5. BD-Rate performance of Test #1

|  | All Intra 10 |  |  | Random Access 10 |  |  | Low Delay B 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 0.11% | 0.08% | 0.18% | 0.07% | 0.03% | 0.25% |  |  |  |
| Class A2 | 0.00% | 0.09% | -0.02% | 0.02% | 0.23% | 0.02% |  |  |  |
| Class B | 0.03% | 0.16% | 0.21% | 0.02% | 0.17% | 0.15% | 0.12% | 0.26% | 0.37% |
| Class C | 0.06% | 0.16% | 0.35% | 0.05% | 0.17% | 0.17% | 0.20% | 0.36% | 0.15% |
| Class E | 0.05% | 0.10% | 0.09% |  |  |  | 0.36% | -0.47% | -0.08% |
| Overall | 0.05% | 0.13% | 0.18% | 0.04% | 0.15% | 0.15% | 0.21% | 0.11% | 0.18% |
| Class D | 0.02% | 0.34% | 0.03% | 0.14% | 0.02% | 0.37% | 0.12% | 0.57% | 0.35% |
| Enc Time | 109% |  |  | 103% |  |  | 103% |  |  |
| Dec Time | 99% |  |  | 96% |  |  | 99% |  |  |

Table 6. BD-Rate performance of Test #2

|  | All Intra 10 |  |  | Random Access 10 |  |  | Low Delay B 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 0.13% | 0.11% | 0.13% | 0.12% | 0.22% | 0.27% |  |  |  |
| Class A2 | 0.04% | 0.13% | 0.09% | 0.02% | 0.28% | 0.12% |  |  |  |
| Class B | 0.04% | 0.19% | 0.20% | 0.04% | 0.10% | 0.24% | 0.10% | 0.31% | 0.33% |
| Class C | 0.10% | 0.16% | 0.36% | 0.05% | 0.40% | 0.30% | 0.12% | 0.41% | 0.58% |
| Class E | 0.05% | 0.07% | 0.16% |  |  |  | 0.18% | -0.69% | -1.47% |
| Overall | 0.07% | 0.14% | 0.20% | 0.05% | 0.24% | 0.24% | 0.13% | 0.09% | -0.04% |
| Class D | 0.08% | 0.49% | 0.16% | 0.09% | -0.11% | 0.42% | 0.16% | 0.97% | 1.07% |
| Enc Time | 110% |  |  | 103% |  |  | 102% |  |  |
| Dec Time | 99% |  |  | 96% |  |  | 99% |  |  |

Table 7. BD-Rate performance of Test #3

|  | All Intra 10 |  |  | Random Access 10 |  |  | Low Delay B 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 0.18% | 0.39% | 0.18% | 0.19% | 0.36% | 0.51% |  |  |  |
| Class A2 | 0.14% | 0.75% | 0.71% | 0.12% | 1.29% | 1.22% |  |  |  |
| Class B | 0.08% | 0.84% | 0.97% | 0.10% | 1.37% | 1.38% | 0.14% | 2.07% | 2.20% |
| Class C | 0.23% | 1.32% | 1.71% | 0.19% | 2.18% | 2.27% | 0.23% | 2.61% | 2.96% |
| Class E | 0.10% | 0.40% | 0.39% |  |  |  | 0.20% | 0.86% | 0.96% |
| Overall | 0.14% | 0.78% | 0.86% | 0.14% | 1.37% | 1.41% | 0.19% | 1.95% | 2.15% |
| Class D | 0.23% | 1.91% | 1.97% | 0.19% | 2.48% | 3.13% | 0.24% | 4.66% | 4.06% |
| Enc Time | 110% |  |  | 103% |  |  | 103% |  |  |
| Dec Time | 98% |  |  | 96% |  |  | 99% |  |  |

Method II

Table 8. BD-Rate performance of Test #4

|  | All Intra 10 |  |  | Random Access 10 |  |  | Low Delay B 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -0.12% | 0.03% | 0.06% | -0.04% | 0.10% | 0.14% |  |  |  |
| Class A2 | -1.02% | -0.54% | -0.65% | -0.19% | 0.00% | -0.25% |  |  |  |
| Class B | -0.04% | 0.07% | 0.16% | -0.12% | -0.05% | -0.14% | -0.27% | -0.19% | -0.22% |
| Class C | 0.04% | 0.11% | 0.18% | 0.06% | 0.06% | 0.19% | -0.10% | -0.02% | 0.33% |
| Class E | 0.12% | 0.08% | 0.05% |  |  |  | 0.12% | 0.37% | -0.55% |
| Overall | -0.17 | -0.03 | -0.01 | -0.07% | 0.02% | -0.02% | -0.12% | 0.01% | -0.12% |
| Class D | 0.07% | 0.24% | 0.04% | 0.09% | -0.05% | 0.40% | -0.07% | 0.14% | -0.25% |
| Enc Time | 108% |  |  | 113% |  |  | 102% |  |  |
| Dec Time | 98% |  |  | 102% |  |  | 99% |  |  |

Table 9. BD-Rate performance of Test #5

|  | All Intra 10 |  |  | Random Access 10 |  |  | Low Delay B 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -0.10% | 0.01% | 0.02% | -0.02% | 0.21% | 0.26% |  |  |  |
| Class A2 | -0.96% | -0.50% | -0.56% | -0.22% | 0.08% | 0.12% |  |  |  |
| Class B | -0.02% | 0.08% | 0.15% | -0.10% | 0.01% | 0.13% | -0.29% | 0.23% | 0.05% |
| Class C | 0.09% | 0.15% | 0.47% | 0.09% | 0.41% | 0.41% | -0.08% | 0.16% | 0.48% |
| Class E | 0.14% | 0.18% | 0.20% |  |  |  | 0.14% | -0.44% | -0.68% |
| Overall | -0.14 | 0.01 | 0.09 | -0.06% | 0.17% | 0.23% | -0.11% | 0.04% | 0.01% |
| Class D | 0.11% | 0.21% | 0.18% | 0.11% | 0.08% | 0.74% | -0.01% | 0.44% | 0.39% |
| Enc Time | 108% |  |  | 113% |  |  | 102% |  |  |
| Dec Time | 98% |  |  | 102% |  |  | 98% |  |  |

Table 10. BD-Rate performance of Test #6

|  | All Intra 10 |  |  | Random Access 10 |  |  | Low Delay B 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | -0.05% | 0.32% | 0.02% | 0.06% | 0.44% | 0.43% |  |  |  |
| Class A2 | -0.86% | 0.15% | 0.05% | -0.11% | 1.16% | 1.10% |  |  |  |
| Class B | 0.02% | 0.73% | 0.97% | -0.05% | 1.06% | 1.23% | -0.25% | 1.72% | 1.83% |
| Class C | 0.20% | 1.27% | 1.72% | 0.22% | 1.97% | 2.14% | 0.02% | 2.41% | 2.91% |
| Class E | 0.17% | 0.38% | 0.47% |  |  |  | 0.04% | 0.78% | 0.29% |
| Overall | -0.07 | 0.63 | 0.74 | 0.03% | 1.20% | 1.29% | -0.09% | 1.72% | 1.81% |
| Class D | 0.26% | 1.78% | 2.06% | 0.19% | 2.48% | 3.13% | 0.03% | 4.73% | 3.95% |
| Enc Time | 108% |  |  | 102% |  |  | 102% |  |  |
| Dec Time | 99% |  |  | 96% |  |  | 98% |  |  |

Complexity analysis

Context coded bin

The throughput performance between VTM2.0.1 and proposed methods in worst-case scenario has also been compared in Table 11.

Table 11. Maximum context coded bins (worst case)

| Subblock | VTM2.0.1 |  | Method I |  |  |  |  |  | Method II |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test #1 |  | Test #2 |  | Test #3 |  | Test #4 |  | Test #5 |  | Test #6 |  |
| 4x4 | 48 | 16 | 34 | 2 | 34 | 2 | 34 | 2 | 28 | 4 | 28 | 4 | 28 | 4 |
| 2x2 | 12 | 4 | 7 | 1 | 5 | 0 | 0 | 0 | 6 | 2 | 3 | 1 | 0 | 0 |

Context model reduction

In some cases of proposed methods (Test #2, Test #3, and Test #6), rem_abs_gt2_flag is not coded, so 11 context models for 2x2 chroma sub-block are saved.

Encoding pass reduction

In VTM2.0.1, as shown in Section I, total four encoding passes exist. In Test #2, Test #3, and Test #6, rem_abs_gt2_flag is not encoded in a sub-block, therefore one encoding pass (i.e., pass 2) can be removed. In addition, according to CE7.1.3b, pass 4 (for encoding abs_remainder) is divided to three sub-passes. However, using the proposed methods (Test #2, Test #3, and Test #6), one of three sub-passes for abs_remainder can be removed.

Reference

- H. Schwarz, et al., “Non-CE7: Alternative Entropy Coding for Dependent Quantization,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0072, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- H. Schwarz, et al., “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1027, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).