| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0327-v1 |
| --- | --- |

| Title: | CE11: Long-tap deblocking filter for luma and chroma (CE11.1.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 masaru.ikeda@sony.com teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

The contribution reports the results for CE11.1.6 [1] which is a HEVC style extended luma and chroma long-tap filter for large blocks. The motivation here is for blocking artifact reduction resulting from the use of large transforms and large block sizes. It is asserted that the proposed deblocking approach improves subjective quality at low bit-rates when compared with the deblocking process existing in the VTM software. The BD-Rate number for Y, U & V reportedly shows -0.03%, -0.66% & -0.59% at AI, -0.08%, -1.17% & -1.10% at RA, -0.10%, -1.53% & -1.66% at LDB and -0.09%, -1.69% & -1.71% at LDP over VTM-2.0.1 with ALF switched on, respectively. For ALF switched off case, the BD-Rate number for Y, U & V reportedly shows -0.04%, -0.81% & -0.66% at AI, -0.12%, -1.36% & -1.23% at RA, -0.01%, -1.63% & -1.96% at LDB and -0.13%, -1.89% & -2.10% at LDP over VTM-2.0.1.

Introduction

The test addresses an aspect of the long-tap deblocking filter in CE11 [2]. This technology was originally proposed as a part of JVET-J0028 [3] in the CfP response in San Diego meeting. The details are described in section 2. The experimental results are shown in section 3, the md5sums for subjective data are listed in section 4 and the conclusion is provided at the end.

Proposed technology

HEVC deblocking filters are extended with longer deblocking filters for luma and chroma. 7 samples on each side of the boundary is filtered when the minimal length orthogonal to block boundary is >= 32, sum of absolute pixel difference and also Laplacians, and comparison with absolute value of pixel difference across boundary in comparison with tc. The decision which has two additional conditions is also modified, accordingly. The first one is for large block decision since the proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 32. The second one is for proposed wide strong filter. The decision is extended based on HEVC strong filter decision (xUseStrongFiltering) as follows.

dp0 = xCalcDP( R[0] ); dp1 = xCalcDP( R[-2] ); dp2 = xCalcDP( R[-4] );

dq0 = xCalcDQ( R[0] ); dq1 = xCalcDQ( R[2] ); dq2 = xCalcDQ( R[4] );

dd0 = 2 * (dp0+dq0); dd2 = 2 * (dp1+dq1); dd4 = 2 * (dp2+dq2);

bdd = ( dd0 < (beta>>2) && dd2 < (beta>>2) && dd4 < (beta>>2) );

dd_strong0 = abs(R[-8] – R[-1]) + abs(R[0] – R[7]);

dd_strong1 = abs(R[-8] – R[-3]) + abs(R[2] – R[7]);

dd_strong2 = abs(R[-8] – R[-5]) + abs(R[4] – R[7]);

dd_strong_flag =

( (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) );

If ( dd_strong_flag && bdd && (abs(R[-1]-R[0]) < ((tc*5+1)>>1)) ) is TRUE, and then the following filter is applied to P side in the same with Q side.

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

In this conribution, long-tap filter is also proposed for chroma. The decision which has three additional conditions is also modified, accordingly. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 8 in chorma sample domain. The second and third one are basically the same as for HEVC luma decison, which are on/off decision and strong filter decision, respectively.

In the first decision, boundary strength (bS) is modified for chorma filtering as follows.

| Conditions | Y | U | V |
| --- | --- | --- | --- |
| At least one of the adjacent blocks is intra | 2 | 2 | 2 |
| At least one of the adjacent blocks has non-zero transform coefficients | 1 | 1 | 1 |
| Absolute difference between the motion vectors that belong to the adjacent blocks is greater than or equal to one integer luma sample | 1 | N/A | N/A |
| Motion prediction in the adjacent blocks refers to vectors is different | 1 | N/A | N/A |
| Otherwise | 0 | 0 | 0 |

The second and third decision is basically the same as HEVC luma strong filter decision as follows.

The second decision:

d < beta

d = Abs(p2,0−2*p1,0+ p0,0) + Abs( q2,0−2*q1,0+q0,0) + Abs(p2,1−2*p1,1+p0,1) + Abs(q2,1−2*q1,1+q0,1);

The third decision:

xUseStrongFiltering( );    // same as luma’s decision

If those decisions are satisfied, and then the following filter is applied to P side in the same with Q side.

p2′= (3*p3+2*p2+p1+p0+q0+4) >> 3

p1′= (2*p3+p2+2*p1+p0+q0+q1+4) >> 3

p0′= (p3+p2+p1+2*p0+q0+q1+q2+4) >> 3

The decision which has three additional conditions is also modified, accordingly. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 8. The second and third one are basically similar to luma’s decision, which are on/off decision and strong filter decision, respectively.

Experimental Results

In the CE, the following two conditions are evaluated: 1) VTM-2.0.1 with ALF switched on 2) VTM-2.0.1 with ALF switched off. And also LDP is mandatory in addition to CTC [4].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -1.58% | -1.06% | 100% | 102% |
| Class A2 | -0.02% | -0.30% | -0.31% | 97% | 102% |
| Class B | -0.02% | -0.55% | -0.48% | 100% | 103% |
| Class C | -0.01% | -0.38% | -0.47% | 101% | 102% |
| Class E | -0.06% | -0.66% | -0.74% | 100% | 101% |
| Overall | -0.03% | -0.66% | -0.59% | 100% | 102% |
| Class D | 0.00% | -0.19% | -0.19% | 101% | 101% |
| Class F (optional) | -0.02% | -0.50% | -0.72% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -2.26% | -2.04% | 102% | 102% |
| Class A2 | -0.09% | -0.97% | -0.87% | 97% | 102% |
| Class B | -0.03% | -1.07% | -1.02% | 99% | 102% |
| Class C | -0.01% | -0.65% | -0.66% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -1.17% | -1.10% | 100% | 103% |
| Class D | 0.00% | -0.37% | -0.44% | 102% | 103% |
| Class F (optional) | -0.01% | -0.75% | -0.79% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -1.81% | -1.93% | 96% | 102% |
| Class C | -0.02% | -0.79% | -1.02% | 97% | 102% |
| Class E | -0.35% | -2.07% | -2.05% | 96% | 99% |
| Overall | -0.10% | -1.53% | -1.66% | 96% | 101% |
| Class D | -0.01% | -0.52% | -0.78% | 100% | 100% |
| Class F (optional) | -0.15% | -0.49% | -0.63% | 96% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -1.71% | -2.06% | 100% | 102% |
| Class C | -0.02% | -0.80% | -1.04% | 100% | 104% |
| Class E | -0.17% | -2.83% | -2.04% | 100% | 106% |
| Overall | -0.09% | -1.69% | -1.71% | 100% | 104% |
| Class D | 0.02% | -0.79% | -1.15% | 100% | 103% |
| Class F (optional) | -0.04% | -0.86% | -0.41% | 100% | 103% |

Table 1 Coding performance over VTM-2.0.1 with ALF switched on

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1_NOALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -2.49% | -1.47% | 100% | 103% |
| Class A2 | -0.02% | -0.32% | -0.25% | 100% | 102% |
| Class B | -0.02% | -0.54% | -0.47% | 101% | 102% |
| Class C | -0.01% | -0.29% | -0.42% | 100% | 102% |
| Class E | -0.07% | -0.76% | -0.89% | 100% | 106% |
| Overall | -0.04% | -0.81% | -0.66% | 100% | 103% |
| Class D | 0.00% | -0.14% | -0.13% | 100% | 101% |
| Class F (optional) | -0.02% | -0.35% | -0.58% | 99% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1_NOALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -3.29% | -2.69% | 83% | 104% |
| Class A2 | -0.12% | -0.83% | -0.83% | 81% | 103% |
| Class B | -0.05% | -1.08% | -1.01% | 96% | 103% |
| Class C | -0.04% | -0.64% | -0.72% | 94% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -1.36% | -1.23% | 90% | 103% |
| Class D | -0.02% | -0.45% | -0.47% | 94% | 103% |
| Class F (optional) | -0.01% | -0.70% | -0.83% | 89% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1_NOALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -1.77% | -2.53% | 97% | 102% |
| Class C | -0.02% | -1.02% | -1.13% | 100% | 100% |
| Class E | 0.04% | -2.21% | -2.11% | 100% | 102% |
| Overall | -0.01% | -1.63% | -1.96% | 99% | 101% |
| Class D | -0.03% | -0.74% | -0.28% | 100% | 101% |
| Class F (optional) | 0.01% | -0.86% | -1.11% | 100% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1_NOALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -2.52% | -2.62% | 100% | 102% |
| Class C | -0.01% | -0.64% | -1.02% | 99% | 103% |
| Class E | -0.26% | -2.51% | -2.68% | 94% | 104% |
| Overall | -0.13% | -1.89% | -2.10% | 98% | 103% |
| Class D | -0.03% | -1.36% | -1.13% | 98% | 103% |
| Class F (optional) | 0.01% | -1.03% | -1.40% | 93% | 103% |

Table 2 Coding performance over VTM-2.0.1 with ALF switched off

Md5sums for subjective test data

The md5sums of reconstructed files for the subjective viewing at ALF switched on and off are provided in Table 3 and Table 4, respectively.

| Name | Mode | QP | Md5sums |
| --- | --- | --- | --- |
| FoodMarket | RA | 34 | af3c58e3ff20672fe623b3ce0c543d37 |
| Park Running | RA | 34 | a18bcfb14d36a79b772173ae6d02caa9 |
| Campfire | RA | 34 | a548fa674315a7fcb90f37aca4adae0f |
| Kimono | LDB | 34 | 97ddd5799c0565305ca9dbede2cb7499 |
| KristenAndSara | LDP | 34 | e0102a9e7b141b11752b64240f9aeced |
| FoodMarket | RA | 39 | 28a3bb61d00dc5919966868e1cc6a68a |
| Park Running | RA | 39 | 4f81378cdf4cf0473e4a02db1df14d51 |
| Campfire | RA | 39 | fe07e42d3ac61a72ce91ef5f6fbb9448 |
| Kimono | LDB | 39 | b6850d04980bb26786d85eb0231aff33 |
| KristenAndSara | LDP | 39 | 650a51f5d80ef8ef12b346d86d663dfd |

Table 3 Md5sums at ALF switched on

| Name | Mode | QP | Md5sums |
| --- | --- | --- | --- |
| FoodMarket | RA | 34 | 4f4d197b06ebb95b571aac6016f1ced9 |
| Park Running | RA | 34 | 8feba63ccf22018019e6fb780333a74f |
| Campfire | RA | 34 | e647bf5b7413b468ad25c445be08638b |
| Kimono | LDB | 34 | 1db9231725df25442fa582e105a64451 |
| KristenAndSara | LDP | 34 | 5f20f6bf0358947e914fd005a182d3fe |
| FoodMarket | RA | 39 | 0f4eee03fef7e7ef41f6a760c59088d0 |
| Park Running | RA | 39 | feeb1d2b5d577f9ed6f756a0e0d97908 |
| Campfire | RA | 39 | 3d6d78917fcef6118654905736f5ef91 |
| Kimono | LDB | 39 | df2915347a778265dc9286e14ad4e624 |
| KristenAndSara | LDP | 39 | 7febad73a39431010890c3be30258d8e |

Table 4 Md5sums at ALF switched off

Conclusion

This contribution reports the performance of CE11.1.6 which is a HEVC style extended luma and chroma long-tap filter for large blocks. It is asserted that the proposed method gives us not only subjective quality improvement but also small objective gains. It is proposed to adopt the method into next test mode and working draft.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- “CE2: Long-tap deblocking filter for luma and chroma (CE2-2.1.6)”, JVET-K0152

- “Description of Core Experiment 11 (CE11): Deblocking”, JVET-K1031

- “Description of SDR and HDR video coding technology proposal by Sony”, JVET-J1028

- “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010