| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0348-v1 |
| --- | --- |

| Title: | CE13: In-loop filters disabled across face discontinuities (Test 4.1.b and Test 4.1.d) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JEVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng-Yen Lin, Lin Liu, Jian-Liang Lin, Hung-Chih Lin, Shen-Kai Chang, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {Sheng-Yen.Lin, Lin.Liu, jl.lin, hungchih.lin, shenkai.chang, yc.zhang, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of Test 4.1.b and Test 4.1.d in CE13: HEC with in-loop filter disabled across face discontinuities. In terms of end-to-end WS-PSNR-Y/U/V, the experimental results reportedly demonstrate that Test 4.1.b achieves an average -0.24%/-0.46%/-0.47% and -0.56%/-0.01%/0.13% BD-rate saving compared to HEC and PHEC, respectively, and Test 4.1.d achieves an average -0.25%/-1.00%/-0.74% and -0.56%/-0.55%/-0.14% BD-rate saving compared to HEC and PHEC, respectively. In addition, the subjective quality of Test 4.1.b and Test 4.1.d are noticeably improved by alleviating the seam artifacts compared to HEC.

# Introduction

In the last meeting, Core Experiment 13 (CE13) [1] was established to study removing subjective artifacts at face boundaries for the 360° video category. This contribution reports the results of de-blocking and SAO filters disabled across face discontinuities proposed in JVET-K0142 [2], Test 4.1.b, and the combination with ALF disabled across face discontinuities, Test 4.1.d, implemented on the hybrid equi-angular cubemap (HEC) projection [3].

The in-loop filters in a conventional video coding system is used to remove undesired artifacts and enhance encoding quality. For a 360° video, the layout for a specific projection format usually has one or multiple discontinuous edges. Applying the in-loop filters on these discontinuous edges directly may result in poor visual quality and coding efficiency because the accessed pixels (to be referenced and/or filtered) cross discontinuous edges are jointly processed (filtered). To address this problem, the conventional in-loop filters are modified such that the discontinuous edge can be properly processed.

# Proposed Method

In this contribution, the in-loop filters including the de-blocking filter, SAO and the ALF, are performed necessarily changes to avoid inappropriately filtering at the discontinuous edge.

## De-blocking Filter

The details of the modification of the de-blocking filter are proposed in JVET-K0142 [2] and JVET-L0347 [4].

## Sample Adaptive Offset Filter

As proposed in JVET-K0142 [2], when the referenced pixels of current pixel in the SAO filtering process cross the discontinuous edge, the proposed modification disables the SAO filtering process of the current pixel to avoid inadequate filtering process.

Fig. 1. The SAO filtering process is disabled when the referenced pixels cross the discontinuous edge

## Adaptive Loop Filter

The details of the modification of the ALF are proposed in JVET-L0232 [5].

# Experimental Results

The simulations of the Test 4.1.b and Test 4.1.d are conducted on HEC. The software is implemented on the 360Lib-7.0 [6] and the BMS-2.0.1 (with fix BMS ticket #78 and #79) [7] followed by the JVET common test conditions and evaluation procedures for 360° video [8] under the VTM-2.0 [9] encoder configurations. The resolution of each face is set to 1280x1280, which yields the frame resolution is 3840x2560. As compared to HEC and PHEC, the BD-rate performance, in terms of end-to-end WS-PSNR, provided by Test 4.1.b and Test 4.1.d are listed in Table I and Table II, respectively.

Table I. BD-rate performance for Test 4.1.b compared to HEC and PHEC under VTM-2.0

|  | Anchor : HEC |  |  | Anchor : PHEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.11% | -0.46% | -0.31% | -0.07% | 0.54% | 0.83% |
| ChairLift | -0.29% | -0.05% | -0.21% | -0.40% | 0.18% | 0.07% |
| KiteFlite | -0.18% | -0.14% | -0.25% | -0.63% | -0.02% | 0.12% |
| Harbor | -0.66% | -0.92% | -1.44% | -0.63% | 0.58% | 0.40% |
| Trolley | -0.17% | -0.48% | -0.20% | -0.83% | 0.00% | -0.11% |
| GasLamp | -0.23% | -0.70% | -0.48% | -0.71% | -0.33% | -0.01% |
| Balboa | -0.23% | -0.27% | -0.19% | -0.35% | -0.24% | 0.28% |
| Broadway | -0.28% | 0.08% | -0.20% | -0.35% | 0.18% | 0.35% |
| Landing2 | -0.21% | -1.50% | -1.06% | -0.71% | -0.48% | -0.22% |
| BranCastle2 | -0.06% | -0.15% | -0.38% | -0.87% | -0.52% | -0.41% |
| Class S1 | -0.27% | -0.46% | -0.48% | -0.55% | 0.16% | 0.22% |
| Class S2 | -0.20% | -0.46% | -0.46% | -0.57% | -0.26% | 0.00% |
| Overall | -0.24% | -0.46% | -0.47% | -0.56% | -0.01% | 0.13% |

Table II. BD-rate performance for Test 4.1.d compared to HEC and PHEC under VTM-2.0

|  | Anchor : HEC |  |  | Anchor : PHEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.16% | -1.16% | -0.52% | -0.12% | -0.16% | 0.62% |
| ChairLift | -0.31% | -0.49% | -0.32% | -0.43% | -0.25% | -0.04% |
| KiteFlite | -0.20% | -0.60% | -0.52% | -0.65% | -0.47% | -0.15% |
| Harbor | -0.64% | -1.60% | -1.82% | -0.61% | -0.11% | 0.02% |
| Trolley | -0.18% | -0.98% | -0.60% | -0.84% | -0.50% | -0.52% |
| GasLamp | -0.25% | -1.02% | -0.74% | -0.72% | -0.66% | -0.28% |
| Balboa | -0.24% | -0.44% | -0.01% | -0.36% | -0.41% | 0.45% |
| Broadway | -0.22% | -0.59% | -0.31% | -0.29% | -0.49% | 0.24% |
| Landing2 | -0.23% | -2.87% | -2.08% | -0.73% | -1.86% | -1.25% |
| BranCastle2 | -0.05% | -0.26% | -0.46% | -0.86% | -0.62% | -0.49% |
| Class S1 | -0.29% | -0.97% | -0.76% | -0.56% | -0.36% | -0.06% |
| Class S2 | -0.18% | -1.04% | -0.72% | -0.56% | -0.84% | -0.26% |
| Overall | -0.25% | -1.00% | -0.74% | -0.56% | -0.55% | -0.14% |

# Subjective Quality Testing

In Fig. 2 to Fig. 10, the cropped viewport images of HEC, Test 4.1.b and Test 4.1.d are demonstrated (under VTM-2.0 with QP = 37) in order to evaluate the seam artifacts appeared at the discontinuous edge.

Fig. 2 The subjective quality of Balboa frame 28 (HEC)

Fig. 3 The subjective quality of Balboa frame 28 (Test 4.1.b)

Fig. 4 The subjective quality of Balboa frame 28 (Test 4.1.d)

Fig. 5 The subjective quality of SkateboardInLot frame 8 (HEC)

Fig. 6 The subjective quality of SkateboardInLot frame 8 (Test 4.1.b)

Fig. 7 The subjective quality of SkateboardInLot frame 8 (Test 4.1.d)

Fig. 8 The subjective quality of ChairliftRide frame 4 (HEC)

Fig. 9 The subjective quality of ChairliftRide frame 4 (Test 4.1.b)

Fig. 10 The subjective quality of ChairliftRide frame 4 (Test 4.1.d)

# Conclusion

In this contribution, the performance of HEC with the in-loop filters disabled across face discontinuities is investigated. The conventional in-loop filters are modified to avoid improper filtering at the discontinuous edge. As compared to the HEC, the Test 4.1.b and Test 4.1.d can remarkably reduce the visible seam artifacts with slightly improved compression efficiency.

# References

- P. Hanhart, J.-L. Lin, and C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1033, July. 2018, Ljubljana, SI.

- S.-Y. Lin, L. Liu, C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang and C.-C. Ju, “AHG8: 360°-based in-loop filters for cubemap projection,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0142, July 2018, Ljubljana, SI.

- Y.-H. Lee, J.-L. Lin, S.-K. Chang and C.-C. Ju,”CE13: Modified Cubemap Projection in JVET-J0019 (Test 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0131, July 2018, Ljubljana, SI.

- S.-Y. Lin, J-L Lin, H-C Lin, S-K Chang, Y-C Chang, C-C Ju, “CE13: De-blocking filter disabled across face discontinuities (Test 4.1.a),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0347, Oct. 2018, Macao, CN.

- P. Hanhart, Y. He and Y. Ye , “CE13: HEC with deblocking and ALF disabled across face discontinuities (Test 4.1.c),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0232, Oct. 2018, Macao, CN.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VTM Benchmark Set, https://vcgit.hhi.fraunhofer.de/JVET-K-CE13/VVCSoftware_BMS

- P. Hanhart, J. Boyce, and K. Choi, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1012, July. 2018, Ljubljana, SI.

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).