| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0350-v1 |
| --- | --- |

| Title: | CE13: Padding and in-loop filters disabled across face discontinuities (Test 7.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng-Yen Lin, Lin Liu, Jian-Liang Lin, Shen-Kai Chang, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {Sheng-Yen.Lin, Lin.Liu, jl.lin, shenkai.chang, yc.zhang, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of Test 7.1 in CE13: Padded HEC (PHEC) with in-loop filters disabled across face discontinuities. The simulation results reportedly show that, in terms of end-to-end WS-PSNR-Y/U/V and codec PSNR-Y/U/V, Test 7.1 achieves an average -0.02%/-0.26%/0.01% and -0.31%/-1.50%/-1.16% BD-rate saving, respectively. In addition, the subjective quality of Test 7.1 is slightly enhanced by alleviating the seam artifacts compared to PHEC.

# Introduction

In the last meeting, Core Experiment 13 (CE13) [1] was established to study removing subjective artifacts at face boundaries for the 360° video category. This contribution reports the results of Test 7.1 in CE13: Padded HEC (PHEC) with in-loop filters disabled across face discontinuities.

# Experimental Results

The experiments are conducted on the 360Lib-7.0 [2] and the BMS-2.0.1 (with fix BMS ticket #78 and #79) [3] followed by the JVET common test conditions (CTC) and evaluation procedures [4] under the VTM-2.0 [5] configuration setting. Following the CTC, the frame size is set to 3840x2560 with the face size of 1280x1280. Compared to PHEC, Table I and Table II demonstrate that the average BD-rate reduction in terms of end-to-end WS-PSNR and codec PSNR for Test 7.1, respectively.

Table I BD-rate performance of Test 7.1 compared to PHEC in terms of end-to-end WS-PSNR

|  | Test 7.1 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| SkateboardInLot | 0.09% | -0.19% | 0.43% |
| ChairLift | -0.07% | -0.14% | -0.09% |
| KiteFlite | -0.02% | -0.01% | 0.00% |
| Harbor | -0.17% | -0.16% | 0.07% |
| Trolley | -0.03% | -0.01% | -0.09% |
| GasLamp | -0.05% | -0.15% | -0.05% |
| Balboa | 0.05% | -0.34% | 0.67% |
| Broadway | 0.04% | -0.11% | 0.21% |
| Landing2 | -0.05% | -1.54% | -1.04% |
| BranCastle2 | 0.02% | 0.11% | -0.03% |
| Class S1 | -0.04% | -0.11% | 0.05% |
| Class S2 | 0.01% | -0.47% | -0.05% |
| Overall | -0.02% | -0.26% | 0.01% |

Table II BD-rate performance of Test 7.1 compared to PHEC in terms of codec PSNR

|  | Test 7.1 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| SkateboardInLot | -0.18% | -1.92% | -1.57% |
| ChairLift | -0.36% | -0.92% | -0.49% |
| KiteFlite | -0.23% | -0.69% | -0.57% |
| Harbor | -0.78% | -1.99% | -2.32% |
| Trolley | -0.20% | -1.15% | -0.66% |
| GasLamp | -0.32% | -1.43% | -1.27% |
| Balboa | -0.34% | -1.20% | -0.08% |
| Broadway | -0.35% | -1.03% | -0.58% |
| Landing2 | -0.30% | -4.29% | -3.49% |
| BranCastle2 | -0.08% | -0.40% | -0.54% |
| Class S1 | -0.35% | -1.35% | -1.15% |
| Class S2 | -0.27% | -1.73% | -1.17% |
| Overall | -0.31% | -1.50% | -1.16% |

# Subjective Quality Testing

In Fig. 1 to Fig. 6, the cropped viewport images of PHEC and Test 7.1 are demonstrated (under VTM-2.0 with QP = 37) in order to evaluate the seam artifacts appeared at the discontinuous edge.

Fig. 1. The subjective quality of Landing2 frame 103 (PHEC)

Fig. 2. The subjective quality of Landing2 frame 103 (Test 7.1)

Fig. 3. The subjective quality of Balboa frame 86 (PHEC)

Fig. 4. The subjective quality of Balboa frame 86 (Test 7.1)

Fig. 5. The subjective quality of SkateboardInLot frame 71 (PHEC)

Fig. 6. The subjective quality of SkateboardInLot frame 71 (Test 7.1)

# Conclusion

In this contribution, the performance of Test 7.1 is investigated. The experimental data reports that Test 7.1 achieves an average -0.02%/-0.26%/0.01% and -0.31%/-1.50/-1.16% BD-rate saving in terms of end-to-end WS-PSNR-Y/U/V and codec PSNR-Y/U/V, respectively. Moreover, the subjective quality of Test 7.1 is slightly enhanced by alleviating the seam artifacts compared to PHEC.

# References

- Philippe Hanhart, Jian-Liang Lin, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-K1033, July 2018, Ljubljana, SI.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VVC software Benchmark Set, https://vcgit.hhi.fraunhofer.de/JVET-K-CE13/VVCSoftware_BMS/

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).