| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0353-v1 |
| --- | --- |

| Title: | CE6-related: MTS using DST-4 and transposed DCT-2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin Jianhua Zheng Quanhe Yu Na Zhang Ce Zhu | Email: | linyongbing@hisilicon.com zhengjianhua@hisilicon.com yuquanhe@hisilicon.com zhangna25@hisilicon.com eczhu@uestc.edu.cn |
| Source: | HiSilicon Technologies Univ. of Electronic Science and Technology of China |  |  |

_____________________________

- Abstract

This document presents a low-complexity MTS (multiple transform selection) approach for VVC. It is proposed to use DST-4 and transpose of DCT-2 as transform cores for the MTS, due to benefits of reusing circuit implementation of traditional DCT-2 transform and enabling partial butterfly implementation for the proposed transpose of DCT-2. Test results show coding performance of 0.06%/0.06%/-0.04%/0.03% for AI/RA/LDB/LDP configurations compared to VTM-2.0.1.

Introduction

In VTM 2, multiple transform cores as well as DCT-2 are exploited to improve coding efficiency. MTS uses DST-7 and DCT-8 as transform cores in both vertical and horizontal directions [1]. As shown in Table 1, there are 4 combinations between vertical and horizontal transforms. The index is signaled to tell decoder which combination is selected. Here A denotes DST-7 and B is DCT-8.

Table 1. MTS approach in VTM2

| Index | Transform |  |
| --- | --- | --- |
|  | Vertical | Horizontal |
| 0 | A | A |
| 1 | A | B |
| 2 | B | A |
| 3 | B | B |

In the last JVET meeting, JVET-K0130 proposed to use type 4 transform only for MTS, i.e., A=DST-4 and B=DCT-4 in Table 1. The claimed benefit is to reuse of traditional DCT-2 circuit implementation [2]. However it leads to coding performance loss. In addition, the proposed type 4 transforms has no fast algorithm implementation, e.g., partial butterfly algorithm.

Proposal

In order to solve above shortcomings while keeping the benefits, we propose to use DST-4 and transposed DCT-2 as transform cores for MTS, i.e., A=DST-4 and B is the transpose of DCT-2 in Table 1.

As shown in JVET-K0130, DST-4 can reuse the existing DCT-2 circuit implementation. It is also known that the transpose of DCT-2 is able to reuse the existing DCT-2 circuit implementation [3]. More important, the transpose of DCT-2 can exploit partial butterfly algorithm to reduce implementation complexity.

The following Figure demonstrates how to derive the proposed transforms from larger DCT-2 transform. As an example, 4-point DST-4 transform (i.e., A) and the transpose of DCT-2 transform (i.e., B) can be derived from 8-point DCT-2 transform. It also means no additional memory is required to store the proposed transform coefficients.

Figure 1. Derivation of the proposed 4-point transforms (i.e., A and B) from 8-point DCT-2 transform

Test results

Simulations based on BMS-2.0.1 have been conducted under common test condition [4]. The rest results show negligible performance change compared to BMS-2.0.1 with VTM configuration. Additional test results with both intra and inter MTS enabled (i.e., --MTS=3) is shown in Table 3.

Table 2. Performance of the proposal against BMS-2.0.1 with VTM configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.46% | 0.49% | 0.57% | 101% | 100% |
| Class A2 | 0.23% | 0.06% | 0.08% | 99% | 98% |
| Class B | 0.05% | 0.03% | 0.03% | 100% | 101% |
| Class C | -0.24% | -0.12% | 0.01% | 101% | 100% |
| Class E | -0.09% | -0.10% | -0.13% | 101% | 99% |
| Overall | 0.06% | 0.06% | 0.10% | 100% | 100% |
| Class D | -0.34% | -0.14% | -0.44% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | 0.42% | 0.37% | 100% | 101% |
| Class A2 | 0.13% | 0.12% | 0.04% | 100% | 99% |
| Class B | 0.07% | 0.16% | 0.00% | 100% | 100% |
| Class C | -0.09% | 0.22% | 0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.22% | 0.11% | 100% | 100% |
| Class D | -0.12% | -0.28% | -0.21% | 100% | 101% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-2.0.1 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.14% | -0.05% | 100% | 99% |
| Class C | -0.02% | 0.12% | -0.12% | 100% | 100% |
| Class E | -0.18% | 0.11% | -0.10% | 100% | 100% |
| Overall | -0.04% | 0.13% | -0.09% | 100% | 99% |
| Class D | -0.01% | 0.32% | 0.15% | 100% | 99% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |
|  | Over BMS-2.0.1 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.30% | 0.45% | 100% | 100% |
| Class C | 0.02% | 0.09% | -0.02% | 100% | 100% |
| Class E | 0.11% | 0.31% | 0.78% | 100% | 102% |
| Overall | 0.03% | 0.23% | 0.37% | 100% | 100% |
| Class D | -0.03% | 0.38% | 0.37% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 3 shows test result of proposal with both intra and inter MTS enabled (i.e., --MTS=3).

Table 3. Performance of proposal with both intra and inter MTS enabled

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM configuration + inter MTS enabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | 0.28% | 0.38% | 101% | 99% |
| Class A2 | 0.26% | 0.20% | 0.17% | 101% | 100% |
| Class B | 0.12% | 0.07% | 0.02% | 102% | 101% |
| Class C | -0.06% | 0.02% | -0.02% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.12% | 0.11% | 101% | 100% |
| Class D | -0.14% | -0.12% | -0.09% | 101% | 101% |
| Class F (optional) |  |  |  |  |  |

Recommendations

It is proposed to use DST-4 and the transposed DCT-2 as transform cores for MTS. The proposed approach has capability to reuse circuit implementation of traditional DCT-2 and enable partial butterfly implementation. The performance difference compared to BMS-2.0.1 is negligible. It is recommended to inclusion of the proposal into next version of VVC.

References

[1] J. Chen, Y. Ye, S. Kim, "Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2)", JVET-K1002, 11th Meeting: Ljubljana, SI, 10–20 Jul. 2018

[2] K. Abe, T. Toma, "CE6-related: Type4 only AMT", JVET-K0130, 11th Meeting: Ljubljana, SI, 10–20 Jul. 2018

[3] Madhukar Budagavi, Arild Fuldseth, Gisle Bjøntegaard, Vivienne Sze, Mangesh Sadafale, "Core Transform Design in the High Efficiency Video Coding (HEVC) Standard", IEEE Journal of Selected Topics in Signal Processing, Volume: 7, Issue: 6, Dec. 2013

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, 11th Meeting: Ljubljana, SI, 10–20 Jul. 2018

Patent rights declaration(s)

HiSilicon Technologies，Huawei Technologies, Univ. of Electronic Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).