| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0366-v1 |
| --- | --- |

| Title: | CE4: Common base for affine merge mode (Test 4.2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of common base for affine merge mode over VTM-2.0.1. The original affine merge mode in VTM-2.0.1 is enhanced by constructing an affine merge candidate list with 5 candidates. The affine merge candidate list is constructed by inserting 2 kinds of candidates: inherited affine candidates and constructed affine candidates.

Simulation results reportedly show on average 0.75%/0.44% luma BD-rate gain with 7%/8% encoding time and 7%/5% decoding time increase in RA/LB configurations, compared to VTM-2.0.1.

Introduction

In the affine merge mode of VTM-2.0.1, only the first available affine neighbour can be used to derive motion information of affine merge mode. In this contribution, a candidate list for affine merge mode is constructed by searching valid affine neighbours and combining the neighbor motion information of each control point.

The affine merge candidate list is constructed as following steps:

- Insert inherited affine candidates

Inherited affine candidate means that the candidate is derived from the affine motion model of its valid neighbor affine coded block. In the common base, as shown in Figure 1, the scan order for the candidate positions is: A1, B1, B0, A0 and B2.

After a candidate is derived, full pruning process is performed to check whether same candidate has been inserted into the list. If a same candidate exists, the derived candidate is discarded.

- Insert constructed affine candidates

If the number of candidates in affine merge candidate list is less than MaxNumAffineCand (set to 5 in this contribution), constructed affine candidates are inserted into the candidate list. Constructed affine candidate means the candidate is constructed by combining the neighbor motion information of each control point.

The motion information for the control points is derived firstly from the specified spatial neighbors and temporal neighbor shown in Figure 1. CPk (k=1, 2, 3, 4) represents the k-th control point. A0, A1, A2, B0, B1, B2 and B3 are spatial positions for predicting CPk (k=1, 2, 3); T is temporal position for predicting CP4.

The coordinates of CP1, CP2, CP3 and CP4 is (0, 0), (W, 0), (H, 0) and (W, H), respectively, where W and H are the width and height of current block.

Figure 1 Candidates position for affine merge mode

The motion information of each control point is obtained according to the following priority order:

For CP1, the checking priority is B2->B3->A2. B2 is used if it is available. Otherwise, if B2 is available, B3 is used. If both B2 and B3 are unavailable, A2 is used. If all the three candidates are unavailable, the motion information of CP1 cannot be obtained.

For CP2, the checking priority is B1->B0.

For CP3, the checking priority is A1->A0.

For CP4, T is used.

Secondly, the combinations of controls points are used to construct an affine merge candidate.

Motion information of three control points are needed to construct a 6-parameter affine candidate. The three control points can be selected from one of the following four combinations ({CP1, CP2, CP4}, {CP1, CP2, CP3}, {CP2, CP3, CP4}, {CP1, CP3, CP4}). Combinations {CP1, CP2, CP3}, {CP2, CP3, CP4}, {CP1, CP3, CP4} will be converted to a 6-parameter motion model represented by top-left, top-right and bottom-left control points.

Motion information of two control points are needed to construct a 4-parameter affine candidate. The two control points can be selected from one of the following six combinations ({CP1, CP4}, {CP2, CP3}, {CP1, CP2}, {CP2, CP4}, {CP1, CP3}, {CP3, CP4}). Combinations {CP1, CP4}, {CP2, CP3}, {CP2, CP4}, {CP1, CP3}, {CP3, CP4} will be converted to a 4-parameter motion model represented by top-left and top-right control points.

The combinations of constructed affine candidates are inserted into to candidate list as following order:

{CP1, CP2, CP3}, {CP1, CP2, CP4}, {CP1, CP3, CP4}, {CP2, CP3, CP4}, {CP1, CP2}, {CP1, CP3}, {CP2, CP3}, {CP1, CP4}, {CP2, CP4}, {CP3, CP4}

For reference list X (X being 0 or 1) of a combination, the reference index with highest usage ratio in the control points is selected as the reference index of list X, and motion vectors point to difference reference picture will be scaled.

After a candidate is derived, full pruning process is performed to check whether same candidate has been inserted into the list. If a same candidate exists, the derived candidate is discarded.

- Padding with zero motion vectors

If the number of candidates in affine merge candidate list is less than 5, zero motion vectors with zero reference indices are insert into the candidate list, until the list is full.

Simulation results

The contribution has been implemented on top of VTM-2.0.1. The test condition descripted in JVET-K1024 [1] is used for verifying the contribution. The summary results are provided in Table 1. Simulation results reportedly show on average 0.75%/0.44% luma BD-rate gain with 7%/8% encoding time and 7%/5% decoding time increase in RA/LB configurations, compared to VTM-2.0.1.

Table 1 Coding performance of test 4.2.1 over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -0.59% | -0.89% | 106% | 108% |
| Class A2 | -1.00% | -0.98% | -1.14% | 107% | 108% |
| Class B | -0.67% | -0.57% | -0.70% | 107% | 106% |
| Class C | -0.48% | -0.46% | -0.54% | 107% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.75% | -0.62% | -0.78% | 107% | 107% |
| Class D | -0.68% | -0.67% | -0.78% | 108% | 112% |
| Class F (optional) | -0.14% | -0.18% | -0.14% | 105% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | -0.28% | -0.64% | 108% | 105% |
| Class C | -0.35% | -0.39% | -0.26% | 107% | 104% |
| Class E | -0.64% | -0.63% | -0.18% | 109% | 106% |
| Overall | -0.44% | -0.40% | -0.40% | 108% | 105% |
| Class D | -0.60% | -0.44% | -0.60% | 107% | 110% |
| Class F (optional) | -0.10% | 0.20% | -0.02% | 106% | 102% |

Conclusions

The original affine merge mode in VTM-2.0.1 is enhanced by constructing an affine merge candidate list with 5 candidates. The affine merge candidate list is constructed by inserting 2 kinds of candidates: inherited affine candidates and constructed affine candidates.

Simulation results reportedly show on average 0.75%/0.44% luma BD-rate gain with 7%/8% encoding time and 7%/5% decoding time increase in RA/LB configurations, compared to VTM-2.0.1.

References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).