| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0367-v1 |
| --- | --- |

| Title: | CE9-related: An early termination of DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The DMVR (Decoder Motion Vector Refinement) is a method of improving coding efficiency without explicitly sending overheads by motion vectors refinement of the merge mode on the both encoder and decoder side. One of the tasks is to reduce the complexity of the decoder side. In this contribution, an early termination of DMVR is improved. As experimental results, the decoding time was reduced by about 5% without additional SIMD, and the coding efficiency is almost unchanged. Those results should be considered together with related CE9.2.7.

Introduction

As previous CE9 [1], several problems of the DMVR have been reported. One of the problem is the complexity of decoder side. In the case of software implementation, the method of early termination is a technique often used for speeding up motion vector search and mode selection. In this contribution, we would like to propose a simple early termination for the DMVR. In the following section, we report our proposed method and experimental results.

Early termination

Current cost determination method of DMVR on BMS-2.1 is as follows:

Step 1: Cost determination of initial MV

If initial matching cost is equal to zero, then the DMVR processing is terminated, else go to step 2.

Step 2: Cost determination of refinement MV

A motion vector with the minimum cost of the search range is searched.

As the matching cost of DMVR, MRSAD (Mean Removed Sum of Absolute Differences) is used. The bit precision of cost is equal to Max(2, 14 – BitDepth) + BitDepth, because the interpolated prediction samples are designed with computational precision of more than two decimal places. Therefore, since the probability of cost equal to zero of step 1 is very small actually, another threshold should be designed.

In this contribution, the cost determination of initial MV is improved as follow.

- If the initial matching cost of step 1 is less than (w*h*(1<<(Max(2, 14 – BitDepth) + (BitDepth-8)+TH)), then the cost is regarded as zero and the DMVR processing is terminated, else go to step 2.

Here, w is the width of block, h is the height of block, BitDepth is the bit depth of samples and TH is the parameter of early termination. As a preliminary experimental result, 0 was selected for the value of TH.

In our proposal, only one line is replaced on BMS-2.1 software and any additional SIMD code is not used.

Experimental results

The simulation based on JVET-K1010 [2] for SDR sequences has been conducted. Additionally, according to the suggestion of CE9, the following test conditions have been used.

- The software used in CE9 is: BMS2.1rc1 (5b93362a62b8e8a54f0a12069cd729c94defe7e8).

- Anchor for Mandatory simulations: BMS2.1rc1 with VTM encoder configuration.

- Test for mandatory simulations: Anchor + DMVR on.

The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

Table 1 Summary of experimental results of SDR sequences on VTM configuration plus to DMVR on

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 + DMVR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.05% | 100% | 97% |
| Class A2 | 0.02% | -0.02% | -0.01% | 100% | 97% |
| Class B | 0.02% | 0.04% | 0.01% | 100% | 93% |
| Class C | 0.01% | 0.04% | 0.06% | 100% | 92% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | 0.03% | 100% | 95% |
| Class D | -0.01% | -0.14% | -0.07% | 100% | 93% |
| Class F (optional) | 0.00% | -0.01% | -0.05% | 99% | 88% |

Table 2 Summary of experimental results of SDR sequences on VTM configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.76% | -1.56% | -1.87% | 103% | 109% |
| Class A2 | -2.31% | -2.47% | -2.54% | 102% | 109% |
| Class B | -1.31% | -1.44% | -1.52% | 104% | 115% |
| Class C | -1.38% | -1.58% | -1.63% | 104% | 118% |
| Class E |  |  |  |  |  |
| Overall | -1.62% | -1.71% | -1.82% | 103% | 113% |
| Class D | -1.75% | -1.88% | -1.91% | 105% | 119% |
| Class F (optional) | -0.53% | -0.70% | -0.71% | 103% | 109% |

Conclusion

In this contribution, the early termination of the DMVR has been proposed. As experimental results, the decoding time was reduced about 5% without additional SIMD, and the coding efficiency was almost unchanged.

We would like to suggest that those results should be considered together with CE9.2.7.

Reference

[1]	S. Esenlik,Y.-W. Chen and F.Chen, Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation, Joint Video Experts Team Document, JVET-K1029, Ljubljana, July 2018.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-K1010, Ljubljana, July 2018.

[3]	X. Xiu, Y. He, Y. Ye, CE9.1.3: Complexity reduction on decoder-side motion vector refinement (DMVR), Joint Video Experts Team Document, JVET-K0348, Ljubljana, July 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).