| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0371-v1 |
| --- | --- |

| Title: | CE4-related: Reducing worst case memory bandwidth in inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes the following changes to reducing the worst case memory bandwidth in motion compensation of VTM-2.0.1:

- Bi-prediction is disabled for 4x4 inter CUs.

- For ATMVP, the sub-block size is fixed to 8x8.

- In affine mode, for bi-prediction of affine block, the sub-block size is set equal to 8x4/4x8; for uni-prediction of affine block, the sub-block size is set equal to 4x4.

The proposed changes reduce the worst case memory bandwidth by disabling 4x4 inter bi-prediction. Simulation results reportedly show on average 0.xx%/0.xx% luma BD-rate changes in RA/LB configurations, compared to VTM-2.0.1.

Introduction

In VTM, the worst case of memory bandwidth of motion compensation is for 4x4 bi-prediction, which is used by following modes: 1) Bi-prediction 4x4 inter CUs; 2) ATMVP with bi-prediction 4x4 size of sub-blocks; 3) Bi-prediction affine CUs.

For the purpose of reducing the worst case memory access, in this contribution, 4x4 bi-prediction is disabled by following aspects:

- Bi-prediction is disabled for 4x4 inter CUs. In merge mode, if the block size is 4x4, the motion vector in L1 direction is discarded for bi-prediction candidates. In inter mode, 4x4 bi-prediction is prohibited and the bin which indicates uni-prediction and bi-prediction is not signaled.

- For ATMVP, the sub-block size is fixed to 8x8. If the width or height of CU is smaller than 8, ATMVP is disabled.

- In affine mode, different sub-block sharps are used, which depend on the prediction direction and the CU size. For bi-prediction of affine block, the sub-block size is set equal to 8x4/4x8; for uni-prediction of affine block, the sub-block size is set equal to 4x4. 8x4 and 4x8 sub-block size for bi-prediction of affine block are selected by the CU size. If the width of CU is larger than or equal to its height, the sub-block size is set equal to 8x4; otherwise, the sub-block size is set equal to 4x8. In affine merge mode, if the CU size is equal to 8x8, in order to make the CU can be both horizontal and vertical spilt, the motion vector in L1 direction is discarded for a bi-prediction candidate, which is similar to the situation for 4x4 CU coded as merge mode.

The average number of reference pixels for motion compensation of a block MxN can be calculated as formula (1), where T means the number of interpolation filter taps, D means the number of directions.

BW=M+T-1*N+T-1*DM*N                              (1)

According to formula (1), average numbers of reference pixels of different block sizes are calculated and shown in Table 1. Therefore, the worst case of memory bandwidth is reduced from 15.1250 (4x4 BI) to 10.3125 (8x4 BI), when compared with VTM-2.0.1. When compared to the worst case of HEVC (7.0313, 8x8 BI), the proposed method increases about 47% memory access.

Table 1 Memory bandwidth of different block sizes

| Block size | Average number of reference pixels |
| --- | --- |
| 4x4 UNI | 7.5625 |
| 4x4 BI | 15.1250 |
| 8x4 UNI | 5.1563 |
| 8x4 BI | 10.3125 |
| 8x8 UNI | 3.5156 |
| 8x8 BI | 7.0313 |

Test specification

The tests for this contribution are descripted in Table 2.

Table 2 test specification for this contribution

| Test # | Test description |
| --- | --- |
| a | Restrictions in 4x4 CU |
| b | Restrictions in ATMVP |
| c | Restrictions in affine |
| d | a + b + c |

Simulation results

The contribution has been implemented on top of VTM-2.0.1. The test condition descripted in JVET-K1024 [1] is used for verifying the contribution. The results are summarized in Table 3. More detailed results are provided from Table 4 to Table 7.

Table 3 Summary of overall performance in RA/LB configuration (over VTM-2.0.1)

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| a | 0.03% | 0.04% | 0.02% |  |  |  |  |  |  |  |
| b |  |  |  |  |  |  |  |  |  |  |
| c | 0.10% | 0.10% | 0.05% |  |  |  |  |  |  |  |
| d |  |  |  |  |  |  |  |  |  |  |

Test a

Table 4 Coding performance of test a over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.09% | 0.04% |  |  |
| Class A2 | 0.02% | -0.03% | -0.02% |  |  |
| Class B | 0.03% | 0.04% | -0.02% |  |  |
| Class C | 0.04% | 0.07% | 0.07% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.04% | 0.02% |  |  |
| Class D | 0.08% | 0.09% | 0.17% |  |  |
| Class F (optional) | -0.02% | 0.03% | 0.06% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Test b

Table 5 Coding performance of test b over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Test c

Table 6 Coding performance of test c over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.04% |  |  |
| Class A2 | 0.18% | 0.17% | 0.10% |  |  |
| Class B | 0.11% | 0.11% | -0.01% |  |  |
| Class C | 0.11% | 0.13% | 0.14% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.10% | 0.05% |  |  |
| Class D | 0.24% | 0.20% | 0.18% |  |  |
| Class F (optional) | 0.17% | 0.14% | 0.12% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Test d

Table 7 Coding performance of test d over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Conclusions

This contribution proposes the following changes to reducing the worst case memory bandwidth in motion compensation of VTM:

- Bi-prediction is disabled for 4x4 inter CUs.

- For ATMVP, the sub-block size is fixed to 8x8.

- In affine mode, for bi-prediction of affine block, the sub-block size is set equal to 8x4/4x8; for uni-prediction of affine block, the sub-block size is set equal to 4x4.

The proposed changes reduce the worst case memory bandwidth by disabling 4x4 bi-prediction. Simulation results reportedly show on average 0.xx%/0.xx% luma BD-rate changes in RA/LB configurations, compared to VTM-2.0.1.

References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).