| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0375-v1 |
| --- | --- |

| Title: | CE4-related: inter prediction sample filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiwei Xu Haitao Yang Yin Zhao Jianle Chen | Tel: Email: | +8613758193753 xuweiwei3@huawei.com haitao.yang@huawei.com yin.zhao@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of inter prediction samples filtering. A spatial filter is applied on the prediction samples from merge/skip mode. A CU-level flag is added to indicate whether using inter prediction samples filtering. Compared to VTM2.0.1 anchor, the average BD-rate is -0.29% in RA.

Introduction

After the inter prediction samples is obtained in MC, the prediction samples is filtered using the spatial neighboring samples in the left and top neighbor as follows.

P(x,y) = (W1*L(-1,y) + W2*A(x,-1) + W3*pred(x,y) + 4) >> 3

W1, W2 and W3 denote the weighting factors, and W1, W2 and W3 are set as 1, 1, and 6, respectively. (x,y) is the coordinates of current prediction samples relative to the above left corner sample.is the filtered prediction sample and Pred(x,y) is the original prediction samples. A CU-level flag is added to indicate whether using inter prediction samples filtering.

Figure 1. Inter prediction sample filtering

Simulation results

The contribution has been implemented on top of VTM2.0.1. The test condition descripted in JVET-K1024 [1] is used for verifying the contribution. The summary results are provided in Table 1. On average, the BD-rate differences are -0.29% in RA when W1, W2 and W3 are set as 2, 2, and 12, respectively.

Table 1 Coding performance over VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.10% | -0.17% |  |  |
| Class A2 | -0.25% | -0.19% | -0.38% |  |  |
| Class B | -0.35% | -0.23% | -0.17% |  |  |
| Class C | -0.27% | -0.09% | -0.32% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.16% | -0.25% |  |  |
| Class D | -0.17% | -0.19% | -0.29% |  |  |
| Class F (optional) | -0.33% | -0.23% | -0.28% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Conclusions

This contribution reports the results of inter prediction sample filtering. Compared to VTM2.0.1 anchor, the average BD-rate differences are-0.29% in RA when W1, W2 and W3 are set as 2, 2, and 12, respectively.

We recommend to study the proposed method in CE.

References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).