| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L378 |
| --- | --- |

| Title: | CE3: Adaptive inter-residual prediction (CE3-5.7.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents results of a core experiment CE3-5.7.2 included CE3 (JVET-K1023). BD-rates for luma and chroma (Y, U, V) are (-0.14, -2.64%, -2.96%), (-0.02, -2.36%, -2.25%), and (-0.01, -0.88%, -1.74%) under the CTC AI, RA, and LDB conditions respectively compared to VTM. Anchor. In a similar way, BD-rates for luma and chroma (Y, U, V) are (-0.12, -2.83, -2.82), (-0.01, -2.57%, -2.28%), and (0.03, -1.11%, -1.48%) under the CTC AI, RA, and LDB conditions respectively comparing to the BMS anchor.

Introduction

Core Experiment 3 (CE3) related to intra prediction and mode coding is established in 11th meeting at Ljubljana on July, 2018 [1]. This contribution is reporting results of CE3-5.7.2 proposed by KDDI in the call for response. The remainder of this contribution is divided into two parts;

- Proposed method in CE3-5.7.2 (Adaptive inter-residual prediction),

- Experimental results under the CTC.

Adaptive inter-residual prediction [2]

CCLM Cb-to-Cr prediction exists in benchmark set. In this contribution, an adaptive inter-residual prediction is introduced as CCLM Cr-to-Cb prediction. Reference component is signaled by an additional flag. The remaining parts like prediction and scaling factor derivation are the same as in benchmark set. One flag for each chroma CU is added to identify the reference chroma component. This is implemented by adding a weighted reconstructed Cb residual to the original Cr intra prediction to form the final Cr prediction:

The scaling factor  is derived in the same way of CCLM Cb-to-Cr prediction.

When the intra prediction modes other than CCLM luma-to-chroma prediction mode is used for chroma component of a CU, CCLM Cb-to-Cr or Cr-to-Cb prediction is used to enhance the Cr or Cb component prediction.

Two tests are proposed. One is test with full RDO and another is test with fast RDO. The details are shown in the following section.

For a fast encoding, a reduction of the number of RD cost check is proposed. The key idea is to decide the reference chroma component by only one-time RD cost check. To realize the idea, the decision is based on the RD cost of DM_Chroma since a probability of DM_Chroma is highest. At the same time, DM_Chroma is moved to the first position in chroma RD-cost-check order. It is noted that a signal processing based decision without RD cost check does not work well in our experiments.

Experimental Results

Coding conditions

The proposed tool was implemented on top of the BMS-2.0.1 software, which can be found athttps://vcgit.hhi.fraunhofer.de/JVET-K-CE3/VVCSoftware_BMS/tree/CE3-5.7. We run the software with following parameter settings in TypeDef.h for CE3-5.7.2.

# define J0016_CE3_CROSSCOLOR_ORDER 1

In order to verify the performance of our proposed method, we have run simulations according to the test conditions of JVET-K1010 [3]. Each of the results are presented in bellow. In the results, “VTM-2.0.1” and “BMS-2.0.1” mean simulation anchor results by using BMS-2.0.1 software with VTM and BMS configuration files respectively.

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective results

VTM aspects

From Table 1 to Table 3 show the experimental results of proposed method with fastRDO OFF compared to VTM-2.0.1 anchor. It can be seen that the BD-rates for AI/RA/LDB chroma (U, V) are (-2.64%, -2.96%)/(-2.36%, -2.25%)/(-0.88%, -1.74%) and the simulation runtime for AI/RA/LDB (EncT, DecT) are (106%,102%)/(103%, 101%)/(102%, 100%) respectively.

On the other hand, from Table 4 to Table 6 show the experimental results of proposed method with fastRDO ON compared to VTM-2.0.1 anchor. It can be seen that the BD-rates for AI/RA/LDB chroma (U, V) are (-2.41%, -2.67%), (-2.25%, -2.11%), and (-0.72%, -1.46%) and simulation runtime for AI/RA/LDB (EncT, DecT) are (101%,100%)/(101%, 100%)/(100%, 100%) respectively.

Table 1 Experimental results for AI condition compared with VTM-2.0.1 fastRDO OFF

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -1.53% | -1.16% | 106% | 100% |
| Class A2 | -0.32% | -3.44% | -2.23% | 110% | 100% |
| Class B | -0.06% | -2.13% | -3.25% | 105% | 101% |
| Class C | -0.12% | -2.76% | -3.04% | 107% | 101% |
| Class E | -0.21% | -3.65% | -4.88% | 103% | 100% |
| Overall | -0.14% | -2.64% | -2.96% | 106% | 100% |
| Class D | -0.13% | -3.21% | -3.36% | 107% | 100% |

Table 2 Experimental results for RA condition compared with VTM-2.0.1 fastRDO OFF

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -1.46% | -0.86% | 104% | 100% |
| Class A2 | -0.09% | -3.42% | -1.18% | 103% | 100% |
| Class B | 0.00% | -2.37% | -3.64% | 103% | 101% |
| Class C | -0.05% | -2.21% | -2.34% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -2.36% | -2.25% | 103% | 101% |
| Class D | -0.02% | -3.16% | -2.85% | 103% | 101% |

Table 3 Experimental results for LDB condition compared with VTM-2.0.1 fastRDO OFF

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.52% | -1.15% | 102% | 100% |
| Class C | 0.02% | -0.60% | -0.96% | 102% | 101% |
| Class E | -0.11% | -1.87% | -3.75% | 100% | 100% |
| Overall | -0.01% | -0.88% | -1.74% | 102% | 100% |
| Class D | 0.04% | -0.09% | -0.12% | 102% | 101% |

Table 4 Experimental results for AI condition compared with VTM-2.0.1 fastRDO ON

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -1.41% | -1.01% | 101% | 100% |
| Class A2 | -0.33% | -3.06% | -2.00% | 102% | 100% |
| Class B | -0.07% | -1.93% | -3.03% | 101% | 100% |
| Class C | -0.12% | -2.53% | -2.69% | 101% | 100% |
| Class E | -0.20% | -3.41% | -4.36% | 100% | 100% |
| Overall | -0.14% | -2.41% | -2.67% | 101% | 100% |
| Class D | -0.15% | -2.77% | -2.92% | 101% | 99% |

Table 5 Experimental results for RA condition compared with VTM-2.0.1 fastRDO ON

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -1.47% | -0.89% | 101% | 100% |
| Class A2 | -0.10% | -3.22% | -1.13% | 101% | 100% |
| Class B | -0.02% | -2.26% | -3.34% | 101% | 100% |
| Class C | -0.04% | -2.08% | -2.22% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -2.25% | -2.11% | 101% | 100% |
| Class D | -0.07% | -3.12% | -2.39% | 101% | 101% |

Table 6 Experimental results for LDB condition compared with VTM-2.0.1 fastRDO ON

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.54% | -1.13% | 100% | 100% |
| Class C | 0.04% | -0.48% | -0.66% | 100% | 100% |
| Class E | -0.11% | -1.36% | -3.08% | 100% | 100% |
| Overall | -0.01% | -0.72% | -1.46% | 100% | 100% |
| Class D | 0.10% | -0.03% | -0.65% | 100% | 101% |

BMS aspects

From Table 1 to Table 3 show the experimental results of proposed method with fastRDO OFF compared to BMS-2.0.1 anchor. It can be seen that the BD-rates for AI/RA/LDB chroma (U, V) are (-2.83%, -2.82%)/(-2.57%, -2.28%)/(-1.11%, -1.48%) and the simulation runtime for AI/RA/LDB (EncT, DecT) are (106%,102%)/(107%, 101%)/(105%, 101%) respectively.

On the other hand, from Table 4 to Table 6 show the experimental results of proposed method with fastRDO ON compared to BMS-2.0.1 anchor. It can be seen that the BD-rates for AI/RA/LDB chroma (U, V) are (-2.47%, -2.46%), (-2.36%, -2.03%), and (-1.11%, -1.31%) and simulation runtime for AI/RA/LDB (EncT, DecT) are (101%,98%)/(101%, 100%)/(101%, 100%) respectively.

Table 7 Experimental results for AI condition compared with BMS-2.0.1 fastRDO OFF

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -1.59% | -1.33% | 107% | 103% |
| Class A2 | -0.29% | -3.39% | -2.39% | 110% | 102% |
| Class B | -0.06% | -2.30% | -2.98% | 105% | 102% |
| Class C | -0.09% | -3.00% | -3.03% | 106% | 101% |
| Class E | -0.16% | -4.17% | -4.17% | 104% | 102% |
| Overall | -0.12% | -2.83% | -2.82% | 106% | 102% |
| Class D | -0.09% | -3.54% | -3.25% | 107% | 101% |

Table 8 Experimental results for RA condition compared with BMS-2.0.1 fastRDO OFF

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -1.80% | -1.13% | 109% | 101% |
| Class A2 | -0.10% | -3.41% | -1.37% | 107% | 100% |
| Class B | -0.01% | -2.43% | -3.41% | 106% | 101% |
| Class C | -0.04% | -2.70% | -2.39% | 107% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -2.57% | -2.28% | 107% | 101% |
| Class D | 0.01% | -3.17% | -2.67% | 106% | 101% |

Table 9 Experimental results for LDB condition compared with BMS-2.0.1 fastRDO OFF

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.71% | -1.10% | 105% | 101% |
| Class C | -0.03% | -0.68% | -0.76% | 106% | 101% |
| Class E | 0.10% | -2.35% | -3.07% | 102% | 101% |
| Overall | 0.03% | -1.11% | -1.48% | 105% | 101% |
| Class D | 0.02% | -0.59% | 0.02% | 104% | 101% |

Table 10 Experimental results for AI condition compared with BMS-2.0.1 fastRDO ON

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -1.29% | -1.03% | 101% | 97% |
| Class A2 | -0.28% | -2.87% | -2.06% | 102% | 99% |
| Class B | -0.06% | -1.97% | -2.61% | 101% | 98% |
| Class C | -0.10% | -2.82% | -2.71% | 101% | 99% |
| Class E | -0.14% | -3.59% | -3.69% | 101% | 98% |
| Overall | -0.12% | -2.47% | -2.46% | 101% | 98% |
| Class D | -0.11% | -3.10% | -2.82% | 101% | 100% |

Table 11 Experimental results for RA condition compared with BMS-2.0.1 fastRDO ON

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -1.63% | -1.05% | 102% | 100% |
| Class A2 | -0.07% | -3.29% | -1.23% | 101% | 100% |
| Class B | -0.02% | -2.19% | -3.07% | 101% | 100% |
| Class C | -0.06% | -2.43% | -2.08% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -2.36% | -2.03% | 101% | 100% |
| Class D | 0.00% | -2.79% | -2.33% | 101% | 101% |

Table 12 Experimental results for LDB condition compared with BMS-2.0.1 fastRDO ON

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.76% | -0.76% | 101% | 99% |
| Class C | -0.04% | -0.70% | -0.42% | 101% | 100% |
| Class E | -0.02% | -2.25% | -3.43% | 100% | 100% |
| Overall | -0.03% | -1.11% | -1.31% | 101% | 100% |
| Class D | 0.04% | -0.44% | 0.20% | 101% | 101% |

Conclusion

This contribution presents results of a core experiment CE3-5.7.2 included CE3 (JVET-K1023 [1]). In this contribution, it was presented that the adaptive inter-residual prediction introduces CCLM Cr-to-Cb prediction. In proposed method, reference component was signaled by an additional flag. BD-rates for luma and chroma (Y, U, V) are (-0.14, -2.64%, -2.96%), (-0.02, -2.36%, -2.25%), and (-0.01, -0.88%, -1.74%) under the CTC AI, RA, and LDB conditions respectively compared to VTM. Anchor. In a similar way, BD-rates for luma and chroma (Y, U, V) are (-0.12, -2.83, -2.82), (-0.01, -2.57%, -2.28%), and (0.03, -1.11%, -1.48%) under the CTC AI, RA, and LDB conditions respectively comparing to the BMS anchor.

- Acknowledgment

We would like to thank Huawei for doing crosschecking of our proposal.

References

- G. Auwera, J. Heo and A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-K1023, Ljubljana, SI, July 2018.

- K. Kawamura, Y. Kidani and S. Naito, “Description of SDR video coding technology proposal by KDDI”, JVET-J0016, San-Diego, US, April 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, , “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, SI, July 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).