| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L382 |
| --- | --- |

| Title: | CE9-related : DMVR with Coarse-to-Fine Search and Block Size Limit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents a proposal of search method for bilateral DMVR. In the proposed method, 10 points including half pixels are searched by 3 stages when search range is 2. Furthermore, disallowing DMVR at large CUs is proposed in this contribution. BD-rate gains for luma is -1.07% under the RA conditions in case of comparing with BMS2.1 rc1 with VTM configuration.

Introduction

Core Experiment 9 (CE9) related to decoder side motion vector refinement (DMVR) is established in 11th meeting at Ljubljana on July, 2018 [1]. This contribution presents a proposal of search method. The proposed method is related to CE9.2. The remainder of this contribution is divided into two parts;

- Proposed method, and

- Experimental results under the CTC.

Proposed Method

Coarse-to-Fine Search method

In the proposed method, searching process is done by bilateral matching manner like [2]. Then “MRSAD” in following explanation means MRSAD (Mean Reduced SAD) between L0 and L1 reference block. In the process, 10 points including half pixels are searched by 3 stages when search range is 2 as follows.

- Stage 1

Calculate MRSADs for 5 points indicated in Figure 1.

Figure 1 Search points of stage 1.

- Stage 2

Calculate MRSADs for more 2 points.

The 2 points are decided by following procedure.

(1) Find minimum MRSAD point from P1, P2, P3, and P4 (corner points of search range).

(2) Change x-component and y-component of minimum MRSAD point to zero respectively.

For example, when P2 is the minimum MRSAD point, additional 2 points are P5 and P6 as shown in Figure 2.

Figure 2 An example of search points of stage 2.

- Stage 3

Calculate MRSADs for more 3 points.

The 3 points are decided by following procedure.

(1) Find minimum and second minimum MRSAD point from P0, P5, P6, and the minimum MRSAD point from P1, P2, P3, and P4.

(2) A integer pixel and 2 half pixels on a straight line connecting 2 points found in (1) are decided to search points of this stage.

For example, when P5 and P6 are the minimum and second minimum point in (1), additional 3 points are P7, P8, and P9 as shown in Figure 3.

Figure 3 An example of search point of stage 3.

Finally, the minimum MRSAD point from P0 to P9 is decided to final MV.

Block Size Limit

In the proposed method, DMVR is disallowed at large CU (number of pixels is 4096 or less than 4096 in a CU). If DMVR is allowed at large CU (e.g. 128x128), large circuit for MRSAD calculation is needed. Therefore, limits of block size can help to save a number of circuit for DMVR. To disallow DMVR at large CU, 2 methods can be considered as follows.

- Simply disallow DMVR at large CU

- In large CU, firstly divide CU into sub-CU, then process DMVR for each sub-CU

In this contribution, the test results under condition of above 1. are presented.

Experimental Results

The extended deblocking filter was implemented on top of the BMS-2.1 rc1 software. In order to verify the performance of the proposed search method, we have run simulations according to the common test conditions of JVET-K1010 [3].

Results under the common test conditions

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective evaluation

Table 1 shows the experimental results of proposed method (both coarse-to-fine search and block size limit). It can be seen that BD-rates for luma is -1.07% for RA can be observed in case of comparing with BMS-2.1 rc1 with VTM config. Table 2 shows the experimental results of proposed method (only coarse-to-fine search). BD-rates for luma is -1.18% can be observed in case of comparing with BMS-2.1 rc1 with VTM config.

Table 1 Experimental results for RA condition compared with BMS2.1 rc1 + VTM configuration (Coarse-to-fine search + Block size limit)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1 rc1 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.33% | -1.19% | -1.51% | 104% | 108% |
| Class A2 | -1.56% | -1.73% | -1.76% | 104% | 110% |
| Class B | -0.82% | -0.96% | -1.04% | 105% | 113% |
| Class C | -0.82% | -1.06% | -1.05% | 105% | 120% |
| Class E |  |  |  |  |  |
| Overall | -1.07% | -1.19% | -1.28% | 105% | 113% |
| Class D | -0.83% | -1.00% | -1.08% | 106% | 122% |

Table 2 Experimental results for RA condition compared with BMS2.1 rc1 + VTM configuration  (Coarse-to-fine search)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1 rc1 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.61% | -1.49% | -1.83% | 105% | 116% |
| Class A2 | -1.71% | -1.89% | -1.93% | 105% | 116% |
| Class B | -0.88% | -1.05% | -1.14% | 106% | 121% |
| Class C | -0.82% | -1.05% | -1.05% | 106% | 123% |
| Class E |  |  |  |  |  |
| Overall | -1.18% | -1.31% | -1.41% | 105% | 120% |
| Class D | -0.83% | -1.00% | -1.09% | 107% | 123% |

Conclusion

This contribution presented results of a proposed method related to core experiment 9.2. It was presented that test results under CTCs. For objective test results, BD-rates was -1.07% in case of comparing with BMS2.1 rc1 with VTM configuration.

- Acknowledgment

We would like to thank Huawei for doing crosschecking of our proposal.

References

- S. Esenlik, Y.-W. Chen, and F. Chen, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-K1029, Ljubljana, SI, July 2018.

- S. Esenlik, I. Krasnov, Z. Zhao, M. Xiang, H. Yang, J. Chen and Y. Li, “CE9: DMVR with Bilateral Matching (Test2.9)”, JVET-K0217, Ljubljana, SI, July 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, SI, July 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).