| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0385-v1 |
| --- | --- |

| Title: | CE10.1.3: Multi-hypothesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu Yu-Wen Huang, Shaw-Min Lei MediaTek Inc. No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
|  | Martin Winken, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-570 {martin.winken, heiko.schwarz, detlev.marpe, thomas.wiegand} @hhi.fraunhofer.de |
| Source: | MediaTek Inc. and Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this document, results for test CE10.1.3 are reported.  Test CE10.1.3 is a combination of the individual tests CE10.1.1.c (MediaTek) and CE10.1.2.b (HHI).  A detailed description of the individual tests can be found in JVET-L0100 for test CE10.1.1.c and JVET-L0148 for test CE10.1.2.b.  Compared to VTM-2.0.1, the following BD-rates and run-times are reported according to the JVET common test conditions:

RA:	-1.39% (Y),		-1.31% (Cb),	-1.32% (Cr), at 118% encoder and 105% decoder run-time

LB:		-1.35% (Y),		-0.97% (Cb),	-1.07% (Cr), at 124% encoder and 105% decoder run-time

- Introduction

In test CE10.1.3, a combination of the two individual tests CE10.1.1.c (MediaTek) and CE10.1.2.b (HHI) is tested. These two individual tests are:

- CE10.1.1.c: Multi-hypothesis prediction for improving intra mode
When applied to improve intra mode, multi-hypothesis prediction combines one intra prediction and one merge indexed prediction. In a merge CU, one flag is signaled for merge mode to select an intra mode from an intra candidate list when the flag is true. For luma component, the intra candidate list is derived from 4 intra prediction modes including DC, planar, horizontal, and vertical modes. One intra prediction mode selected by the intra mode index and one merge indexed prediction selected by the merge index are combined using weighted average. For chroma component, DM is always applied without extra signaling.
A detailed description can be found in JVET-L0100 [3].

- CE10.1.2.b: Multi-hypothesis inter-prediction using 4-tap interpolation filter
In the multi-hypothesis inter prediction mode, one or more additional inter prediction signals are signaled, in addition to the conventional uni/bi prediction signal.  The resulting overall prediction signal is obtained by sample-wise weighted superposition.  For the sub-pel interpolation filtering of the additional inter prediction signals, the 4-tap chroma interpolation filter is used.
A detailed description can be found in JVET-L0148 [4].

- Simulation results

The proposed method has been implemented based on BMS-2.0.1.  The simulations were performed following the JVET common test conditions (CTC) [2]. The following table shows an overview of the simulation results. More detailed results can be found in the accompanying Excel file.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.14% | -1.36% | 113% | 103% |
| Class A2 | -1.00% | -1.31% | -1.27% | 118% | 107% |
| Class B | -2.10% | -1.98% | -1.76% | 119% | 104% |
| Class C | -1.12% | -0.59% | -0.79% | 121% | 106% |
| Class E |  |  |  |  |  |
| Overall | -1.39% | -1.31% | -1.32% | 118% | 105% |
| Class D | -0.82% | -0.64% | -0.58% | 124% | 105% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.71% | -1.76% | -1.78% | 122% | 102% |
| Class C | -1.16% | -0.67% | -0.83% | 123% | 106% |
| Class E | -1.03% | -0.04% | -0.21% | 128% | 108% |
| Overall | -1.35% | -0.97% | -1.07% | 124% | 105% |
| Class D | -0.82% | -0.67% | -1.43% | 125% | 105% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |

- Conclusion

This contribution reports the results of test 1.3 of Core Experiment 10 on combined and multi-hypothesis prediction [1].  This test CE10.1.3 is a combination of the two individual tests CE10.1.1.c (MediaTek) [3] and CE10.1.2.b (HHI) [4].  The following gains relative to VTM-2.0.1 are reported according to the JVET common test conditions (CTC) [2]:

RA:	-1.39% (Y),		-1.31% (Cb),	-1.32% (Cr), at 118% encoder and 105% decoder run-time

LB:		-1.35% (Y),		-0.97% (Cb),	-1.07% (Cr), at 124% encoder and 105% decoder run-time

- References

- C.-W. Hsu, M. Winken, “Description of Core Experiment 10: Combined and multi-hypothesis prediction,” JVET-K1030, Ljubljana, SI, July 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, SI, July 2018.

- M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode,” JVET-L0100, Macao, CN, October 2018.

- M. Winken, H. Schwarz, D. Marpe, T. Wiegand, “CE10: Multi-hypothesis inter prediction (Tests 1.2.a - 1.2.c),” JVET-L0148, Macao, CN, October 2018.

- Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).