| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0387-v2 |
| --- | --- |

| Title: | CE6.3.3: Secondary Transforms Coupled with a Simplified Primary Transformation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Amir Said, Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results of CE6.3.3 which tests a simplification proposed in JVET-K0405 to the transform scheme in JVET-K0374. The proposed simplification is based on the idea of coupling primary and secondary transforms and signaling them together as in JVET-K0085 and JVET-K0305. This proposal combines the secondary transforms in JVET-K0374 with a simplified set of primary transforms consisting of DST-7 and DCT-2 only for intra coding. The experimental results show that:

- if both 4x4 and 8x8 NSSTs are enabled, the proposed method provides 1.3%, 0.73% and 0.34% average luma BD-rate gains over VTM 2.0.1 under AI, RA and LD configurations, respectively.

- if only 4x4 NSSTs are enabled, the proposed method provides 0.36%, 0.19% and 0.10% average luma BD-rate gains over VTM 2.0.1 under AI, RA and LD configurations, respectively.

# Introduction

In JVET-C0053, a set of secondary transforms, namely HyGT, is proposed to replace the original 4x4 matrix-based NSSTs. Moreover, in JVET-C0063, a set of HyGTs on top of 8x8 NSST is proposed to achieve higher coding performance. JVET-J0062 proposes a method to implement NSSTs of dimension 64 (i.e., 8x8 NSSTs) using Hierarchically Structured Matrix-based Transform (HSMT) as an alternative to HyGT [1,2].

As in HyGT, HSMT implements NSST using multiple passes of smaller transforms for a given block. However, instead of using the pairwise Givens rotations (i.e., butterfly structures) or a full matrix, a hierarchical structure with multiple passes consisting of smaller matrices and permutations are used to define a non-separable transform. Figure 1 illustrates how HSMT is used for transforming 64 samples (i.e., 8×8 top-left subblocks in NSST) defined by a collection of 8×8 matrices and permutations.

Figure 1:  Hierarchically Structured Matrix-based Transformation

In JVET-K0374, HSMTs are applied as secondary transforms on top of the primary transformation defined in BMS, where primary and secondary transform indexes are signaled separately, and the encoder searches multiple combinations of primary and secondary transforms. Coupling of primary and secondary transforms as in JVET-K0085 and JVET-K0305 allows the encoder to search fewer number of candidates and signal both primary and secondary transform information jointly.

This contribution proposes to couple the NSSTs in JVET-K0374 with a reduced set of primary transforms. Section 2 describes the proposed scheme in detail and the corresponding results are presented in Section 3.

# CE Description: Proposed method

The proposed method couples NSSTs in JVET-K0374 with a two set of primary transforms, defined by DCT-2 and DST-7 for luma intra coding.

Figure 2 illustrates the signaling used for the coupled primary and secondary transforms, where the signaling is the same as AMT signaling in BMS (with a 1-bit flag and 2-bit signaling of AMT indexes). The only difference is assignment of the set of transforms Tset = {T0, T1, T2, T3, T4}.

Figure 2:  Signaling of the coupled transforms T0, T1,T2,T3 and T4 and their assignment.

In JVET-K0085and JVET-K0305, look-up table-based assignments of coupled AMT and NSSTs are proposed. This contribution proposes the following block-size dependent selection of the transform set (Tset) without requiring a look-up table for intra transformation:

- If the with or height of a block is smaller than 8, Tset is defined as:

T0 = {DCT2,DCT2}, which applies DCT-2 vertically and horizontally.

T1 = {DST7,DST7}, which applies DST-7 vertically and horizontally.

T2 = {DST7,DST7,NSST1}, which applies DST-7 vertically and horizontally and NSST1 as the secondary transform.

T3 = {DST7,DST7,NSST2}, which applies DST-7 vertically and horizontally and NSST2 as the secondary transform.

T4 = {DST7,DST7,NSST3}, which applies DST-7 vertically and horizontally and NSST3 as the secondary transform.

- Otherwise, Tset is defined as:

T0 = {DCT2,DCT2}, which applies DCT-2 vertically and horizontally.

T1 = {DST7,DST7}, which applies DST-7 vertically and horizontally.

T2 = { DCT2, DCT2,NSST1}, which applies DCT -2 vertically and horizontally and NSST1 as the secondary transform.

T3 = {DCT2, DCT2,NSST2}, which applies DCT-2 vertically and horizontally and NSST2 as the secondary transform.

T4 = {DCT2, DCT2,NSST3}, which applies DCT-2 vertically and horizontally and NSST3 as the secondary transform.

# Results

The results of the proposed method tested under CTC [5] in Table 1.1.

Table 1.1: Results of the proposed method over VTM 2.0.1 obtained (CE6.3.3-a) by enabling 4x4 NSSTs only and (CE6.3.3-b) by enabling both 4x4 and 8x8 NSSTs in JVET-K0374.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test: CE6.3.3-a Over VTM2.0.1 |  |  |  |  | Test: CE6.3.3-b Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.19% | -0.25% | 95% | 95% | -1.08% | -0.44% | -0.66% | 95% | 96% |
| Class A2 | -0.04% | -0.24% | -0.27% | 98% | 97% | -0.64% | -0.41% | -0.51% | 98% | 98% |
| Class B | -0.25% | -0.47% | -0.40% | 97% | 98% | -1.09% | -0.98% | -0.89% | 98% | 98% |
| Class C | -0.70% | -0.91% | -0.81% | 94% | 99% | -1.96% | -1.42% | -1.46% | 96% | 100% |
| Class E | -0.68% | -0.68% | -0.77% | 96% | 97% | -1.64% | -1.36% | -1.40% | 98% | 100% |
| Overall | -0.36% | -0.52% | -0.51% | 96% | 97% | -1.30% | -0.96% | -1.00% | 97% | 99% |
| Class D | -0.79% | -0.75% | -1.22% | 94% | 98% | -1.61% | -1.20% | -1.52% | 98% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Test: CE6.3.3-a Over VTM2.0.1 |  |  |  |  | Test: CE6.3.3-b Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.09% | -0.08% | 98% | 98% | -0.63% | 0.09% | -0.14% | 98% | 99% |
| Class A2 | -0.14% | -0.25% | -0.22% | 99% | 100% | -0.42% | 0.05% | -0.12% | 99% | 101% |
| Class B | -0.14% | -0.17% | -0.11% | 101% | 102% | -0.68% | -0.22% | -0.20% | 101% | 103% |
| Class C | -0.36% | -0.07% | -0.20% | 98% | 97% | -1.11% | -0.23% | -0.38% | 98% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.19% | -0.15% | -0.15% | 99% | 100% | -0.73% | -0.11% | -0.22% | 99% | 100% |
| Class D | -0.37% | -0.50% | -0.22% | 99% | 101% | -0.76% | -0.45% | -0.01% | 100% | 101% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
|  | Test: CE6.3.3-a Over VTM2.0.1 |  |  |  |  | Test: CE6.3.3-b Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.08% | 0.00% | 101% | 104% | -0.22% | -0.20% | 0.01% | 101% | 103% |
| Class C | -0.09% | -0.04% | -0.14% | 99% | 100% | -0.37% | -0.08% | -0.19% | 100% | 100% |
| Class E | -0.27% | -0.19% | -0.21% | 100% | 99% | -0.50% | 0.23% | -0.48% | 100% | 96% |
| Overall | -0.10% | -0.09% | -0.10% | 100% | 101% | -0.34% | -0.05% | -0.18% | 100% | 100% |
| Class D | -0.08% | 0.92% | -0.38% | 99% | 101% | -0.18% | 0.28% | -0.15% | 100% | 99% |

# Conclusions

In this contribution, a simplified set of primary transforms are defined and combined with the NSSTs in JVET-0374. The proposed scheme provides AI 1.3% coding gain over VTM 2.0.1, tested on BMS 2.0.1 with VTM configurations (i.e., CTC [5]) if both 4x4 and 8x8 NSSTs are used, while the AI coding gain significantly drops to 0.36% if only 4x4 NSSTs are enabled.

# References

- X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi, “EE2.7: TU-level non-separable secondary transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-C0053, 3rd Meeting

- X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, “EE2.7 related: Improved non-separable secondary transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-D0120, 4th Meeting

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 10th meeting, San Diego, CA, USA, April 2018.

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, Slovenia, July 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).