| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0388-v1 |
| --- | --- |

| Title: | CE3: Multiple neighbor-based linear model (Test 5.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hui-Yu Jiang Hong-Jheng Jhu Yao-Jen Chang | Email: | teresa.hy.jiang@foxconn.com ted.hj.jhu@fii-foxconn.com morriss.yj.chang@foxconn.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of multiple neighbor-based linear model (MNLM) in CE3-5.3.1. On top of CCLM + CCLM Cb-to-Cr (CE3-5.2.2), MNLM further enables 3 MMLM modes: MMLM with Above & Left neighbouring data, MMLM with Above-only neighbouring data, and MMLM with Left-only neighbouring data. One line buffer restriction proposed in CE3-5.8.1 for the CUs on the above CTU boundary is also applied to the proposed MNLM. On top of VTM, AI results show -0.6%/-6%/-6% YUV-BDrate with 102/102% coding time change, and RA results show -x.x%/-x.x%/-x.x% YUV-BDrate with 102/102% coding time change.

- Introduction

In JVET-K0073 [1], the multiple neighbor-based linear model (MNLM) uses multiple neighbor sets for MMLM to cover various linear relationships between luma samples and chroma samples in CU. It was included into Core Experiment (CE) 3 [2] to evaluate the performance [3]. The following table shows the description for CE test reported in this document:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 5.3.1 | On top of CE3-5.2.2 (CCLM + CCLM Cb-to-Cr) and CE3-5.8.1 (CTU line buffer restriction), multiple neighbor-based LM (MNLM) is proposed as follows: Mode 0: CCLM (Above & Left neighboring data) Mode 1: MMLM (Above & Left neighboring data) Mode 2: MMLM (Above-only neighboring data) Mode 3: MMLM (Left-only neighboring data) Note: Mode 0 is already in VTM 2.0.1 | Hui-Yu Jiang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |

- Multiple Neighbor-based Linear Model (MNLM)

On top of Test 5.2.2 (CCLM + CCLM Cb-to-Cr) and Test 5.8.1 (CTU line buffer restriction), the MNLM uses multiple neighbor sets for MMLM to cover various linear relationships between luma samples and chroma samples in CU. As depicted in Fig. 1, three neighbor sets, N1, N2, N3, are enabled in the MMLM:

N1: A, B, C, D

N2: C, D, F, H

N3: A, B, E, G

Fig. 1. Multiple neighbors for LM and MMLM

The different cross component linear model (CCLM) prediction modes in MNLM are listed in the table below:

| Mode index | Description |
| --- | --- |
| 0 | CCLM (Above & Left neighbors) |
| 1 | MMLM (Above & Left neighbors) |
| 2 | MMLM (Above neighbors) |
| 3 | MMLM (Left neighbors) |

Mode 0, Mode 1, Mode 2, Mode 3, and Mode 4 use the same down-sampling filter, but different neighbor sets for LM and MMLM derivation.

- Experiment result

The MNLM is tested on top of VTM under the common test conditions specified in [2] and [3]. Table 1 shows the experimental results for Test 5.3.1. Table 2 shows the supplemental experiment results for Test 5.3.1 with the CTU line buffer restriction disabled (Test 5.3.1 + Test 5.8.1 disabled). All coding time are not reliable (update soon).

Table 1. Summary of experimental results (Test 5.3.1 with CTU line buffer restriction v.s. VTM anchor)

Table 2. Summary of supplemental experiment results (Test 5.3.1 without CTU line buffer restriction v.s. VTM anchor)

- Conclusion

This contribution shows the experimental results of Test 5.3.1. It is observed that the proposed MNLM with CTU line buffer restriction could provide the best performance in terms of bitrate reduction, coding time, and complexity among CE3-5 tests. Therefore, it is recommended to adopt MNLM into VTM3.0.

- Reference

- Y.-J. Chang, H.-Y. Jiang, “CE3: Multiple neighbor-based linear model (Tests 4.1.3, 4.1.4, and 4.1.5),” JVET-K0073, Ljubljana, Slovenia, Jul. 2018.

- G. V. der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-K1023, Ljubljana, Slovenia, Jul. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, Slovenia, Jul. 2018.

- Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).