| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0397-v1 |
| --- | --- |

| Title: | CE7: Complexity Reduction of Context Model Selection of Transform Coefficient Levels |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Cheung Auyeung Jianle Chen | Tel: Email: | 1-408-330-4690 cheung.auyeung@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This CE 7.1.1 is intended to reduce the complexity in the context model selection of transform coefficient levels in JVET-K0072. In JVET-K0072, the context model selection is based on the location of the transform coefficient. In this CE, the context model selection is based on the location of the coefficient group (CG) for all CG except the CG containing the DC. For VTM standard AI, RA, and LB configuration, the luma BDR is 0.05, 0.04 and 0.01 respectively. For VTM low complexity AI, RA, and LB configuration, the luma BDR is 0.01, 0.01 and -0.01 respectively.

# Introduction

This document reports the approach and results of CE7.1.1. This CE is intended to reduce the complexity in the context model selection of transform coefficient levels in JVET-K0072, which was adopted in Versatile Video Coding Draft 2 (VVC2).

The selection of the index of the probability models for the syntax element sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, rem_abs_gt2_flag in VVC2 is determined by a sum of a context index base and a context index offset. The context index offset depends on the values of the absolute levels or partially reconstructed absolute levels in a local neighbourhood. The template used is illustrated in Figure 1.

Figure 1: Illustration of the template used for selecting probability models. The black square specifies the current scan position and the blue squares represent the local neighbourhood used.

The context index base depends on the location of the transform coefficient. An example of the offset index for sig_coeff_flag and par_level_flag is shown in Figure 2 and Figure 3, respectively.

Figure 2: VVC2 sig_coeff_flag base index of 16x16 luma block.

Figure 3: VVC2 par_level_flag base index of 16x6 luma block.

# Base Index in CE7.1.1

# In VTM-2.0.1, the context index base is computed as a function of the current position ( xC , yC ). of the transform coefficient. To reduce computation, this document defines the index base as a function of the location of the coefficient group for all coefficient groups except the coefficient group containing the DC transform coefficient.

# For the coefficient group containing the DC transform coefficient, the index base is the same as VVC2 and it is a function the diagonal position d, which is set according to

d = xC + yC

# For the other coefficient groups, the index base for all coefficients within the same coefficient group are the same to reduce computation. The index base may change from one coefficient group to another coefficient group.

## Context modelling of the significance flag

The probability model used for coding the sig_coeff_flag with cgIdx==0 is determined as follows:

- If the current colour component represents the luma component, the context index ctxIdSig is determined according to

ctxIdBaseSig = ( d < 2 ? 12 : ( d < 5 ? 6 : 0 ) )

ctxIdSig = 18 * max(0, state−1) + min( sumAbs1, 5 ) + ctxIdBaseSig

- Otherwise (the current colour component represents a chroma component), the context index ctxIdSig is determined according to

ctxIdBaseSig = ( d < 2 ? 6 : 0 )

ctxIdSig = 12 * max(0, state−1) + min( sumAbs1, 5 ) + ctxIdBaseSig

The probability model used for coding the sig_coeff_flag with cgIdx!=0 is determined as follows:

- If the current colour component represents the luma component, the context index ctxIdSig is determined according to

ctxIdBaseSig = 0

ctxIdSig = 18 * max(0, state−1) + min( sumAbs1, 5 ) + ctxIdBaseSig

- Otherwise (the current colour component represents a chroma component), the context index ctxIdSig is determined according to

ctxIdBaseSig = 0

ctxIdSig = 12 * max(0, state−1) + min( sumAbs1, 5 ) + ctxIdBaseSig

An example of the ctxIdBaseSig for 16x16 luma block from this CE is shown in Figure 4.

Figure 4: CE7.1.1 sig_coeff_flag base index of 16x16 luma block.

## Context modelling of the parity flag

Similar as for the significance flag, different sets of probability models are used for luma and chroma. The same set of probability models is used for both chroma components.

For coefficient group with cgIdx==0, the context index ctxIdPar specifying the probability model used is determined as follows:

- If the current scan position is equal to the position of the last non-zero level (as indicted by the transmitted x and y coordinates), ctxIdPar is set equal to 0;

- Otherwise, if the current colour component is the luma component, the context index is determined according to

ctxIdBasePar = ( d == 0 ? 15 : ( d < 3 ? 10 : ( d < 10 ? 5 : 0 ) ) )

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 ) + ctxIdBasePar

- Otherwise (the current colour component is the chroma component), the context index is determined according to

ctxIdBasePar = ( d == 0 ? 5 : 0 )

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 ) + ctxIdBasePar

For coefficient group with cgIdx>0, the context index ctxIdPar specifying the probability model used is determined as follows:

- If the current scan position is equal to the position of the last non-zero level (as indicted by the transmitted x and y coordinates), ctxIdPar is set equal to 0;

- Otherwise, if the current colour component is the luma component, the context index is determined according to

ctxIdBasePar = cgIdx>3 ? 0 : 5

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 ) + ctxIdBasePar

- Otherwise (the current colour component is the chroma component), the context index is determined according to

ctxIdBasePar = 0

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 ) + ctxIdBasePar

An example of the ctxIdBasePar for 16x16 luma block from this CE is shown in Figure 5.

Figure 5: CE7.1.1 par_level_flag base index of 16x6 luma block.

## Context modelling of the greater flags and abs_remainder

There are no changes in the context modelling of greater flags and abs_remainder flag from JVET - K0072- v2 where

. 			ctxIdGt1 = ctxIdPar
			ctxIdGt2 = ctxIdPar

# Simulation Results

## VTM2 standard configuration results

The following results are generated with the CTC configuration files.

## VTM2 low complexity configuration results

The following results are generated with the CTC configuration files and the following options:

--DepQuant=0 --RDOQ=0 --RDOQTS=0 --SignHideFlag=1

# Reference

- H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, “Non-CE7: Alternative Entropy Coding for Dependent Quantization”, JVET-K0072-v2, 11th JVET Meeting, Ljubljana, SI, 10–18 July, 2018.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2)”, JVET-K1001-v4, 11th JVET Meeting, Ljubljana, SI, 10–18 July, 2018.

- J. Chen, Y. Ye, S.H. Kim, “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2)”, JVET-K1002-v1, 11th JVET Meeting, Ljubljana, SI, 10–18 July, 2018.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

# Patent rights declaration(s)

Huawei Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).