| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0415-v1 |
| --- | --- |

| Title: | Tile groups for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes that tiles are adopted into VVC and that slices in VVC consist of an integer number of complete tiles instead of consisting of an integer number of complete CTUs.

The following is proposed:

- Adopt tiles into the VVC draft (HEVC tiles or flexible tiles as in JVET-K0260)

- Change the name of slices to tile groups. Keep the slice header but call it tile group header

- Replace the slice address by a tile group address in the tile group header

  - This means that the existing slices become tile groups that always contain an integer number of complete tiles

  - This also means that raster-scan CTU slices are no longer supported

- Add a syntax element num_tiles_in_tile_group that specifies the number of tiles there is in the tile group. This is used both for the decoder to know when the tile group ends and to know how many tile pointers there are in the tile group header

- Remove the end_of_slice_flag syntax element, instead the end of the tile group is given by the tile group address and the num_tiles_in_tile_group code word.

This is proposed as a starting point for tiles.

# Introduction

The slice is a well-known concept in video coding and has been around for a long time. In the H.263 specification, slices are specified in Annex K which includes the following text: “The purposes of this mode are to provide enhanced error resilience capability, to make the bitstream more amenable to use with an underlying packet transport delivery, and to minimize video delay.” To those properties one can also add the capability of providing parallel processing.

The tile is also a well-known concept in video coding. It provides similar functionalities as slices but does not use as much header data. On the other hand, tiles are not as error robust as slices.

# Proposal

We propose that tiles are adopted into VVC and that slices in VVC are made to consist of an integer number of complete tiles instead of consisting of an integer number of complete CTUs.

The following is proposed as the starting point for tiles in VVC:

- Adopt tiles into the VVC draft (HEVC tiles or flexible tiles as in JVET-K0260)

- Change the name of slices to tile groups. Keep the slice header but call it tile group header

- Replace the slice address by a tile group address in the tile group header

  - This means that the existing slices become tile groups that always contain an integer number of complete tiles

  - This also means that raster-scan CTU slices are no longer supported

- Add a syntax element num_tiles_in_tile_group that specifies the number of tiles there is in the tile group. This is used both for the decoder to know when the tile group ends and to know how many tile pointers there are in the tile group header

- Remove the end_of_slice_flag syntax element, instead the end of the tile group is given by the tile group address and the num_tiles_in_tile_group code word.

## Picture parameter set option 1

We propose as an option 1 that the HEVC syntax for the tile grid is proposed as a starting point, as indicated by red text below. The semantics is identical to the semantics in HEVC v5.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| tiles_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Picture parameter set option 2

As an option 2, we propose flexible tiles as presented in JVET-K0260. The semantics are as described in JVET-K0260.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| tiles_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| number_of_tiles_in_picture_minus2 | ue(v) |
| subtile_width_minus1 | ue(v) |
| subtile_height_minus1 | ue(v) |
| for ( i=0; i < number_of_tiles_in_picture_minus2+2 ; i++ ) { |  |
| if ( i > 0 ) |  |
| use_previous_tile_size_flag | u(1) |
| if ( use_previous_tile_size_flag == 0) { |  |
| tile_width_minus1[ i ] | ue(v) |
| tile_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Tile group header

The name of the slice header is replaced by tile group header. The slice address is replaced by a tile group address that is expressed in units of tiles. Tile pointers are used to locate each tile in the VCL of the tile group.

| tile_group_headerslice_header( ) { | Descriptor |
| --- | --- |
| slicetile_group_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| if( tiles_enabled_flag ) |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |

tile_group_address specifies the location of the first CTU in the tile group.

num_tiles_in_tile_group_minus1 + 1 specifies the number of tiles in the tile group. The value of num_tiles_in_tile_group_minus1 shall be in the range of 0 to NumTilesInPic-1, inclusive

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset_minus1[ i ] syntax elements. The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

entry_point_offset_minus1[ i ] as in HEVC

## Tile group data

We propose that the slice_data() is renamed to tile_group_data() and that the tile group address and the number of tiles in the tile group given in the slice header are used to determine the end of the tile group. No code word similar to end_of_slice_flag is necessary.

| slice_data( ) { | Descriptor |
| --- | --- |
| do { |  |
| coding_tree_unit( ) |  |
| end_of_slice_flag | ae(v) |
| CtbAddrInRs++ |  |
| } while( !end_of_slice_flag ) |  |
| } |  |

| tile_group_data( ) { | Descriptor |
| --- | --- |
| do { |  |
| coding_tree_unit( ) |  |
| } while( !end_of_tile_group() ) |  |
| } |  |

end_of_tile_group( ) is specified as follows:

–	If the last decoded CTU is the last CTU in decoding order of the last tile in the tile group in decoding order, the return value of end_of_tile_group( ) is equal to TRUE

–	Otherwise, the return value of end_of_tile_group( ) is equal to FALSE

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).