| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0421-v1 |
| --- | --- |

| Title: | CE6-related: fast implementation of MTS transforms using matrix multiplication |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Gagan Rath, Edouard Francois (Technicolor) | Tel: Email: | karam.naser@technicolor.com gagan.rath@technicolor.com edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

The current MTS design considers the two additional transforms of DST-7 and DCT-8. Compared to DCT-2, they cannot be decomposed into partial butterfly representation. Although there exist some recent proposals to speed-up the implementation of DST-7 and DCT-8, they are generally specific to each transform size, and they contain several un-repeated and non-generic operations, which is asserted to reduce the possibility of optimal parallelization. This contribution proposes to decompose the MTS transforms basis into smaller matrices and to perform matrix multiplication to implement the transforms. The proposed approach is reported to be generic, independent from the transform size, and SIMD implementation friendly. Compared to a full matrix multiplication approach of VTM2 MTS transforms, the proposal is reported to significantly reduce the required memory (5% of the full matrices memory need) and the number of operations (number of multiplications reduced by 5 to 6). It is also reported that the proposed implementation has no impact on the coding performance

# Introduction

Speed-up methods for the MTS transforms are currently under investigation, where multiple contributions have been proposed. The first one is based on the DCT-2 decomposition along with extra adjustment stages [1]. Those stages consist of multiplication with sparse matrices defined for each MTS transform. A second proposal is based on the intrinsic properties of the MTS transform matrices, where some redundant numbers or zeros exist [3]. Exploiting these properties can reduce the overall number of multiplications and additions. A theoretical approach is also proposed in [2] to implement MTS transforms using Discrete Fourier Transform (DFT). Even though the number of operations is reduced with these approaches, matrix multiplication is still highly preferred. This is thanks to its generic and simple architecture, which is also hardware and/or software friendly due to the possibility of parallelization and SIMD implementation.

# Proposed method

The MTS transforms (DST-7 and DCT-8) are related to each other by simple operations of order reversal and sign changing. Therefore, the rest of the document will focus on DCT-8 implementation. The present method is highly inspired by the one in [2], where the DCT-8/DST-7 are implemented via the prime factor algorithm of DFT for real input data and imaginary output data. Namely, a length N transform is represented by a length 2N+1 DFT, which in turn reshapes into N1xN2 matrix using prime factor algorithm, N1xN2 being equal to (2N+1). In other words, the 2N+1 DFT is implemented by a two stages DFT, where the first stage consists of N2 DFT’s of length N1, and the second stage consists of N1 DFT’s of size N2. The following points are important for the fast design:

- The input samples are real. This simplifies the first stage N2 DFT’s of length N1 to a Real Input DFT (DFT_RI).

- The transform coefficients correspond to the imaginary output part of DFT. This simplifies the second stage N1 DFT’s of length N2 to an imaginary output DFT (DFT_IO).

- The input to the first DFT of the second stage is real data. This simplifies it to a Real Input and Imaginary Output DFT (DFT_RI_IO), see Figure 1.

- Since the input data to the first stage are real, the first stage output is even symmetric. Thus, only half the coefficients need to pass to the second stage. This reduces the number of the second stage DFT’s to (N2+1)/2.

Putting this all together comes up with the generic design shown in Figure 1. It should be noted that the input and output permutations include also some sign changes, which is not mentioned in detail. The MTS transforms for sizes of 16 and 32 are represented by 33 and 65 DFTs, which are decomposed into 3x11 and 5x13 DFT (respectively).

Figure 1. Generic implementation of DCT-8/DST-7 using prime factor algorithm.

The DFT matrix can be written as a sum of real (Re) and imaginary (Im) matrices:

DFT = Re + j × Im

where each coefficient is defined as follows (for a length N DFT):

dft(n,m) = cos(2π n m/N) + j× sin(2π n m/N)

The real matrix corresponds to the cosine part, and the imaginary correspond to the sine part. The following can be noticed:

- In first line of the DFT matrix (n=0), the real matrix is 1, and imaginary part is zero

- The real part of the DFT matrix is even symmetric and the imaginary part is odd symmetric. Thus, only the first quadrant of these matrices is needed for the multiplication.

Algorithm2: Multiplying by the imaginary matrix of DFT

Input: Input samples row vector (In) of length N, and imaginary matrix of DFT (Im)

Output: Output coefficient row vector (Out) of length N

Out[ 0 ] = 0

Out[ 1..(N–1)/2 ] = ( In[ 1..(N–1)/2 ] – In[ N–1..(N+1)/2 ] ) × Im[ 0..(N-1)/2 – 1 , 0..(N-1)/2 – 1 ]

Out[ N–1..(N+1)/2 ] = – Out[ 1..(N–1)/2 ]

Algorithm1: Multiplying by the real matrix of DFT

Input: Input samples row vector (In) of length N, and real matrix of DFT (Re)

Output: Output coefficient row vector (Out) of length N

Out[ 0 ] = sum(In)

Out[ 1..(N–1)/2 ] = ( In[ 1..(N–1)/2 ] + In[ N–1..(N+1)/2 ] ) × Re[ 0..(N-1)/2 – 1 , 0..(N-1)/2 – 1 ]

Out[ N-1..(N+1)/2 ] = Out[ 1..(N–1)/2 ]

The algorithms for multiplying the input by the real and imaginary parts are depicted in pseudo-code below.

Note: “×” is used as scalar multiplier symbol and as matrix multiplier symbol.

## DFT_RI

The DFT of real row input vector r can be described as:

C = r × (Re + j×Im) = r×Re + j×r×Im

The real input data (r) is multiplied with both the real (Re) and imaginary (Im) of the DFT matrix. Thus, both Algorithm 1 and 2 are used. Below is the C++ implementation of the function.

void FFT_RealIn_MM(const TCoeff *src_r, TCoeff* dst_r, TCoeff* dst_i, const TMatrixCoeff* FFT_R, const TMatrixCoeff* FFT_I, int n, int line)

{

const TMatrixCoeff* fft_r = FFT_R;

const TMatrixCoeff* fft_i = FFT_I;

dst_r[0] = 0;

dst_i[0] = 0;

for (int i = 0; i < n; i++)

dst_r[0] += src_r[i];

for (int i = 1; i <= (n - 1) / 2; i++)

{

DFT_c1[i - 1] = src_r[i] + src_r[n - i];

DFT_c2[i - 1] = src_r[i] - src_r[n - i];

}

for (int i = 1; i <= (n - 1) / 2; i++)

{

TCoeff t1 = src_r[0] << TCH_FFT_Shift;

TCoeff t2 = 0;

for (int j = 0; j < (n - 1) / 2; j++)

{

t1 += DFT_c1[j] * fft_r[j];

t2 += DFT_c2[j] * fft_i[j];

}

fft_r += (n - 1) / 2;

fft_i += (n - 1) / 2;

t1 >>= FFT_Shift;

t2 >>= FFT_Shift;

dst_r[i*line] = t1;

dst_i[i*line] = t2;

dst_r[(n - i)*line] = t1;

dst_i[(n - i)*line] = -t2;

}

}

FFT_Shift is the design shift used for the integer representation.

The above function requires:

- (N – 1)^2 / 2 multiplications

- (N – 1) * ( (N+5) / 2 ) additions

## DFT_RI_IO

As in previous case, the DFT for real row input vector r can be described as:

C = r × (Re + j×Im) = r×Re + j× r×Im

When the imaginary output is only considered, the above function is reduced to:

Ci =  r×Im

It requires the multiplication of the input data (r) by the imaginary DFT matrix (Im) only. Thus, only Algorithm 2 is employed. Its c++ implementation is below:

void FFT_RealIn_ImagOut_MM(const TCoeff *src_r, TCoeff* dst_i, const TMatrixCoeff* FFT_I, int n)

{

const TMatrixCoeff* fft_i = FFT_I;

dst_i[0] = 0;

for (int i = 1; i <= (n - 1) / 2; i++)

DFT_c2[i - 1] = src_r[i] - src_r[n - i];

for (int i = 1; i <= (n - 1) / 2; i++)

{

dst_i[i] = 0;

for (int j = 0; j < (n - 1) / 2; j++)

dst_i[i] += DFT_c2[j] * fft_i[j];

fft_i += (n - 1) / 2;

dst_i[n - i] = -dst_i[i];

}

}

This function requires:

- (N – 1)^2 / 4 multiplications

- (N – 1)^2 / 4 additions

## DFT_IO

The DFT for complex data (made of real row vector r and imaginary row vector i) can be described as:

C = (r + j×i) × (Re + j×Im) = r×Re – i×Im + j× (r×Im + r×Re)

When the imaginary part is only considered:

Ci =  r×Im + i×Re

Then, this function requires the multiplication of the real input data (r) by the imaginary DFT matrix (Im), and the imaginary input data (i) by the real DFT matrix (Re). Thus, both algorithm 1 and algorithm 2 are required. The c++ implementation is provided below:

void FFT_ImagOut_MM(const TCoeff *src_r, const TCoeff *src_i, TCoeff* dst_i, const TMatrixCoeff* FFT_R, const TMatrixCoeff* FFT_I, const TMatrixCoeff cDFT, int n)

{

const TMatrixCoeff* fft_r = FFT_R;

const TMatrixCoeff* fft_i = FFT_I;

dst_i[0] = 0;

for (int i = 0; i < n; i++)

dst_i[0] += src_i[i];

dst_i[0] *= DFT_SCALE;

for (int i = 1; i <= (n - 1) / 2; i++)

{

DFT_c1[i - 1] = src_i[i] + src_i[n - i];

DFT_c2[i - 1] = src_r[i] - src_r[n - i];

}

for (int i = 1; i <= (n - 1) / 2; i++)

{

TCoeff t1 = src_i[0] * DFT_SCALE;

TCoeff t2 = 0;

for (int j = 0; j < (n - 1) / 2; j++)

{

t1 += DFT_c1[j] * fft_r[j];

t2 += DFT_c2[j] * fft_i[j];

}

fft_r += (n - 1) / 2;

fft_i += (n - 1) / 2;

dst_i[i] = t1 + t2;

dst_i[n - i] = t1 - t2;

}

}

Where DFT_SCALE is a constant used to scale the coefficients as in the VTM software.

This function requires:

- (N – 1)^2 / 2 multiplications

- (N – 1) * (N/2 + 2) – 1 additions

## Overall implementation

The implementation requires a generic function to compute the DFT for the 3 different cases (DFT_RI, DFT_IO and DFT_RI_IO) along with the input/output permutation for each transform type (forward vs inverse transform for DCT-8 and DST-7). The DFT matrices are stored as short integers, where only half rows and half columns are kept. For example, the 13 DFT is stored as 6x6 real matrix and 6x6 odd matrix. This is because only half the rows and half the columns are required in this method. The overall number of coefficients, as shown in Table 2, is only 132. This represents 5% (132/2560) of the total number of coefficients required by the VTM2 software (see Table 2), and thus the total memory is reduced to 5% of its initial design.

| Matrix | Number of Coefficients |
| --- | --- |
| DFT_3_r | 1 |
| DFT_3_i | 1 |
| DFT_5_r | 4 |
| DFT_5_i | 4 |
| DFT_11_r | 25 |
| DFT_11_i | 25 |
| DFT_13_r | 36 |
| DFT_13_i | 36 |
| Sum | 132 |

Table 1. Number of coefficients required for the implementation of the proposed method.

| Matrix | Number of Coefficients |
| --- | --- |
| DST7_16 | 256 |
| DST7_32 | 1024 |
| DCT8_16 | 256 |
| DCT8_32 | 1024 |
| Sum | 2560 |

Table 2. Number of coefficients required for the implementation of the VTM MTS transforms.

# Experimental Results

## Basic results

The method is implemented on the VTM 2.0.1 software. The CTCs are used on RA configuration, with VTM software as the anchor. The results are provided in Table 3. It is observed that the proposed approach has no impact on coding performance. Regarding the encoding/decoding time, the impact is not significant as the transform takes a small part of the encoding/decoding runtime.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.0% | 0.0% | 0.0% | 100% | 100% |
| Class A2 | 0.0% | -0.1% | -0.1% | 98% | 100% |
| Class B | 0.0% | 0.0% | 0.1% | 100% | 100% |
| Class C | 0.0% | 0.2% | 0.1% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 99% | 99% |
| Class D | 0.0% | -0.3% | -0.1% | 99% | 97% |

Table 3. BD rate performance on RA configuration.

## Number of operations

In this section, the number of operations required to implement each transform is counted. As shown in table 5, the proposed implementation leads to 19% multiplications and 64% additions for 16x16 DCT8/DST7 transforms, and  to 15% multiplications and 42% additions for 32x32 DCT8/DST7 transforms, compared to the current VTM design.

| Process | #Multiplications | #Additions |
| --- | --- | --- |
| N2 DFT_RI(N1) | N2 * (N1 – 1)^2/2 | N2 * (N1 – 1) * (1+N1/2) |
| 1 DFT_RI_RO(N2) | (N2 – 1)^2/4 | (N2-1) * (N2+5)/4 |
| (N1-1)/2 DFT_RO(N2) | (N1 – 1)/2 * ( (N1 – 1)^2/2 ) | (N1 – 1)/2 * ((N2 – 1) * (N2/2+2) – 1) |

Table 4. Total number of operations required to implement DCT8/DST7 whose DFT size is N1*N2.

| Transform | DFT length | Decomposition | Multiplications saving | Additions saving |
| --- | --- | --- | --- | --- |
| 16x16 DCT8/DST7 | 33 | 3x11 | 49 / 256 = 0.19 | 154 / 240 = 0.64 |
| 32x32 DCT8/DST7 | 65 | 5x13 | 156 / 1024 = 0.15 | 421 / 992 = 0.42 |

Table 5. Multiplications and Additions saving (compared to the default VTM design) for MTS transform sizes of 16*16 and 32*32.

## Speedup Test

To show the amount of speed-up, we evaluated the time required specifically by the transform process, using the proposed method, compared to the default method. We ran an independent c++ test and calculated the time for computing 1000 times using both methods. The average time is used. To provide a fair comparison, we set the pragma option as:

#pragma loop(no_vector)

The results shown in Table 6.

| Default Method | Average time (second) | Proposed | Average time (second) | Saving |
| --- | --- | --- | --- | --- |
| DST7_32 | 103.971 | DST7_Fast_32 | 86.595 | 83% |
| InvDST7_32 | 373.498 | InvDST7_Fast_32 | 354.691 | 95% |
| DCT8_32 | 103.703 | DCT8_Fast_32 | 87.665 | 85% |
| InvDCT8_32 | 373.929 | InvDCT8_Fast_32 | 344.952 | 92% |

Table 6. Average time and saving compared to the current VTM2 design.

# Conclusion

This contribution proposes a fast implementation of MTS transforms using matrix multiplication. The process is generic, non-size dependent and is therefore SIMD friendly. Compared to a full matrix multiplication approach of VTM2 MTS transforms, the proposal significantly reduces the required memory (5% of the full matrices memory need) and the number of operations (number of multiplications reduced by 5 to 6). Meanwhile, those simplifications are obtained without any impact on the coding performance.

# Reference

- A. Said, Y-H. Chao, H. Egilmez, M. Karczewicz, V. Seregin, “CE6.1.2: Efficient Implementations of AMT with Transform Adjustment Stages” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0272

- M. Koo, M. Salehifar, J. Lim, S. Kim, “CE 6.1.11: AMT replacement and restriction” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0096

- Z. Zhang, X. Zhao, X. Li, S. Liu, “CE6-related: Fast DST-7/DCT-8 with dual implementation support” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0291

# Patent rights declaration

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).