| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0426-v1 |
| --- | --- |

| Title: | CE5-related: Alternative implementation of CABAC range sub-interval derivation for test CE 5.1.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Paul Haase, Heiner Kirchhoffer, Stefan Matlage, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-409 paul.haase@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution describes an alternative implementation for CABAC range sub-interval derivation, for the state-based probability estimator proposed in [1] and was part of CE5 (CE 5.1.4) [2]. The proposed method uses a combination of the range-probability product computation and table lookup approaches presented in CE5 (Subtest 2) [2], which means a (1-dimensional) table lookup and a multiplication operation.

# Introduction

CABAC range sub-interval derivation in [1] is performed by a lookup in a 2-dimensional table, which outputs the coding interval associated with the LPS (Least Probable Symbol). CE5 includes experiments on different lookup table sizes (e.g. 32x8, 16x8 etc.) and on replacing the table lookup by range probability product computation. An alternative implementation, that uses a combination of both approaches, is proposed. The coding performance is virtually the same.

# Proposed method

The interval subdivision is realized using a one dimensional lookup table (probTabLPS) of size 32 (or 16) and a multiplication operation which is equivalent to using a 32x8 (or 16x8) table.

rangeLPS=probTabLPS16/32stateIdx16/32∙ range>>5+x>>3

with

stateIdx16=abs S0<<4+S1>>8

stateIdx32=abs S0<<4+S1>>7

Parameter x may be 0 or 1.

A 32x8 (or 16x8) rangeTabLPS can be computed by performing the multiplication above, and can therefore be used equivalently.

## Range division using a probTabLPS of dimension 32

The values of rangeLPS, equivalent to those of a 32x8 table, can be obtained by performing:

rangeLPS=probTabLPS32stateIdx32∙ range>>5>>3

Note that x was set to 0. For this configuration probTabLPS32 contains the following values:

probTabLPS32=[ 126, 110, 95, 83, 72, 64, 56, 49, 44, 38, 34, 30, 26, 23, 20, 17,

15, 14, 12, 11, 10, 8, 8, 7, 6, 5, 4, 4, 3, 3, 2, 2]

## Range division using a probTabLPS of dimension 16

The values of rangeLPS, equivalent to those of a 16x8 table, can be obtained by performing:

rangeLPS=probTabLPS16stateIdx16∙ range>>5>>3

Note that x was set to 0. For this configuration probTabLPS16 contains the following values:

probTabLPS16= 117, 90, 69, 53, 41, 32, 24, 19, 15, 12, 9, 7, 5, 4, 3, 2

## Range subdivision lookup tables for CE 5.3.1

The rangeTabLPS (32x8 and 16x8) for CE 5.3.1 are derived with a scheme, that slightly differs from the one proposed. Here x=1:

rangeLPS=probTabLPS16/32stateIdx16/32∙ range>>5+1>>3

with,

probTabLPS16= 107, 82, 63, 48, 38, 29, 22, 17, 13, 11, 8, 7, 5, 4, 3, 2

probTabLPS32=[ 114, 100, 87, 75, 66, 58, 51, 45, 40, 35, 31, 27, 24, 21, 18, 16,

14, 13, 11, 10, 9, 8, 7, 7, 5, 5, 4, 4, 3, 3, 2, 2]

Simulation results show, that the scheme can be reduced to the one proposed, virtually without affecting the coding performance.

# Experimental results

Test results may be incomplete and restricted to 49 Frames. Full VTM 2.1 and BMS 2.1 test results will follow.

The probability estimator of CE 5.1.4 [2] with custom window sizes enabled was used for all tests. This and CABAC range sub-interval derivation were implemented in BMS-2.0.1 and simulations were conducted according to [3]. Runtimes are not accurate due to the simulation conditions (runtimes vary depending on the cluster load).

## Results for probTabLPS of dimension 32

VTM configuration, AI, Reference: VTM 2.0.1-anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM][49fr] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.00% | -0.98% | -1.46% | 109% | 100% |
| Class A2 | -0.67% | -1.57% | -1.42% | 108% | 103% |
| Class B | -0.98% | -1.65% | -1.90% | 109% | 101% |
| Class C | -0.97% | -1.65% | -1.60% | 109% | 103% |
| Class E | -0.84% | -1.14% | -1.39% | 108% | 98% |
| Overall | -0.91% | -1.44% | -1.59% | 109% | 101% |
| Class D | -0.86% | -1.14% | -1.31% | 108% | 100% |
| Class F (optional) | -0.75% | -1.13% | -0.86% | 110% | 100% |

VTM configuration, AI, Reference: CE 5.1.4:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm-2.0_vcgit.hhi.fraunhofer.de_JVET-K-CE5_VVCSoftware_BMS.git_CE5.1.4_021424e_VTM_CM1_49fr |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 109% | 104% |
| Class A2 | 0.00% | 0.00% | 0.00% | 106% | 104% |
| Class B | 0.01% | 0.01% | 0.01% | 106% | 103% |
| Class C | 0.02% | 0.02% | 0.02% | 106% | 102% |
| Class E | 0.03% | 0.03% | 0.03% | 108% | 104% |
| Overall | 0.01% | 0.01% | 0.01% | 107% | 103% |
| Class D | 0.03% | 0.03% | 0.03% | 105% | 102% |
| Class F (optional) | 0.03% | 0.03% | 0.03% | 106% | 102% |

VTM configuration, RA, Reference: VTM 2.0.1-anchor:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM][49fr] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class A2 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class B | -0.90% | -1.41% | -1.23% | 103% | 100% |
| Class C | -0.95% | -1.34% | -0.93% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class D | -0.71% | -0.77% | -0.37% | 103% | 100% |
| Class F (optional) | -0.67% | -0.48% | -0.31% | 102% | 98% |

VTM configuration, RA, Reference: CE 5.1.4:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm-2.0_vcgit.hhi.fraunhofer.de_JVET-K-CE5_VVCSoftware_BMS.git_CE5.1.4_021424e_VTM_CM1_49fr |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class A2 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class B | 0.01% | 0.01% | 0.01% | 109% | 104% |
| Class C | 0.02% | 0.02% | 0.02% | 108% | 104% |
| Class E |  |  |  |  |  |
| Overall | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class D | 0.03% | 0.03% | 0.03% | 107% | 105% |
| Class F (optional) | 0.03% | 0.03% | 0.03% | 107% | 106% |

VTM configuration, LB, Reference: VTM 2.0.1-anchor:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM][49fr] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.93% | -0.89% | -1.06% | 103% | 98% |
| Class C | -0.91% | -1.39% | -1.21% | 103% | 97% |
| Class E | -1.07% | -0.63% | -1.95% | 100% | 95% |
| Overall | -0.96% | -0.99% | -1.33% | 102% | 97% |
| Class D | -0.76% | -0.29% | -0.38% | 103% | 98% |
| Class F (optional) | -0.39% | -0.92% | -1.19% | 103% | 98% |

VTM configuration, LB, Reference: CE 5.1.4:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm-2.0_vcgit.hhi.fraunhofer.de_JVET-K-CE5_VVCSoftware_BMS.git_CE5.1.4_021424e_VTM_CM1_49fr |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 111% | 109% |
| Class C | 0.01% | 0.01% | 0.01% | 111% | 100% |
| Class E | 0.06% | 0.05% | 0.05% | 114% | 100% |
| Overall | 0.01% | 0.01% | 0.01% | 112% | 104% |
| Class D | 0.04% | 0.03% | 0.03% | 107% | 100% |
| Class F (optional) | 0.03% | 0.03% | 0.03% | 112% | 104% |

## Results for probTabLPS of dimension 16

VTM configuration, AI, Reference: VTM 2.0.1-anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM][49fr] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.89% | -0.87% | -1.35% | 109% | 97% |
| Class A2 | -0.57% | -1.47% | -1.32% | 108% | 98% |
| Class B | -0.87% | -1.55% | -1.79% | 109% | 93% |
| Class C | -0.86% | -1.55% | -1.50% | 109% | 104% |
| Class E | -0.73% | -1.03% | -1.28% | 108% | 100% |
| Overall | -0.80% | -1.34% | -1.49% | 108% | 98% |
| Class D | -0.74% | -1.02% | -1.20% | 108% | 104% |
| Class F (optional) | -0.64% | -1.02% | -0.74% | 110% | 99% |

VTM configuration, AI, Reference: CE 5.1.4:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm-2.0_vcgit.hhi.fraunhofer.de_JVET-K-CE5_VVCSoftware_BMS.git_CE5.1.4_021424e_VTM_CM1_49fr |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.11% | 0.11% | 109% | 100% |
| Class A2 | 0.10% | 0.10% | 0.10% | 105% | 99% |
| Class B | 0.12% | 0.12% | 0.12% | 106% | 94% |
| Class C | 0.13% | 0.13% | 0.13% | 106% | 103% |
| Class E | 0.13% | 0.13% | 0.13% | 108% | 105% |
| Overall | 0.12% | 0.12% | 0.12% | 106% | 100% |
| Class D | 0.15% | 0.14% | 0.15% | 105% | 106% |
| Class F (optional) | 0.14% | 0.14% | 0.14% | 106% | 100% |

VTM configuration, RA, Reference: VTM 2.0.1-anchor:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM][49fr] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class A2 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class B | -0.78% | -1.29% | -1.12% | 106% | 99% |
| Class C | -0.83% | -1.22% | -0.81% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class D | -0.59% | -0.65% | -0.25% | 103% | 99% |
| Class F (optional) | -0.55% | -0.36% | -0.20% | 107% | 97% |

VTM configuration, RA, Reference: CE 5.1.4:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm-2.0_vcgit.hhi.fraunhofer.de_JVET-K-CE5_VVCSoftware_BMS.git_CE5.1.4_021424e_VTM_CM1_49fr |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class A2 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class B | 0.13% | 0.13% | 0.13% | 113% | 103% |
| Class C | 0.14% | 0.14% | 0.14% | 108% | 104% |
| Class E |  |  |  |  |  |
| Overall | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class D | 0.15% | 0.15% | 0.15% | 108% | 104% |
| Class F (optional) | 0.15% | 0.15% | 0.15% | 112% | 105% |

VTM configuration, LB, Reference: VTM 2.0.1-anchor:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM][49fr] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.81% | -0.77% | -0.94% | 104% | 103% |
| Class C | -0.79% | -1.27% | -1.09% | 98% | 95% |
| Class E | -0.95% | -0.51% | -1.83% | 103% | 98% |
| Overall | -0.84% | -0.87% | -1.21% | 102% | 99% |
| Class D | -0.63% | -0.16% | -0.25% | 104% | 100% |
| Class F (optional) | -0.27% | -0.80% | -1.07% | 104% | 100% |

VTM configuration, LB, Reference CE 5.1.4:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm-2.0_vcgit.hhi.fraunhofer.de_JVET-K-CE5_VVCSoftware_BMS.git_CE5.1.4_021424e_VTM_CM1_49fr |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | 0.11% | 0.11% | 113% | 115% |
| Class C | 0.14% | 0.13% | 0.13% | 106% | 98% |
| Class E | 0.18% | 0.17% | 0.17% | 118% | 103% |
| Overall | 0.14% | 0.13% | 0.13% | 112% | 106% |
| Class D | 0.17% | 0.16% | 0.16% | 108% | 102% |
| Class F (optional) | 0.16% | 0.16% | 0.16% | 113% | 107% |

## Results for CE 5.3.1

The experimental results for CE 5.3.1 can be found in CE5: Summary Report.

# References

- H. Kirchhoffer, J. Stegemann, D. Marpe, H. Schwarz, T. Wiegand “CE5-related: State-based probability estimator“, JVET-K0430, 11-th Meeting, Ljubljana, SI, July 2018.

- H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine”, JVET-K1025, 11-th Meeting, Ljubljana, SI, July 2018

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).