| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0427-v1 |
| --- | --- |

| Title: | CE15-related: Separate Palette Coding for Luma and Chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Roman Chernyak Sergey Ikonin Jianle Chen | Tel: Email: | +7-925-008-12-86 chernyak.roman@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

The contribution proposes a unification of Palette Coding tool for Single and Dual Trees. For screen content sequences it demonstrates 9.0% / 3.9% / 1.3% (AI / RA / LDB) on class TGM and 4.0% / 3.3% / 2.4% (AI / RA / LDB) on class F of Luma BD-rate saving on top of CE15.2, if Dual Tree is disabled.

# Background

In the VVC 2 draft [1], separate tree approach may be used depend on codec configuration file. Thus, if separate tree in enabled, codec can operate with either Single CU, which includes one Luma CB and two Chroma CBs; or Dual CUs, which assumes separated Luma CU with one Luma CB and Chroma CU with two Chroma CBs. On the other hand if separate tree is disabled, codec always operates with Single CU.

CE15 includes two Palette Coding implementations, both of them are harmonized with separate tree as it shown in Figure 1.

Figure 1 - A diagram of partitioning trees possibilities and Palette Coding processing in CE15.2

As can be seen at Figure 1, depending on SPS Dual Tree flag it may be two branches of Single CU processing. Both current Palette Coding implementations in CE15 perform Palette Coding processing and signalling in a different way for Single CU in Dual Tree on and off cases.

# Proposed method

This contribution proposes a unified method of Palette Coding in VVC codec, if separate tree approach is possible. It is proposed always applying Palette Coding separately for Y component and for joint Cb-Cr regardless of partitioning tree type for both separate tree enabled and disabled cases. Figure 1 illustrates such approach.

Figure 2 - A diagram of partitioning trees possibilities and Palette Coding processing in proposed method

# Simulation results

Simulations were conducted on top of HEVC SCC Palette Coding implementation in VVC SW (CE15.2). To illustrate experimental results for proposed solution Common Test Conditions (CTC) [2] were used with disabled Dual Tree approach. Table 1 demonstrates the results.

Table 1 – Simulation results of proposed method under CTC w/o Dual Tree

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.04% | 0.08% | 114% | 98% |
| Class A2 | 0.12% | 0.04% | 0.09% | 114% | 100% |
| Class B | 0.07% | 0.00% | 0.07% | 114% | 101% |
| Class C | 0.08% | 0.00% | 0.11% | 114% | 105% |
| Class E | 0.11% | 0.01% | 0.17% | 115% | 101% |
| Overall | 0.08% | 0.02% | 0.10% | 114% | 101% |
| Class D | 0.12% | 0.07% | 0.06% | 114% | 106% |
| Class F (optional) | -9.12% | -9.05% | -10.20% | 123% | 90% |
| Class TGM 4:2:0 (optional) | -25.67% | -25.39% | -24.92% | 134% | 65% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.02% | -0.03% | 110% | 101% |
| Class A2 | 0.08% | 0.12% | 0.01% | 112% | 102% |
| Class B | 0.11% | 0.13% | 0.09% | 114% | 102% |
| Class C | 0.16% | -0.05% | -0.02% | 118% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.05% | 0.02% | 114% | 102% |
| Class D | 0.19% | 0.32% | 0.11% | 120% | 106% |
| Class F (optional) | -7.02% | -7.24% | -8.34% | 120% | 99% |
| Class TGM 4:2:0 (optional) | -12.78% | -13.89% | -13.02% | 128% | 90% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | 0.22% | 0.20% | 115% | 104% |
| Class C | 0.16% | 0.19% | 0.06% | 117% | 106% |
| Class E | 0.32% | -1.07% | 0.80% | 116% | 106% |
| Overall | 0.19% | -0.11% | 0.30% | 116% | 105% |
| Class D | 0.24% | 0.32% | 0.40% | 118% | 103% |
| Class F (optional) | -4.72% | -4.62% | -4.88% | 117% | 102% |
| Class TGM 4:2:0 (optional) | -6.59% | -7.48% | -6.66% | 123% | 93% |

In Table 1 an anchor is BMS2.0.1 in VTM configuration with disabled Dual Tree; a test is the anchor plus HEVC SCC Palette Coding implementation (CE15.2) with proposed modification.

For comparison purposes HEVC SCC Palette Coding implementation was also tested under same test conditions. Table 2 demonstrates proposed method on top of CE15.2 with disabled Dual Tree.

Table 2 – Simulation results of proposed method on top of CE15.2 (HEVC SCC Palette Coding)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.07% | 100% | 99% |
| Class A2 | 0.01% | -0.03% | -0.02% | 100% | 100% |
| Class B | 0.01% | 0.04% | 0.10% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.06% | 101% | 100% |
| Class E | 0.00% | -0.02% | 0.01% | 100% | 100% |
| Overall | 0.01% | 0.01% | 0.05% | 100% | 100% |
| Class D | 0.01% | 0.01% | 0.01% | 101% | 101% |
| Class F (optional) | -4.01% | -2.07% | -2.89% | 104% | 93% |
| Class TGM 4:2:0 (optional) | -8.98% | -7.85% | -7.37% | 105% | 78% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | -0.03% | 99% | 100% |
| Class A2 | -0.01% | 0.08% | -0.08% | 99% | 100% |
| Class B | 0.01% | 0.07% | 0.14% | 98% | 100% |
| Class C | 0.00% | 0.03% | -0.06% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.01% | 99% | 100% |
| Class D | -0.02% | 0.08% | -0.13% | 99% | 101% |
| Class F (optional) | -3.31% | -1.56% | -2.30% | 100% | 98% |
| Class TGM 4:2:0 (optional) | -3.93% | -3.73% | -3.05% | 101% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.16% | -0.06% | 99% | 100% |
| Class C | -0.01% | -0.13% | -0.25% | 98% | 101% |
| Class E | 0.07% | -0.69% | 0.92% | 97% | 100% |
| Overall | 0.02% | -0.15% | 0.12% | 98% | 100% |
| Class D | -0.01% | 0.96% | 0.31% | 97% | 98% |
| Class F (optional) | -2.41% | -0.37% | -0.29% | 98% | 99% |
| Class TGM 4:2:0 (optional) | -1.32% | -1.08% | -0.47% | 99% | 96% |

# Conclusion

Proposed modification unifies Palette Coding design and demonstrates 9.0% / 3.9% / 1.3% (AI / RA / LDB) on class TGM and 4.0% / 3.3% / 2.4% (AI / RA / LDB) on class F of Luma BD-rate saving on top of CE15.2, if Dual Tree is disabled.

# Acknowledgment

# The proposed method was cross-checked by S. Bandyopadhyay (InterDigital). Proponents are grateful for the assistance. Cross-check report is available in JVET-L0612 document.

# References

- B. Bross, J. Chen, S. Liu „Versatile Video Coding (Draft 2)“, JVET-K1001-v5, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, Joint Video Experts Team, doc. JVET-K1010, Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).