| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-L0453-v1 |
| --- | --- |

| Title: | Bugfix for restrictions of bi-prediction for small CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Donggyu Sim | Tel: Email: | +82-2-941-6470 yjahn@digitalinsights.co.kr dgsim@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

# Abstract

In this contribution, a bugfix is proposed to restrict bi-directional prediction for small CUs. In VTM and BMS, the restriction of bi-prediction is applied to reduce worst-case memory bandwidth. However, the restriction does not work in current VTM and BMS version. This contribution proposes the bugfix for restrictions of bi-prediction for small CUs and the proposal claims 0.1% BD-rate losses compared to BMS-2.0.1 with VTM configuration.

# Introduction

In order to reduce worst-case memory bandwidth requirements, simply limiting small blocks to use only uni-directional prediction is applied in HEVC. In the same reason, the restriction is also applied in VVC. However, the restriction does not work in current VTM and BMS version. In VTM and BMS software, it would be caused by applying multi-type tree (MTT) structure and subblock-based temporal motion vector prediction (SBTMVP) as shown in Figure 1 and Figure 2.

Figure 1. Conditions of restrictions of bi-directional prediction in VTM and BMS

Figure 2. Bi-prediction restriction for inter prediction direction

# Proposed method

This contribution proposes bugfix to restrict bi-directional prediction for small CUs (4x4, 4x8, and 8x4 CUs). The proposed method is able to also affect SBTMVP for small CUs. It means that subblocks in SBTMVP coded small CUs are predicted by uni-directional. However, it depends on subblock size of SBTMVP, and it is not able to affect when the subblock size is greater-than or equal to 8x8. This contribution also proposes to fix the related descriptions of Draft of VVC [1].

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1, or bi-prediction is used for the current coding unit according to 4. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. Table 7-4 – Name association to inter prediction mode inter_pred_idc; Name of inter_pred_idc. ( cbWidth + cbHeight ) > 12; ( cbWidth + cbHeight ) <= 12. 0; PRED_L0; PRED_L0. 1; PRED_L1; PRED_L1. 2; PRED_BI; n.a.. When inter_pred_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to PRED_L0.

Figure 3. Semantic for inter_pred_idc in Draft of VVC

#### 9.5.3.7 Binarization process for inter_pred_idc Input to this process is a request for a binarization for the syntax element inter_pred_idc. Output of this process is the binarization of the syntax element. The binarization for the syntax element inter_pred_idc is specified in Table 9-8. Table 9-8 – Binarization for inter_pred_idc Value of inter_pred_idc; Name of inter_pred_idc; Bin string. cbWidth + cbHeight > 12; cbWidth + cbHeight <= 12. 0; PRED_L0; 00; 0. 1; PRED_L1; 01; 1. 2; PRED_BI; 1; -.

Figure 4. Description of Binarization process for inter_pred_idc in Draft of VVC

Figure 5. Modified conditions of restrictions of bi-directional prediction in VTM and BMS

Figure 6. Modified bi-prediction restriction for inter prediction direction

# Performance evaluation

The proposed method was evaluated under JVET common test conditions (CTC) [2]. The proposed method was integrated on BMS-2.0.1 with VTM configuration. Table 1 shows BD-rate performance.

Table 1. BD-rate reduction of the test 10.3.3.a

|  | Over BMS-2.0.1 with VTM configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.09% | 0.17% | 0.08% | 98% | 100% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.25% | 0.21% | 0.33% | 100% | 101% |  |  |  |  |  |

# Conclusions

In order to reduce worst-case memory bandwidth, restrictions of bi-prediction for small CUs are applied. To avoid potential confusions from different between VTM/BMS software and draft text of VVC. It is recommended to adopt the proposed change to bugfix restrictions of bi-prediction for small CUs.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 2),” JVET-K1001, 11th JVET meeting, Ljubljana, SI, July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Shuhring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th JVET meeting, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Digital Insights Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).