| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0461-v2 |
| --- | --- |

| Title: | CE5: Counter-based probability estimation and CABAC coding interval subdivision (CE5.1.3 and CE5.2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Paul Haase, Jan Stegemann, Heiner Kirchhoffer, Heiko Schwarz, Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-409 paul.haase@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this contribution two experiments on probability estimation and coding interval subdivision of the arithmetic coding engine are reported. Results for tests CE5.1.3 (with configurations 1 and 2) and CE5.2.3 (with configurations 1 and 2), described in JVET-K1025 , are presented.

# Introduction

Different experiments were conducted on the arithmetic coding engine within CE5. Test CE5.1.3 investigated a counter-based probability estimator with two custom window sizes na and nb. Two configurations were tested [1]. In configuration 1 the window sizes were set to na=4 and nb=7. Configuration 2 applied individual values for the window sizes between 2 and 9 for each context model. A 4-bit value x per context model was used for initialization of na and nb.

na=2+x>>2 & 3

nb=3+ na+x & 3

Consequently, the values for na are between 2 and 5. The values of  nb depend on the value of na and may be between 5 and 9, but at least na+2. Parameter x is chosen, such that  nb does not exceed a value of 9.

In test CE5.2.3 two configurations on the coding interval subdivision using a 2 dimensional lookup table (rangeTabLPS) were conducted [1]. Configuration 1 used a lookup table of size 32x8. A table of size 16x8 was used in configuration 2.

# Summary of results

The test results of CE5.1.3 are as follows:
CE5.1.3 - 1 compared to VTM-2.0.1: -0.89%, -0.52%, -0.38% and -0.42% BD-rate (AI, RA, LB, LP)
CE5.1.3 - 2 compared to VTM-2.0.1: -1.01%, -0.94%, -0.91% and -0.84% BD-rate (AI, RA, LB, LP)

CE5.1.3 - 1 compared to CEM1: 0.03%, -0.01%, 0.13% and 0.12% BD-rate (AI, RA, LB, LP)
CE5.1.3 - 2 compared to CEM1: -0.09%, -0.43%, -0.31% and -0.30% BD-rate (AI, RA, LB, LP)

The test results of CE5.2.3 are as follows:
CE5.2.3 - 1 compared to VTM-2.0.1: -0.86%, -0.46%, -0.58% and -0.51% BD-rate (AI, RA, LB, LP)
CE5.2.3 - 2 compared to VTM-2.0.1: -0.73%, -0.34%, -0.44% and -0.47% BD-rate (AI, RA, LB, LP)

CE5.2.3 - 1 compared to CEM1: 0.05%, 0.05%, 0.03% and 0.03% BD-rate (AI, RA, LB, LP)
CE5.2.3 - 2 compared to CEM1: 0.19%, 0.17%, 0.17% and 0.17% BD-rate (AI, RA, LB, LP)

Configuration CEM1 is defined in [1] (CABACEngine 1 with windows sizes 4 and 7).

# Detailed results

The proposed methods were implemented in BMS-2.0.1. The simulations were conducted according to JVET common test conditions [2] and configurations described in CE5 [1]. All tests were compared to VTM-2.0.1 and CEM1.

## Results of CE5.1.3 – Configuration 1

AI, Reference: VTM 2.0.1-anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.95% | -1.25% | -1.12% | 107% | 103% |
| Class A2 | -0.97% | -1.32% | -1.41% | 106% | 103% |
| Class B | -0.89% | -0.85% | -0.96% | 106% | 104% |
| Class C | -0.87% | -0.72% | -0.69% | 105% | 103% |
| Class E | -0.79% | -0.65% | -0.97% | 106% | 103% |
| Overall | -0.89% | -0.94% | -1.00% | 106% | 103% |
| Class D | -0.73% | -0.19% | -0.57% | 104% | 108% |
| Class F (optional) | -0.74% | -0.73% | -0.66% | 106% | 105% |

RA, Reference: VTM 2.0.1-anchor:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.55% | -1.00% | -1.02% | 101% | 100% |
| Class A2 | -0.59% | -1.09% | -1.11% | 101% | 102% |
| Class B | -0.54% | -1.04% | -0.72% | 102% | 106% |
| Class C | -0.42% | -0.64% | -0.41% | 102% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -0.94% | -0.77% | 102% | 105% |
| Class D | -0.20% | -0.58% | -0.19% | 103% | 117% |
| Class F (optional) | -0.37% | -0.51% | -0.50% | 103% | 106% |

LB, Reference: VTM 2.0.1-anchor:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.52% | -0.80% | 0.14% | 102% | 105% |
| Class C | -0.36% | -0.88% | -0.69% | 102% | 110% |
| Class E | -0.57% | -0.84% | -1.93% | 99% | 100% |
| Overall | -0.48% | -0.84% | -0.65% | 101% | 105% |
| Class D | -0.09% | -0.05% | 0.03% | 102% | 115% |
| Class F (optional) | -0.35% | -0.09% | -0.40% | 101% | 106% |

LP, Reference: VTM 2.0.1-anchor:

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.53% | -0.54% | 0.40% | 103% | 104% |
| Class C | -0.37% | -1.06% | -0.61% | 104% | 110% |
| Class E | -0.29% | -0.30% | -1.05% | 100% | 102% |
| Overall | -0.42% | -0.65% | -0.30% | 103% | 105% |
| Class D | -0.08% | 0.09% | 0.29% | 103% | 115% |
| Class F (optional) | -0.09% | -0.33% | -0.68% | 102% | 106% |

AI, Reference: CEM1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.01% | 0.34% | 106% | 102% |
| Class A2 | -0.34% | 0.03% | -0.10% | 104% | 103% |
| Class B | 0.10% | 0.53% | 0.53% | 104% | 102% |
| Class C | 0.07% | 0.43% | 0.40% | 103% | 99% |
| Class E | 0.06% | 0.41% | 0.24% | 104% | 102% |
| Overall | 0.03% | 0.32% | 0.31% | 104% | 101% |
| Class D | 0.09% | 0.29% | 0.27% | 102% | 103% |
| Class F (optional) | 0.03% | 0.29% | 0.34% | 103% | 101% |

RA, Reference: CEM1:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -0.22% | -0.11% | 109% | 104% |
| Class A2 | -0.21% | -0.56% | -0.38% | 108% | 105% |
| Class B | -0.02% | -0.24% | -0.30% | 109% | 106% |
| Class C | 0.03% | -0.06% | -0.36% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.25% | -0.29% | 108% | 104% |
| Class D | 0.04% | -0.11% | -0.04% | 106% | 105% |
| Class F (optional) | 0.01% | 0.07% | 0.03% | 108% | 105% |

LB, Reference: CEM1:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | -0.72% | -1.32% | 109% | 103% |
| Class C | 0.11% | -0.07% | -0.79% | 107% | 104% |
| Class E | 0.20% | -0.57% | -1.82% | 116% | 107% |
| Overall | 0.13% | -0.47% | -1.27% | 110% | 104% |
| Class D | 0.19% | -0.43% | -0.74% | 106% | 102% |
| Class F (optional) | 0.07% | -0.20% | -0.53% | 112% | 105% |

LP, Reference: CEM1:

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.75% | -1.40% | 107% | 103% |
| Class C | 0.09% | -0.51% | -0.81% | 105% | 104% |
| Class E | 0.28% | 0.09% | -1.54% | 111% | 107% |
| Overall | 0.12% | -0.46% | -1.24% | 107% | 104% |
| Class D | 0.13% | -0.41% | -0.47% | 104% | 101% |
| Class F (optional) | 0.13% | 0.37% | -0.39% | 109% | 108% |

## Results of CE5.1.3 – Configuration 2

AI, Reference: VTM 2.0.1-anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.15% | -1.23% | -1.08% | 109% | 105% |
| Class A2 | -0.97% | -1.39% | -1.26% | 107% | 103% |
| Class B | -1.03% | -1.09% | -1.20% | 107% | 106% |
| Class C | -0.97% | -1.13% | -1.05% | 106% | 105% |
| Class E | -0.91% | -1.15% | -1.45% | 107% | 106% |
| Overall | -1.01% | -1.18% | -1.20% | 107% | 105% |
| Class D | -0.86% | -0.80% | -1.28% | 106% | 107% |
| Class F (optional) | -0.83% | -1.06% | -1.02% | 107% | 108% |

RA, Reference: VTM 2.0.1-anchor:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | -1.07% | -0.97% | 104% | 103% |
| Class A2 | -0.84% | -0.71% | -0.79% | 103% | 103% |
| Class B | -1.00% | -1.22% | -0.72% | 102% | 105% |
| Class C | -0.90% | -1.07% | -0.96% | 103% | 113% |
| Class E |  |  |  |  |  |
| Overall | -0.94% | -1.05% | -0.85% | 103% | 106% |
| Class D | -0.80% | -1.39% | -0.90% | 103% | 117% |
| Class F (optional) | -0.78% | -0.89% | -0.88% | 104% | 110% |

LB, Reference: VTM 2.0.1-anchor:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.95% | -0.76% | 0.48% | 105% | 110% |
| Class C | -0.85% | -1.14% | -0.83% | 103% | 109% |
| Class E | -0.93% | -1.75% | -1.91% | 101% | 102% |
| Overall | -0.91% | -1.13% | -0.56% | 103% | 107% |
| Class D | -0.65% | -0.22% | -0.53% | 104% | 118% |
| Class F (optional) | -0.84% | -0.09% | -0.97% | 103% | 108% |

LP, Reference: VTM 2.0.1-anchor:

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.91% | -0.43% | 0.85% | 106% | 109% |
| Class C | -0.77% | -1.00% | -0.90% | 105% | 112% |
| Class E | -0.81% | -1.59% | -1.19% | 101% | 102% |
| Overall | -0.84% | -0.91% | -0.24% | 104% | 108% |
| Class D | -0.55% | -0.23% | -0.19% | 104% | 119% |
| Class F (optional) | -0.71% | -1.18% | -1.41% | 104% | 106% |

AI, Reference: CEM1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | 0.38% | 107% | 103% |
| Class A2 | -0.33% | -0.03% | 0.07% | 106% | 102% |
| Class B | -0.05% | 0.28% | 0.29% | 105% | 103% |
| Class C | -0.04% | 0.02% | 0.04% | 104% | 100% |
| Class E | -0.07% | -0.09% | -0.25% | 105% | 105% |
| Overall | -0.09% | 0.07% | 0.12% | 105% | 103% |
| Class D | -0.04% | -0.33% | -0.44% | 103% | 103% |
| Class F (optional) | -0.06% | -0.04% | -0.02% | 104% | 104% |

RA, Reference: CEM1:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.30% | -0.06% | 111% | 106% |
| Class A2 | -0.46% | -0.18% | -0.05% | 110% | 107% |
| Class B | -0.47% | -0.42% | -0.31% | 109% | 104% |
| Class C | -0.45% | -0.50% | -0.91% | 107% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.43% | -0.37% | -0.37% | 109% | 106% |
| Class D | -0.56% | -0.92% | -0.75% | 107% | 106% |
| Class F (optional) | -0.41% | -0.31% | -0.36% | 110% | 109% |

LB, Reference: CEM1:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.68% | -1.02% | 112% | 108% |
| Class C | -0.38% | -0.33% | -0.93% | 108% | 103% |
| Class E | -0.16% | -1.48% | -1.80% | 118% | 108% |
| Overall | -0.31% | -0.77% | -1.19% | 112% | 107% |
| Class D | -0.37% | -0.60% | -1.35% | 108% | 104% |
| Class F (optional) | -0.42% | -0.19% | -1.08% | 115% | 107% |

LP, Reference: CEM1:

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | -0.64% | -0.98% | 109% | 108% |
| Class C | -0.31% | -0.45% | -1.10% | 107% | 105% |
| Class E | -0.24% | -1.20% | -1.68% | 113% | 107% |
| Overall | -0.30% | -0.72% | -1.19% | 109% | 107% |
| Class D | -0.34% | -0.72% | -0.99% | 106% | 105% |
| Class F (optional) | -0.49% | -0.48% | -1.13% | 111% | 109% |

## Results of CE5.2.3 – Configuration 1

AI, Reference: VTM 2.0.1-anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.06% | -1.19% | -1.38% | 107% | 104% |
| Class A2 | -0.60% | -1.31% | -1.27% | 105% | 103% |
| Class B | -0.93% | -1.32% | -1.42% | 105% | 104% |
| Class C | -0.89% | -1.10% | -1.04% | 105% | 103% |
| Class E | -0.80% | -1.01% | -1.16% | 105% | 103% |
| Overall | -0.86% | -1.20% | -1.26% | 105% | 103% |
| Class D | -0.78% | -0.43% | -0.79% | 104% | 106% |
| Class F (optional) | -0.73% | -0.98% | -0.96% | 105% | 107% |

RA, Reference: VTM 2.0.1-anchor:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.62% | -0.73% | -0.85% | 101% | 100% |
| Class A2 | -0.33% | -0.48% | -0.69% | 101% | 102% |
| Class B | -0.48% | -0.75% | -0.37% | 101% | 106% |
| Class C | -0.41% | -0.53% | -0.01% | 102% | 112% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | -0.63% | -0.43% | 101% | 105% |
| Class D | -0.20% | -0.43% | -0.10% | 102% | 115% |
| Class F (optional) | -0.33% | -0.53% | -0.48% | 103% | 109% |

LB, Reference: VTM 2.0.1-anchor:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.58% | -0.03% | 1.61% | 103% | 106% |
| Class C | -0.44% | -0.77% | 0.13% | 102% | 106% |
| Class E | -0.75% | -0.23% | -0.11% | 99% | 98% |
| Overall | -0.58% | -0.33% | 0.69% | 102% | 104% |
| Class D | -0.26% | 0.41% | 0.82% | 102% | 116% |
| Class F (optional) | -0.37% | 0.18% | 0.19% | 101% | 106% |

LP, Reference: VTM 2.0.1-anchor:

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | 0.27% | 1.94% | 104% | 106% |
| Class C | -0.43% | -0.51% | 0.24% | 103% | 108% |
| Class E | -0.55% | -0.35% | 0.49% | 100% | 103% |
| Overall | -0.51% | -0.14% | 1.01% | 103% | 106% |
| Class D | -0.19% | 0.52% | 0.82% | 103% | 116% |
| Class F (optional) | -0.17% | -0.65% | -0.23% | 103% | 106% |

AI, Reference: CEM1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.07% | 0.07% | 105% | 102% |
| Class A2 | 0.06% | 0.06% | 0.06% | 104% | 103% |
| Class B | 0.06% | 0.06% | 0.06% | 103% | 102% |
| Class C | 0.05% | 0.05% | 0.05% | 103% | 99% |
| Class E | 0.04% | 0.05% | 0.05% | 104% | 102% |
| Overall | 0.05% | 0.05% | 0.05% | 104% | 101% |
| Class D | 0.04% | 0.04% | 0.04% | 102% | 101% |
| Class F (optional) | 0.04% | 0.04% | 0.04% | 102% | 103% |

RA, Reference: CEM1:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.05% | 0.05% | 108% | 104% |
| Class A2 | 0.05% | 0.05% | 0.05% | 107% | 105% |
| Class B | 0.05% | 0.05% | 0.05% | 108% | 105% |
| Class C | 0.05% | 0.05% | 0.05% | 106% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.05% | 0.05% | 107% | 105% |
| Class D | 0.04% | 0.04% | 0.04% | 105% | 104% |
| Class F (optional) | 0.04% | 0.04% | 0.04% | 108% | 108% |

LB, Reference: CEM1:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.04% | 0.04% | 110% | 105% |
| Class C | 0.03% | 0.03% | 0.03% | 107% | 100% |
| Class E | 0.02% | 0.02% | 0.02% | 116% | 104% |
| Overall | 0.03% | 0.03% | 0.03% | 110% | 103% |
| Class D | 0.02% | 0.02% | 0.02% | 106% | 102% |
| Class F (optional) | 0.05% | 0.05% | 0.05% | 113% | 106% |

LP, Reference: CEM1:

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.04% | 0.04% | 107% | 104% |
| Class C | 0.03% | 0.03% | 0.03% | 105% | 102% |
| Class E | 0.02% | 0.02% | 0.02% | 112% | 108% |
| Overall | 0.03% | 0.03% | 0.03% | 108% | 104% |
| Class D | 0.02% | 0.02% | 0.02% | 104% | 102% |
| Class F (optional) | 0.05% | 0.05% | 0.05% | 110% | 109% |

## Results of CE5.2.3 – Configuration 2

AI, Reference: VTM 2.0.1-anchor:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.88% | -1.02% | -1.21% | 106% | 104% |
| Class A2 | -0.46% | -1.17% | -1.13% | 105% | 103% |
| Class B | -0.78% | -1.17% | -1.28% | 105% | 104% |
| Class C | -0.77% | -0.98% | -0.92% | 105% | 104% |
| Class E | -0.71% | -0.92% | -1.07% | 105% | 104% |
| Overall | -0.73% | -1.06% | -1.13% | 105% | 104% |
| Class D | -0.68% | -0.34% | -0.70% | 104% | 109% |
| Class F (optional) | -0.65% | -0.90% | -0.88% | 105% | 105% |

RA, Reference: VTM 2.0.1-anchor:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -0.58% | -0.71% | 101% | 100% |
| Class A2 | -0.20% | -0.35% | -0.56% | 101% | 101% |
| Class B | -0.35% | -0.62% | -0.24% | 101% | 105% |
| Class C | -0.31% | -0.43% | 0.09% | 102% | 111% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.51% | -0.31% | 101% | 105% |
| Class D | -0.11% | -0.34% | -0.01% | 102% | 119% |
| Class F (optional) | -0.26% | -0.46% | -0.41% | 103% | 107% |

LB, Reference: VTM 2.0.1-anchor:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | 0.15% | 1.79% | 103% | 105% |
| Class C | -0.32% | -0.65% | 0.26% | 102% | 107% |
| Class E | -0.64% | -0.11% | 0.01% | 99% | 99% |
| Overall | -0.44% | -0.18% | 0.83% | 102% | 104% |
| Class D | -0.15% | 0.53% | 0.94% | 102% | 116% |
| Class F (optional) | -0.28% | 0.26% | 0.27% | 100% | 104% |

LP, Reference: VTM 2.0.1-anchor:

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | 0.45% | 2.12% | 104% | 105% |
| Class C | -0.31% | -0.39% | 0.36% | 103% | 109% |
| Class E | -0.44% | -0.22% | 0.61% | 100% | 101% |
| Overall | -0.37% | 0.00% | 1.16% | 102% | 106% |
| Class D | -0.08% | 0.63% | 0.94% | 103% | 116% |
| Class F (optional) | -0.08% | -0.56% | -0.15% | 102% | 103% |

AI, Reference: CEM1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.25% | 0.24% | 0.24% | 105% | 102% |
| Class A2 | 0.20% | 0.20% | 0.20% | 104% | 102% |
| Class B | 0.20% | 0.20% | 0.20% | 103% | 101% |
| Class C | 0.17% | 0.17% | 0.17% | 103% | 100% |
| Class E | 0.14% | 0.14% | 0.14% | 104% | 103% |
| Overall | 0.19% | 0.19% | 0.19% | 104% | 102% |
| Class D | 0.14% | 0.14% | 0.14% | 102% | 104% |
| Class F (optional) | 0.12% | 0.12% | 0.12% | 102% | 101% |

RA, Reference: CEM1:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.20% | 0.20% | 109% | 104% |
| Class A2 | 0.18% | 0.18% | 0.18% | 107% | 104% |
| Class B | 0.18% | 0.18% | 0.18% | 108% | 105% |
| Class C | 0.14% | 0.14% | 0.14% | 106% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | 0.17% | 0.17% | 107% | 104% |
| Class D | 0.14% | 0.14% | 0.14% | 106% | 108% |
| Class F (optional) | 0.11% | 0.11% | 0.11% | 108% | 106% |

LB, Reference: CEM1:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.21% | 0.21% | 0.21% | 110% | 104% |
| Class C | 0.15% | 0.15% | 0.15% | 107% | 101% |
| Class E | 0.13% | 0.14% | 0.14% | 116% | 106% |
| Overall | 0.17% | 0.17% | 0.17% | 111% | 103% |
| Class D | 0.13% | 0.13% | 0.13% | 107% | 102% |
| Class F (optional) | 0.14% | 0.14% | 0.14% | 111% | 103% |

LP, Reference: CEM1:

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.21% | 0.21% | 0.21% | 107% | 104% |
| Class C | 0.15% | 0.15% | 0.15% | 105% | 103% |
| Class E | 0.14% | 0.15% | 0.15% | 111% | 106% |
| Overall | 0.17% | 0.18% | 0.18% | 107% | 104% |
| Class D | 0.13% | 0.13% | 0.13% | 104% | 102% |
| Class F (optional) | 0.14% | 0.14% | 0.14% | 109% | 106% |

# References

- H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine”, JVET-K1025, 11-th Meeting, Ljubljana, SI, July 2018

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).