| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0462 |
| --- | --- |

| Title: | CE5: State-based probability estimation (CE5.1.4, CE5.1.5) and coding interval subdivision (CE5.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Christian Bartnik, Paul Haase, Stefan Matlage, Jan Stegemann, Detlev Marpe, Heiko Schwarz, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin | Tel: Email: | +49 30 31002-184 heiner.kirchhoffer@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution describes the combination of the state-based probability estimator of JVET-K0430 with the concept of custom window sizes and presents experimental results for state-based probability estimator of tests CE5.1.4, CE5.1.5, and CE5.3.1.

# Combination of the state-based probability estimator with custom window sizes

In order to combine the probability estimator of JVET-K0430 with custom window sizes, the probability update equations need to modified. Similar to the method proposed in JVET-K430, two signed integer state variables S0 and S1 are stored per context model which require 8 and 12 bit, respectively. However, the state variable update after encoding or decoding a bin is slightly altered according to the following pseudo code:

if( bin )

{

S0 +=  Tab[ 16 + (S0 >> 3) ] >> shiftS0;

S1 += (Tab[ 16 + (S1 >> 7) ] << 4) >> shiftS1;

}

else

{

S0 -= Tab[ 16 + (-S0 >> 3) ] >> shiftS0;

S1 -= (Tab[ 16 + (-S1 >> 7) ] << 4) >> shiftS1;

}

where lookup table ‘Tab’ may for example be defined as:

Tab[32] = { 157, 143, 129, 115, 101,  87,  73,  59,

45,  35,  29,  23,  17,  13,   9,   5,

4,   4,   4,   4,   4,   4,   4,   4,

4,   4,   4,   4,   4,   4,   4,   0 }

The two new parameters shiftS0 and shiftS1 are defined as variables to be stored per context model in order to improve coding efficiency by supplying different state variable update agilities. Note that 0≤shiftS0≤3 and shiftS0≤shiftS1≤7. This corresponds to the custom window sizes of the counter-based probability estimator as e.g. tested in CE5.1.3.

Note that the implementation in the CE software is different in that the state variables S0 and S1 are both defined as 16 bit signed integers. The precision of 8 or 12 bit, respectively, is realized by interpreting S0 and S1 as left aligned 16 bit integer variables and applying respective bit shifting / clearing of the LSBs.

# Test results

Only the VTM configuration was tested based on the BMS-2.0.1 software as specified in the CE5 description (JVET-K1025). All results are reported over VTM-2.0.1 and over the CEM1 software according to the CE5 description.

Note that runtimes are not accurate due to the simulation conditions (runtimes vary depending on the cluster load).

## Test CE5.1.4, git commit id 021424ed: Custom window sizes

In this test, parameters shiftS0 and shiftS1 are set individually for each context model.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,18% | -1,28% | -1,63% | 111% | 100% |
| Class A2 | -0,67% | -1,52% | -1,31% | 109% | 102% |
| Class B | -1,03% | -1,80% | -1,77% | 110% | 100% |
| Class C | -0,99% | -1,71% | -1,63% | 108% | 102% |
| Class E | -0,89% | -1,55% | -1,64% | 109% | 101% |
| Overall | -0,96% | -1,61% | -1,62% | 109% | 101% |
| Class D | -0,88% | -1,18% | -1,63% | 107% | 104% |
| Class F (optional) | -0,79% | -1,26% | -1,24% | 109% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,95% | -0,77% | -1,30% | 105% | 105% |
| Class A2 | -0,64% | -0,40% | -0,72% | 106% | 105% |
| Class B | -0,92% | -1,12% | -0,80% | 107% | 105% |
| Class C | -0,87% | -1,29% | -1,05% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0,86% | -0,95% | -0,95% | 106% | 104% |
| Class D | -0,82% | -1,41% | -1,16% | 106% | 106% |
| Class F (optional) | -0,67% | -0,79% | -0,88% | 107% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,92% | -0,25% | 0,03% | 107% | 103% |
| Class C | -0,85% | -1,01% | -1,03% | 105% | 96% |
| Class E | -1,18% | -0,83% | -1,60% | 107% | 99% |
| Overall | -0,96% | -0,65% | -0,73% | 106% | 99% |
| Class D | -0,75% | -0,11% | -1,22% | 106% | 101% |
| Class F (optional) | -0,68% | -0,53% | -1,03% | 106% | 101% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,05% | -0,03% | -0,19% | 111% | 101% |
| Class A2 | -0,01% | -0,16% | 0,01% | 109% | 103% |
| Class B | -0,05% | -0,43% | -0,29% | 109% | 102% |
| Class C | -0,06% | -0,56% | -0,55% | 108% | 100% |
| Class E | -0,05% | -0,50% | -0,44% | 109% | 102% |
| Overall | -0,05% | -0,36% | -0,30% | 109% | 102% |
| Class D | -0,06% | -0,71% | -0,80% | 106% | 104% |
| Class F (optional) | -0,02% | -0,24% | -0,24% | 107% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,28% | 0,01% | -0,40% | 115% | 111% |
| Class A2 | -0,25% | 0,14% | 0,02% | 114% | 111% |
| Class B | -0,39% | -0,32% | -0,38% | 114% | 109% |
| Class C | -0,42% | -0,71% | -0,99% | 112% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0,35% | -0,27% | -0,47% | 114% | 109% |
| Class D | -0,58% | -0,95% | -1,01% | 112% | 109% |
| Class F (optional) | -0,30% | -0,21% | -0,35% | 114% | 110% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,31% | -0,18% | -1,46% | 116% | 104% |
| Class C | -0,37% | -0,21% | -1,12% | 112% | 97% |
| Class E | -0,42% | -0,54% | -1,47% | 123% | 105% |
| Overall | -0,36% | -0,28% | -1,35% | 116% | 102% |
| Class D | -0,47% | -0,50% | -2,01% | 112% | 103% |
| Class F (optional) | -0,26% | -0,65% | -1,15% | 117% | 104% |

### Test CE5.1.4, git commit id ac6cc028: Fixed window sizes

In this test, shiftS0 = 1 and shiftS1 = 4 for all context models.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,09% | -1,45% | -1,39% | 102% | 97% |
| Class A2 | -0,72% | -1,47% | -1,43% | 102% | 100% |
| Class B | -0,94% | -1,40% | -1,41% | 102% | 99% |
| Class C | -0,91% | -1,23% | -1,20% | 102% | 101% |
| Class E | -0,81% | -0,96% | -1,22% | 101% | 100% |
| Overall | -0,90% | -1,31% | -1,33% | 102% | 99% |
| Class D | -0,78% | -0,70% | -0,97% | 102% | 102% |
| Class F (optional) | -0,73% | -1,00% | -0,88% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,75% | -0,82% | -1,06% | 92% | 94% |
| Class A2 | -0,54% | -0,51% | -0,88% | 93% | 95% |
| Class B | -0,60% | -0,92% | -0,49% | 94% | 97% |
| Class C | -0,46% | -0,94% | -0,14% | 95% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0,58% | -0,82% | -0,59% | 94% | 96% |
| Class D | -0,27% | -0,72% | 0,10% | 96% | 98% |
| Class F (optional) | -0,41% | -0,69% | -0,51% | 94% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,51% | -0,63% | 0,14% | 95% | 97% |
| Class E | -0,76% | 0,10% | -0,17% | 88% | 95% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,24% | 0,57% | 0,72% | 96% | 100% |
| Class F (optional) | -0,41% | 0,03% | 0,64% | 91% | 95% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | -0,19% | 0,06% | 102% | 98% |
| Class A2 | -0,07% | -0,11% | -0,11% | 101% | 100% |
| Class B | 0,04% | -0,03% | 0,08% | 101% | 101% |
| Class C | 0,02% | -0,08% | -0,12% | 102% | 99% |
| Class E | 0,03% | 0,10% | -0,02% | 101% | 102% |
| Overall | 0,02% | -0,06% | -0,02% | 101% | 100% |
| Class D | 0,03% | -0,23% | -0,14% | 101% | 102% |
| Class F (optional) | 0,05% | 0,02% | 0,12% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,07% | -0,05% | -0,15% | 101% | 99% |
| Class A2 | -0,15% | 0,02% | -0,14% | 100% | 100% |
| Class B | -0,07% | -0,13% | -0,08% | 101% | 101% |
| Class C | -0,01% | -0,36% | -0,08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,07% | -0,14% | -0,11% | 101% | 100% |
| Class D | -0,03% | -0,26% | 0,25% | 101% | 101% |
| Class F (optional) | -0,04% | -0,11% | 0,01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,04% | 0,17% | 0,04% | 101% | 98% |
| Class E | 0,01% | 0,36% | -0,04% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0,04% | 0,18% | -0,07% | 101% | 101% |
| Class F (optional) | 0,01% | -0,07% | 0,49% | 101% | 99% |

### Test CE5.1.5, git commit id cb97d1f5: One state, custom window size

In this test, S0 uses a 12 bit signed integer and shiftS0 is set individually for each context model.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,96% | -1,32% | -1,36% | 109% | 100% |
| Class A2 | -0,79% | -1,51% | -1,41% | 108% | 100% |
| Class B | -0,83% | -1,36% | -1,22% | 107% | 100% |
| Class C | -0,75% | -1,38% | -1,20% | 106% | 101% |
| Class E | -0,71% | -1,37% | -1,25% | 107% | 101% |
| Overall | -0,81% | -1,38% | -1,28% | 107% | 101% |
| Class D | -0,68% | -0,99% | -1,25% | 105% | 104% |
| Class F (optional) | -0,57% | -0,96% | -0,91% | 106% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,65% | -1,06% | -1,34% | 105% | 103% |
| Class A2 | -0,55% | -0,82% | -1,17% | 106% | 103% |
| Class B | -0,74% | -1,47% | -1,06% | 106% | 102% |
| Class C | -0,65% | -1,37% | -0,98% | 105% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0,66% | -1,23% | -1,12% | 106% | 103% |
| Class D | -0,65% | -1,49% | -0,89% | 106% | 104% |
| Class F (optional) | -0,38% | -0,93% | -0,79% | 105% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,70% | -0,88% | -0,51% | 107% | 105% |
| Class C | -0,59% | -1,50% | -1,21% | 105% | 101% |
| Class E | -0,83% | -1,33% | -1,55% | 107% | 102% |
| Overall | -0,70% | -1,20% | -1,00% | 106% | 103% |
| Class D | -0,57% | -0,27% | -1,08% | 106% | 108% |
| Class F (optional) | -0,44% | -0,21% | -0,62% | 106% | 102% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,17% | -0,06% | 0,09% | 109% | 101% |
| Class A2 | -0,14% | -0,16% | -0,09% | 107% | 101% |
| Class B | 0,15% | 0,01% | 0,27% | 106% | 102% |
| Class C | 0,19% | -0,23% | -0,11% | 105% | 100% |
| Class E | 0,13% | -0,31% | -0,05% | 107% | 103% |
| Overall | 0,11% | -0,13% | 0,04% | 107% | 101% |
| Class D | 0,14% | -0,51% | -0,41% | 104% | 104% |
| Class F (optional) | 0,21% | 0,06% | 0,09% | 104% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,03% | -0,28% | -0,43% | 114% | 109% |
| Class A2 | -0,17% | -0,28% | -0,43% | 114% | 109% |
| Class B | -0,21% | -0,67% | -0,65% | 114% | 107% |
| Class C | -0,19% | -0,79% | -0,92% | 111% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0,15% | -0,55% | -0,64% | 113% | 108% |
| Class D | -0,41% | -1,02% | -0,74% | 111% | 108% |
| Class F (optional) | -0,01% | -0,35% | -0,27% | 113% | 109% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,09% | -0,81% | -1,97% | 116% | 107% |
| Class C | -0,12% | -0,69% | -1,30% | 112% | 102% |
| Class E | -0,06% | -1,04% | -1,45% | 123% | 108% |
| Overall | -0,09% | -0,83% | -1,62% | 116% | 106% |
| Class D | -0,29% | -0,65% | -1,90% | 112% | 110% |
| Class F (optional) | -0,02% | -0,31% | -0,76% | 117% | 106% |

### Test CE5.3.1, git commit id 5abc998b: rangeTabLPS 32x8

In this test, the same probability estimator as in CE5.1.4 with custom window sizes (git commit id 021424ed) is used, but with a rangeTabLPS of size 32x8. Results are presented over CE5.1.4 where a range subdivision table of size 512x64 is used. Results over VTM-2.0.1 and over CEM1 can be found in the accompanying Excel sheets.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5.1.4_021424e |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,00% | 0,00% | 100% | 101% |
| Class A2 | 0,00% | 0,00% | 0,00% | 100% | 101% |
| Class B | 0,00% | 0,00% | 0,00% | 100% | 101% |
| Class C | 0,02% | 0,02% | 0,02% | 100% | 101% |
| Class E | 0,02% | 0,02% | 0,02% | 100% | 101% |
| Overall | 0,01% | 0,01% | 0,01% | 100% | 101% |
| Class D | 0,03% | 0,03% | 0,03% | 100% | 103% |
| Class F (optional) | 0,03% | 0,03% | 0,03% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE5.1.4_021424e |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | 0,00% | 0,01% | 99% | 100% |
| Class A2 | 0,01% | 0,01% | 0,01% | 99% | 98% |
| Class B | 0,01% | 0,01% | 0,01% | 100% | 100% |
| Class C | 0,02% | 0,02% | 0,02% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0,01% | 0,01% | 0,01% | 99% | 100% |
| Class D | 0,03% | 0,03% | 0,03% | 100% | 102% |
| Class F (optional) | 0,03% | 0,03% | 0,03% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5.1.4_021424e |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | -0,01% | -0,01% | 98% | 97% |
| Class C | 0,02% | 0,02% | 0,02% | 100% | 103% |
| Class E | 0,08% | 0,07% | 0,07% | 100% | 100% |
| Overall | 0,02% | 0,02% | 0,02% | 99% | 100% |
| Class D | 0,03% | 0,03% | 0,03% | 100% | 103% |
| Class F (optional) | 0,03% | 0,03% | 0,03% | 100% | 100% |

### Test CE5.3.1, git commit id 9f8ad1fd: rangeTabLPS 16x8

In this test, the same probability estimator as in CE5.1.4 with custom window sizes (git commit id 021424ed) is used, but with a rangeTabLPS of size 16x8. Results are presented over CE5.1.4 where a range subdivision table of size 512x64 is used. Results over VTM-2.0.1 and over CEM1 can be found in the accompanying Excel sheets.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5.1.4_021424e |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,09% | 0,10% | 0,10% | 100% | 101% |
| Class A2 | 0,09% | 0,10% | 0,10% | 100% | 100% |
| Class B | 0,11% | 0,10% | 0,11% | 100% | 100% |
| Class C | 0,13% | 0,13% | 0,13% | 100% | 100% |
| Class E | 0,13% | 0,13% | 0,13% | 102% | 99% |
| Overall | 0,11% | 0,11% | 0,11% | 100% | 100% |
| Class D | 0,14% | 0,14% | 0,14% | 101% | 99% |
| Class F (optional) | 0,14% | 0,14% | 0,14% | 101% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE5.1.4_021424e |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,12% | 0,12% | 0,12% | 100% | 99% |
| Class A2 | 0,13% | 0,13% | 0,13% | 100% | 99% |
| Class B | 0,12% | 0,12% | 0,12% | 100% | 99% |
| Class C | 0,14% | 0,14% | 0,14% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0,13% | 0,13% | 0,13% | 100% | 99% |
| Class D | 0,15% | 0,15% | 0,15% | 101% | 98% |
| Class F (optional) | 0,15% | 0,14% | 0,14% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5.1.4_021424e |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,11% | 0,11% | 0,11% | 99% | 100% |
| Class C | 0,14% | 0,14% | 0,14% | 101% | 104% |
| Class E | 0,21% | 0,20% | 0,20% | 100% | 102% |
| Overall | 0,15% | 0,14% | 0,14% | 100% | 102% |
| Class D | 0,16% | 0,16% | 0,16% | 100% | 104% |
| Class F (optional) | 0,16% | 0,16% | 0,16% | 100% | 102% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).