| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0475-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-L0281 (CE4-related: Size constrain for inherited affine motion prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Huanbang Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-L0281 (CE4-related: Size constrain for inherited affine motion prediction). The simulations were carried out based on the common test condition (CTC). The test results are matched for all points and the software matches the description in the proposal.

Introduction

This contribution JVET-L0281 proposes to disable inherited affine motion prediction from a neighboring affine coded block if the width or height of the neighboring block is less than 16. So, if the width or height of a neighboring affine coded block is less than 16, the inherited affine motion prediction from the block is treated as unavailable.

Simulation results

Table 1. Summary results of the proposed algorithm

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | -0.01% | 100% | 102% |
| Class A2 | 0.05% | 0.04% | -0.03% | 100% | 101% |
| Class B | 0.02% | -0.01% | -0.05% | 99% | 102% |
| Class C | 0.04% | 0.14% | 0.05% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.07% | -0.01% | 100% | 101% |
| Class D | 0.05% | 0.10% | -0.09% | 99% | 99% |
| Class F (optional) | 0.01% | 0.05% | 0.05% | 99% | 100% |
|  | Low Delay B Main 10 |  |  |  |  |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.16% | -0.09% | 100% | 103% |
| Class C | 0.02% | 0.11% | -0.09% | 99% | 104% |
| Class E | -0.08% | 0.39% | -0.16% | 100% | 102% |
| Overall | -0.01% | 0.07% | -0.10% | 100% | 103% |
| Class D | 0.04% | 0.57% | -0.43% | 99% | 101% |
| Class F (optional) | -0.06% | 0.51% | 0.73% | 99% | 101% |

References

- H. Huang, W.-J. Chien, M. Karczewicz, “CE4-related: Size constrain for inherited affine motion prediction”, JVET-L0281, Oct. 2018, Macao, CN.

Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.