| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0491-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-L0207 (CE4 related: simplified non-sub-block STMVP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang 8910 University Center Ln, San Diego, CA, U.S.A. 92122 | Tel: Email: | +1 8589997093 lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results of JVET-L0207 [1] which proposes to add non-sub-block spatial temporal motion vector prediction (STMVP). The results of the proposal on top of VTM2.0.1 match with that provided in JVET-L0207.

# Experimental results

JVET-L0207 [1] was implemented on top of VTM2.0.1 and also on top of CE4.4.7 reference software as proposed in JVET-L0266 [2]. Two sets of simulations have been performed following the common test conditions [4]. Simulations were performed with the VTM configurations:

Test #1 (Table 1 in JVET-L0207):

- Anchor: VTM2.0.1 with merge list size equal to 6.

- Tested: JVET-L0207.

Test #2 (additional test):

- Anchor: CE4.4.7.a with merge list size equal to 6 [3].

- Tested: JVET-L0207 + anchor.

The results of Test #1 are shown in Table 1.

Table 1: Coding performance of JVET-L0207 on top of CE4.4.7.c

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.57% | -0.35% | -0.52% | 101% | 113% |  |  |  |  |  |
| Class A2 | -0.51% | -0.49% | -0.52% | 101% | 112% |  |  |  |  |  |
| Class B | -0.39% | -0.30% | -0.34% | 102% | 118% | -0.03% | 0.25% | 0.27% | 100% | #NUM! |
| Class C | -0.36% | -0.23% | -0.38% | 106% | 130% | -0.01% | 0.27% | 0.15% | 100% | 116% |
| Class E |  |  |  |  |  | -0.29% | -0.35% | -0.42% | 101% | 138% |
| Overall | -0.44% | -0.33% | -0.42% | 103% | 119% | -0.09% | 0.11% | 0.06% | 100% | #NUM! |

The results of Test #2 are shown in Table 2.

Table 2: Coding performance of JVET-L0207 on top of CE4.4.7.a

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.64% | -0.66% | -1.09% | 101% | 112% |  |  |  |  |  |
| Class A2 | -0.87% | -1.38% | -1.45% | 100% | 111% |  |  |  |  |  |
| Class B | -1.16% | -1.19% | -1.25% | 100% | 116% | -0.88% | -1.01% | -0.94% | 101% | #NUM! |
| Class C | -0.75% | -0.79% | -0.89% | 102% | 127% | -0.46% | -0.67% | -0.66% | 102% | 115% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.89% | -1.02% | -1.16% | 101% | 117% |  |  |  |  |  |

# References

- F. Le Léannec, T. Poirier, F. Galpin, JVET-L0207, “CE4 related: simplified non-sub-block STMVP”, Oct. 2018.

- L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, “CE4: History-based Motion Vector Prediction (Test 4.4.7)”, Oct. 2018.

- https://vcgit.hhi.fraunhofer.de/JVET-K-CE4/VVCSoftware_VTM/tree/CE4.4.7

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, July. 2018.