| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0498-v1 |
| --- | --- |

| Title: | Crosscheck for JVET-L0053 (CE3-related: Chroma DM modification) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ling Li Jin Heo Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 l.li@lge.com jin78.heo@lge.com jangwon84.choi@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This document reports the crosscheck result for JVET-L0053: CE3-related: Chroma DM modification.

Introduction

The algorithm described in JVET-L0053 proposes to change chroma direct mode (DM) from top-left to central area. In addition, it also proposes to use only one DM instead of multiple DM (MDMS).

Experimental Results

The algorithm is evaluated on BMS 2.0.1 [1] under VTM configuration according to the common test conditions [2]. Both all intra and random access are tested. The summarized results are shown in Tables 1 and 2. The detailed results can be found in Excel files in the package.

One minor bit mismatch is found in sequence Tango with QP 22 of TEST 1 random access case. It is on the fourth digit after decimal point.

Table 1 TEST1: crosscheck result for changing DM from top-left to central area using BMS 2.0.1 with VTM config as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.00% | -0.06% | 100% | 99% |
| Class A2 | -0.10% | 0.06% | -0.03% | 101% | 100% |
| Class B | -0.07% | 0.07% | 0.11% | 100% | 100% |
| Class C | -0.05% | -0.01% | 0.24% | 100% | 101% |
| Class E | -0.24% | 0.06% | -0.25% | 101% | 101% |
| Overall | -0.10% | 0.04% | 0.03% | 100% | 100% |
| Class D | -0.06% | 0.15% | -0.10% | 99% | 96% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.02% | -0.01% | 101% | 101% |
| Class A2 | -0.02% | -0.03% | -0.13% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.09% | 100% | 101% |
| Class C | 0.01% | 0.08% | 0.15% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.02% | 100% | 101% |
| Class D | 0.00% | -0.22% | 0.43% | 99% | 94% |

Table 2 TEST2: crosscheck result for using on central DM instead of MDMS using BMS 2.0.1 with VTM config as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.42% | -0.52% | 100% | 100% |
| Class A2 | -0.19% | -0.72% | -0.75% | 101% | 100% |
| Class B | -0.12% | -0.85% | -0.91% | 100% | 100% |
| Class C | -0.14% | -0.70% | -0.46% | 100% | 101% |
| Class E | -0.32% | -1.53% | -1.51% | 100% | 101% |
| Overall | -0.17% | -0.84% | -0.82% | 100% | 100% |
| Class D | -0.13% | -0.39% | -0.73% | 100% | 97% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.27% | -0.34% | 101% | 102% |
| Class A2 | -0.04% | -0.59% | -0.48% | 100% | 100% |
| Class B | 0.03% | -0.98% | -1.17% | 100% | 101% |
| Class C | 0.04% | -0.65% | -0.83% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.67% | -0.78% | 100% | 101% |
| Class D | 0.02% | -0.76% | -0.20% | 100% | 94% |

Summary

The crosscheck results match in general besides one minor mismatch.

References

- BMS-2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.