| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0517 |
| --- | --- |

| Title: | Crosscheck of JVET-L0319:CE4-related: Sub-block MV clipping in planar motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang | Email: | zhangna25@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report for CE4 related technology: Sub-block MV clipping in planar motion vector prediction, which is proposed in JVET-L0319. It is confirmed the available VTM results match with the data provided by the proponents.

Introduction

The software package provided by the proponents was reviewed and confirmed that 1) the VTM-2.0.1 software was used as the basic software to implement the proposal in this contribution; 2) the proposed changes were related to sub-block MV clipping in planar motion vector prediction to reduce the memory bandwidth; 3) two methods are proposed. The software was evaluated under the following testing environment,

- Testing conditions follow the specification of CTC [1] and CE4 [2]

- RA and LB were tested for VTM configuration.

- The source code was compiled by using visual studio 2015

The available results are shown in the following tables, and detailed data can be found in the attached spreadsheet. It should be noted that the encoding/decoding runtime may be not accurate due to the workload imbalance.

Experimental results

Table. 1. The BD bit-rate of the first method.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.84% | -0.58% | -0.81% | 102% | 119% |
| Class A2 | -0.84% | -0.82% | -1.06% | 104% | 115% |
| Class B | -0.49% | -0.43% | -0.58% | 104% | 111% |
| Class C | -0.42% | -0.43% | -0.53% | 105% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.61% | -0.54% | -0.71% | 104% | 113% |
| Class D | -0.63% | -0.73% | -0.69% | 105% | 116% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.24% | -0.14% | 105% | 106% |
| Class C | -0.24% | -0.30% | -0.39% | 105% | 98% |
| Class E | -1.09% | -1.01% | -0.77% | 105% | 103% |
| Overall | -0.46% | -0.45% | -0.38% | 105% | 102% |
| Class D | -0.27% | 0.41% | -0.46% | 106% | 106% |

Table. 2. The BD bit-rate of the second method.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.49% | -0.49% | -0.56% | 104% | 111% |
| Class C | -0.43% | -0.35% | -0.49% | 105% | 110% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.63% | -0.73% | -0.68% | 105% | 114% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.13% | -0.12% | 106% | 111% |
| Class C | -0.19% | -0.20% | -0.33% | 106% | 109% |
| Class E | -1.03% | -0.97% | -0.75% | 106% | 119% |
| Overall | -0.43% | -0.36% | -0.35% | 106% | 112% |
| Class D | -0.24% | 0.32% | -0.48% | 106% | 112% |

Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, SI, 10–18 Jul. 2018.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-K1024, Ljubljana, SI, 10–18 Jul. 2018.