| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0518-v1 |
| --- | --- |

| Title: | Cross check of JVET-L0297 (CE8-related: CPR mode with local search range optimization) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gayathri Venugopal Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002 287 gayathri.venugopal@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany |  |  |

_____________________________

# Abstract

This document is the cross-check report of JVET-L0297 (CE8-related: CPR mode with local search range optimization). The proposed method increases the effective search range for CPR mode with local reference constraints, without increasing the memory to store reconstructed samples for CPR referencing. The obtained BD-rate gain matches with that of the proponent’s. However, noticeable differences is observed in the run-time values. The overall encoder run-time is 20% more for AI configuration when tested against VTM-2.0.1 and 6% more when tested against VTM-2.1rc1+CPR+CE8.3.1b.

# Introduction

The software package provided by the proponent was reviewed. JVET-L0297 proposes an improvement on the test CE8.3.1b [1]. In the new method, the effective search range for CPR mode with local reference constraints is increased, while it is asserted that the required memory to store reconstructed samples for CPR referencing is kept unchanged.

The proposed method are tested for all intra (AI) and random access (RA) configuration according to the test conditions in [2].

The BD-rate gain matches with that of the proponent’s. However, differences are observed in the run-time values. Especially, in the encoding run-time values. The overall encoder run-time is 21% more for AI configuration when tested against VTM-2.0.1 and 6% more when tested against VTM-2.1rc1+CPR+CE8.3.1b. Similar behavior in run-time can be noticed in RA configuration also. The overall encoder run-time is 9% more when tested against VTM-2.0.1 and 5% more when tested against VTM-2.1rc1+CPR+CE8.3.1b. The decoder run-time values are more or less the same.

# Results

The proposed method is tested against VTM (using BMS-2.0.1 as VTM-2.0.1) and VTM+CPR+CE8.3.1b (using BMS-2.1rc1 as VTM-2.1rc1+ CPR+ CE8.3.1b). The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, turbo boost off) with Scientific Linux 7.4 OS and GCC 7.2.1 compiler.

Table 1 Proposed method against BMS-2.0.1 as VTM-2.0.1 for AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.06% | 0.02% | 152% | 99% |
| Class A2 | -0.17% | -0.19% | -0.19% | 150% | 99% |
| Class B | -0.39% | -0.55% | -0.37% | 160% | 101% |
| Class C | -0.15% | -0.25% | -0.04% | 158% | 102% |
| Class E | -0.65% | -0.61% | -0.59% | 164% | 101% |
| Overall | -0.27% | -0.33% | -0.24% | 157% | 101% |
| Class D | -0.08% | 0.13% | 0.37% | 153% | 100% |
| Class F | -13.61% | -13.66% | -13.57% | 182% | 98% |
| Class TGM | -41.31% | -40.31% | -40.62% | 168% | 88% |

Table 2 Proposed method against BMS-2.0.1 as VTM-2.0.1 for RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.08% | 0.09% | 109% | 99% |
| Class A2 | -0.18% | 0.02% | -0.05% | 110% | 102% |
| Class B | -0.20% | -0.56% | -0.26% | 114% | 100% |
| Class C | 0.05% | -0.11% | -0.20% | 121% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.19% | -0.13% | 114% | 100% |
| Class D | 0.09% | -0.18% | 0.17% | 125% | 99% |
| Class F | -11.13% | -10.97% | -11.17% | 143% | 99% |
| Class TGM | -25.50% | -24.92% | -25.51% | 124% | 94% |

Table 3 Proposed method against BMS-2.1rc1 as VTM-2.1rc1+CPR+CE8.3.1b for AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM+CPR+CE8.3.1b |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | -0.03% | 105% | 99% |
| Class A2 | -0.02% | 0.02% | -0.03% | 105% | 97% |
| Class B | -0.10% | -0.14% | -0.06% | 105% | 101% |
| Class C | -0.06% | -0.05% | -0.02% | 105% | 100% |
| Class E | -0.20% | -0.24% | -0.12% | 105% | 99% |
| Overall | -0.08% | -0.09% | -0.05% | 105% | 99% |
| Class D | -0.06% | -0.02% | 0.04% | 105% | 101% |
| Class F | -2.31% | -2.21% | -2.08% | 109% | 98% |
| Class TGM | -7.88% | -7.40% | -7.48% | 107% | 97% |

Table 4 Proposed method against BMS-2.1rc1 as VTM-2.1rc1+CPR+CE8.3.1b for RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM+CPR+CE8.3.1b |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 101% | 98% |
| Class A2 | -0.02% | 0.32% | 0.03% | 100% | 98% |
| Class B | -0.08% | -0.13% | 0.02% | 101% | 94% |
| Class C | -0.02% | 0.02% | -0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.02% | -0.01% | 101% | 97% |
| Class D | -0.06% | 0.02% | -0.08% | 102% | 99% |
| Class F | -1.85% | -1.48% | -1.68% | 104% | 101% |
| Class TGM | -4.31% | -4.01% | -4.16% | 102% | 99% |

# References

[1] X. Xu, X. Li, S. Liu and E. Chai, " CE8-related: CPR mode with local search range optimization, JVET-L0297," Macau, CN, 2018

[2 ]X. Xu, K. Müller and L. Wang, " Description of Core Experiment 8: Current Picture Referencing, JVET-K1028," Ljubljana, SI, 2018.