| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0522 |
| --- | --- |

| Title: | CE4-related : simplified constructed temporal affine merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin, A. Robert, F. LeLéannec | Tel: Email: | franck.galpin@technicolor.com antoine.robert@technicolor.com fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of a simplified version of the Merge Affine mode described in JVET-K1024 [1] sub-CE 4.2.4, implemented in the JVET VTM-2.0.1. The proposed simplified Merge Affine mode adding constructed affine temporal candidates gives an average luma BD-rate gain of -0.82%/-0.52% in RA/LDB configuration. An additional test with ATMVP candidate in the affine merge mode gives an average luma BD-rate gain of -0.88%/-0.64% in RA/LDB configuration. The number of candidate comparisons is reduced to 57 compared to test 4.2.4b.

# Introduction

This contribution reports the results of a simplified version of the sub-CE 4.2.4b as described in JVET-K0124 [1], adding new constructed temporal affine merge candidates.

As described in JVET-K0267 [2], it was proposed to add constructed temporal candidates to the merge affine motion vector prediction.

The current contribution reports the results of adding these temporal candidates on top of the common base for Merge Affine from sub-CE 4.2.1, simplifying the pruning process, and adapting the coding of the affine merge flag. An additional test with ATMVP candidate in the affine merge mode is also presented.

# Description

The added candidates are described in JVET-L0256.

Additionally, the merge affine flag coding is changed as follow:

- Instead of re-using the AMVP affine flag coding which uses 3 contexts depending on affine model availability, a single independent context is added for the affine merge flag.

In the common base 4.2.1, the complexity for candidate comparison is given by:

C=(M-1)(M-2)/2+(M-1)(P-M+1)

where M is the final merge list size and P the total number of potential candidates.

In this contribution, we propose to simplify the comparison by comparing a given candidate to the last 3 candidates in the list only. The new complexity is given by:

C=3+3*(P-3)

The second test consists in moving the ATMVP in the affine merge mode, similarly to CE4.2.5b and CE4.2.8. The candidate is put at the top of the list.

# Results

## Simplified 4.2.4b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -0.78% | -1.05% | 111% | 112% |
| Class A2 | -1.05% | -0.93% | -1.20% | 113% | 111% |
| Class B | -0.72% | -0.55% | -0.64% | 111% | 105% |
| Class C | -0.63% | -0.50% | -0.61% | 110% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.82% | -0.66% | -0.83% | 111% | 107% |
| Class D | -0.92% | -0.75% | -0.93% | 111% | 111% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.35% | -0.66% | 117% | 114% |
| Class C | -0.43% | -0.22% | -0.40% | 110% | 110% |
| Class E | -0.85% | -0.89% | -0.75% | 111% | 113% |
| Overall | -0.52% | -0.44% | -0.60% | 113% | 113% |
| Class D | -0.71% | 0.22% | -0.64% | 106% | 104% |

## Simplified 4.2.4b + ATMVP candidate in affine merge

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.02% | -0.65% | -1.05% | 108% | 109% |
| Class A2 | -1.11% | -1.02% | -1.18% | 111% | 114% |
| Class B | -0.79% | -0.61% | -0.69% | 109% | 107% |
| Class C | -0.71% | -0.63% | -0.70% | 109% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.88% | -0.71% | -0.86% | 109% | 109% |
| Class D | -1.08% | -1.03% | -1.07% | 110% | 112% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.42% | -0.34% | -0.35% | 115% | 115% |
| Class C | -0.45% | -0.48% | -0.46% | 107% | 106% |
| Class E | -1.26% | -1.15% | -1.34% | 108% | 114% |
| Overall | -0.64% | -0.59% | -0.63% | 110% | 112% |
| Class D | -0.60% | -0.09% | -0.79% | 105% | 101% |

# Complexity analysis

| Complexity analysis of list construction |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max number of |  |  |  |  |  |  |
| M Merge list size | Potential candidates 4prms/6prms | candidate comparison 3+3(P-3) | MV scaling | temporal candidates | Additional local buffer | Max number of memory access | Others 4 prms/6 prms |
| 8 | 5+12/5+16 | 45/57 | 12/16 | 6/8 | 0 | 17/21 | Filtering 12/16 |

## Additional complexity analysis

As in 4.2.4, in addition to the test to reject pure translational candidates present in the common base, a test to reject spurious affine models is added, by checking the relative motion of the corners. This test is added to the common base test of pure translation can be done together after the model construction.

For each spatial candidate, the CPMV corners are tested. The complexity of the test for each list consists in:

- 8 affine model with 4 parameters model: each model requires the comparison of 2 CPMV, i.e. 8 motion vectors comparisons × 2 (pure translation testing and spurious affine model testing)

- 8 affine model with 6 parameters model: each model requires the comparison of 3 CPMV, i.e. 16 motion vectors comparisons × 2 (pure translation testing and spurious affine model testing)

The total number of individual motion vector comparisons is then 48.

# Conclusion

The original affine merge mode in VTM-2.0.1 is enhanced and simulation results reportedly show on average 0.88%/0.64% luma BD-rate gain with 9%/12% decoding time increase in RA/LDB configurations, compared to VTM-2.0.1. The complexity for candidate comparisons is similar to the one of the common base 4.2.1.

# References

[1] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1024, 11th Meeting, Ljubljana, July 2018.

[2] F. Galpin, A. Robert, T. Poirier, F. Leleannec, “Non CE4: Virtual Temporal Affine”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0267, 11th Meeting, Ljubljana, July 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).