| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0534 |
| --- | --- |

| Title: | Crosscheck of L0279: CE3-related: Unification of angular intra prediction for square and non-square blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang 8910 University Center Ln, Suit 400 San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-705-2600 yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This document reports the cross-check results for the two modifications to unify the angular intra prediction for square and non-square blocks reported in JVET-L0279. The cross-check results match the coding performance reported by the proponent.

# Introduction

The source code for JVET-L0279 [1] was provided by the proponent. The software is built based on VTM-2.0.1 and is well implemented to reflect the algorithms described in JVET-L0279. Two tests are crosschecked. In test 1, the modified values in angTable is shown in Table 1. With this modification, more integer-position samples are used as predictor so that interpolation error is further reduced.

Table 1: Modified angular direction table

| Original table | 0 | 1 | 2 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 49 | 54 | 60 | 68 | 79 | 93 | 114 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modified table | 0 | 1 | 2 | 3 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 51 | 57 | 64 | 73 | 86 | 102 | 128 |

In test 2, on top of test 1, it is proposed to keep all available angular directions within the range between bottom-left diagonal direction and top-right diagonal direction for all block sizes.

# Results Over VTM-2.0.1

The crosscheck results are illustrated in Table 1 and Table 2. The detailed results could be seen in the attached excel files.

Table 1. Results of Test 1 in L0279

Table 2. Results of Test 2 in L0279

# Conclusion

The results obtained in the cross-check match the results presented in JVET-L0279.

# Reference

- L. Zhao, X. Zhao, S. Liu, X. Li, “CE3-related: Unification of angular intra prediction for square and non-square blocks” JVET-L0279 Oct. 2018, Macau, CN.

# Patent rights declaration(s)

Kwai Inc. does not have any current or pending patent rights relating to the technology described in this contribution.