| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0505 |
| --- | --- |

| Title: | Crosscheck of JVET-L0451 (CE15-related: Palette predictor list enhancement) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This document reports the cross-check results for palette predictor list enhancement as proposed in JVET-L0451. The cross-check results match the coding performance reported by the proponent.

Introduction

The source code for JVET-L0451 [1] was provided by the proponent. The software is built based on BMS-2.0.1 and is well implemented to reflect the algorithms described in JVET-L0451.

Results Over VTM-2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.14% | 0.09% | 114% | 108% |
| Class A2 | 0.16% | 0.12% | 0.11% | 114% | 107% |
| Class B | 0.09% | 0.13% | 0.11% | 113% | 108% |
| Class C | 0.07% | 0.11% | 0.28% | 114% | 109% |
| Class E | 0.11% | 0.11% | -0.06% | 114% | 104% |
| Overall | 0.09% | 0.12% | 0.11% | 114% | 107% |
| Class D | 0.08% | 0.20% | 0.02% | 113% | 105% |
| Class F (optional) | -11.52% | -9.14% | -9.25% | 124% | 95% |
| TGM | -33.58% | -28.23% | -28.28% | 138% | 73% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.01% | 0.09% | 109% | 105% |
| Class A2 | 0.12% | 0.16% | -0.02% | 113% | 104% |
| Class B | 0.11% | 0.06% | 0.23% | 112% | 102% |
| Class C | 0.13% | 0.19% | 0.26% | 114% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.10% | 0.16% | 112% | 102% |
| Class D | 0.23% | -0.05% | 0.35% | 117% | 99% |
| Class F (optional) | -8.89% | -8.21% | -8.80% | 117% | 97% |
| TGM | -17.09% | -16.00% | -15.69% | 130% | 91% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.09% | -0.31% | 114% | 103% |
| Class C | 0.17% | 0.30% | 0.02% | 116% | 99% |
| Class E | 0.04% | 0.18% | 0.12% | 115% | 104% |
| Overall | 0.11% | 0.18% | -0.09% | 115% | 102% |
| Class D | 0.25% | 0.75% | 0.15% | 119% | 101% |
| Class F (optional) | -5.74% | -5.38% | -6.54% | 121% | 104% |
| TGM | -9.06% | -8.99% | -8.32% | 127% | 97% |

Conclusion

The results obtained in the cross-check match the results presented in JVET-L0451.

Reference

- J. Ye, X. Li, X. Xu, S. Liu, “CE15-related: Palette predictor list enhancement” JVET-L0451 Oct. 2018, Macau, CN.

Patent rights declaration(s)

Alibaba Group does not have any current or pending patent rights relating to the technology described in this contribution.