| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 08–12 Oct. 2018 | Document: JVET-L0556 |
| --- | --- |

| Title: | Crosscheck of JVET-L0307 on CE15-related: Palette index map scan order constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Junghak Nam 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution presents the crosscheck results of JVET-L0307 contribution, which match those provided by the proponents.

Introduction

The Table 1 shows the crosscheck results of JVET-L0307. The coding performance matches those provided by the proponent. The encoding and decoding are slightly differences about 1~2%.

Table 1. Simulation results of JVET-L0307

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.14% | 0.07% | 117% | 103% |
| Class A2 | 0.17% | 0.14% | 0.13% | 115% | 104% |
| Class B | 0.09% | 0.11% | 0.11% | 115% | 104% |
| Class C | 0.08% | 0.13% | 0.31% | 115% | 111% |
| Class E | 0.11% | 0.10% | -0.05% | 116% | 105% |
| Overall | 0.09% | 0.12% | 0.12% | 115% | 106% |
| Class D | 0.08% | 0.18% | -0.01% | 114% | 110% |
| Class F | -11.50% | -9.23% | -9.23% | 125% | 93% |
| SCC | -33.45% | -28.17% | -28.22% | 138% | 72% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.03% | 0.05% | 110% | 104% |
| Class A2 | 0.12% | 0.13% | 0.07% | 113% | 104% |
| Class B | 0.13% | 0.07% | 0.24% | 115% | 105% |
| Class C | 0.14% | 0.18% | 0.20% | 118% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.10% | 0.16% | 114% | 106% |
| Class D | 0.25% | 0.01% | 0.36% | 120% | 115% |
| Class F | -8.87% | -8.37% | -8.80% | 121% | 105% |
| SCC | -16.87% | -15.88% | -15.59% | 130% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | 0.20% | -0.18% | 115% | 105% |
| Class C | 0.18% | 0.23% | -0.10% | 117% | 106% |
| Class E | 0.03% | 0.07% | 0.02% | 115% | 105% |
| Overall | 0.11% | 0.18% | -0.11% | 116% | 105% |
| Class D | 0.26% | 0.74% | 0.12% | 121% | 119% |
| Class F | -5.63% | -5.04% | -6.36% | 117% | 107% |
| SCC | -9.12% | -9.05% | -8.50% | 124% | 98% |