| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0561-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-L0381 (CE3-related: 4-tap interpolation filter selection with quantization parameter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this document, a crosscheck result of 4-tap interpolation filter selection with quantization parameter, proposed by KDDI, is reported. In the proponent's document, one of two different interpolation filters, 4-tap cubic filter and 4-tap Gaussian filter, is selected by the value in a table, which is pre-defined by the quantization parameter and the block size. It is reported that the proposed method achieved 0.40% and 0.14% in AI and RA conditions, with QPs defined in CTC. It is confirmed that the reported result of the proponent and the result from the crosschecker closely matches.

Crosscheck result

The proposed method has been implemented using BMS-2.0.1 as software base with VTM configurations according to the common test conditions defined in JVET-K1010 [2] and Core Experiment description in JVET-K1023 [3].

Table 1 Crosscheck results with VTM configurations

|  | All Intra Main10 – VTM configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 AI (VTM config.) |  |  | Over BMS-2.0.1 RA (VTM config.) |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.03% | -0.13% | -0.14% | -0.07% | -0.22% | -0.11% |
| Class A2 | -0.16% | -0.25% | -0.34% | -0.06% | -0.30% | -0.32% |
| Class B | -0.39% | -0.58% | -0.55% | -0.14% | -0.45% | -0.37% |
| Class C | -0.68% | -1.03% | -0.96% | -0.24% | -0.30% | -0.35% |
| Class E | -0.63% | -0.62% | -0.87% |  |  |  |
| Overall | -0.40% | -0.56% | -0.59% | -0.14% | -0.33% | -0.30% |
| Class D | -0.50% | -0.56% | -0.90% | -0.23% | -0.69% | -0.22% |
| Enc Time [%] | 103% |  |  | 101% |  |  |
| Dec Time [%] | 101% |  |  | 101% |  |  |

Conclusion

The experimental results from the proponent and the crosschecker are almost identical from each other. The crosscheck is confirmed. In some results in RA configuration, there are very minor (0.0001) difference in PSNR values, but it comes from the different method of the calculation of hexadecimal PSNR values to double type.

References

- Y. Kidani, K. Kawamura, K. Unno, S. Naito, “CE3-related: 4-tap interpolation filter selection with quantization parameter,” Document of Joint Video Experts Team (JVET), JVET-L0381, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team (JVET), JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” Document of Joint Video Experts Team (JVET), JVET-K1023, 11th Meeting: Ljubljana, SI, 10–18 July 2018

Patent rights declaration(s)

LG Electronics Inc. does not have any current or pending patent rights relating to the technology described in this contribution.