| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0575-v2 |
| --- | --- |

| Title: | CE4-related: CTU-level Initialization of History-based Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiwei Xu Haitao Yang Yin Zhao Jianle Chen | Tel: Email: | +8613758193753 xuweiwei3@huawei.com haitao.yang@huawei.com yin.zhao@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of CTU level initialization of history-based motion vector prediction. In this test, the initialization of history-based motion vector candidates list are performed in CTU level. When starting encoding or decoding a new CTU or current CTU locates at the start of a CTU line, the history-based motion vector candidates list is initialized. Compared to CE4.4.7 anchor of history-based method, the average BD-rate is reportedly 0.01%~0.03% in RA, respectively.

Introduction

History-based Motion Vector Prediction (HMVP) method was introduced In JVET-K0104 [1]. A HMVP candidate is defined as the motion information of a previously coded block. Then a list with multiple HMVP candidates can be maintained during the encoding/decoding process as shown in figure 1. HMVP method [1] is performed:

Figure 1. HMVP procedure

1, The HMVP list is emptied when a new slice is encountered.

2, Find next CU in current CTU of current slice.

3, The HMVP is inserted into merge candidate list or AMVP candidate list.

4, Decode current block.

5, Update HMVP list using MV of current CU, then go to step 2

So the HMVP have heavy dependency on the CU structure of last CTU line and previous CTU, the paralleling process of the CTU lines are seriously influenced.

In this contribution of CTU level initialization of history-based MVP (HMVP) method, The HMVP list is initialized using mv of spatial neighbouring blocks when a new CTU is encountered as shown in figure 2.

1, Find next CTU in current slice.

2, The HMVP list is initialized.

3, Find next CU in current CTU.

4, The HMVP is inserted into merge candidate list or AMVP candidate list.

5, Decode current block.

6, Update HMVP list using MV of current CU

Figure 2. initialize HMVP list at CTU level

Two kinds of tests are performed:

Test1 and Test2: When a new CTU start, the HMVP list is emptied. And then the spatial neighboring block of current CTU are added into HMVP list from top to bottom, and from left to right interleaved as shown in figure 3. Pruning is performed to avoid duplicated HMVP. The different between test1 and test2 lines in the interval of selecting spatial neighbour.

Test3 and Test4: When a new CTU line start (current CTU is the first CTU in a CTU line), the HMVP list is emptied. And then the spatial neighboring block of current CTU are added into HMVP list from top to bottom, and from left to right interleaved as shown in figure 3. Pruning is performed to avoid duplicated HMVP. The different between test3 and test4 lines in the interval of selecting spatial neighbour.

Figure 3. spatial neighouring blocks at CTU level

Simulation results

The contribution has been implemented on top of CE4.4.7. The test condition descripted in JVET-K1010 [2] is used for verifying the contribution. The summary results when history-based candidate is initialized in CTU level are provided below.

Test1: The summary results when neighboring MV are selected with the interval of 8 are provided in Table 1, the BD-rate differences are 0.03% in RA, respectively.

Table 1 Coding performance over CE4.4.7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.10% | 0.15% | 100% | 100% |
| Class A2 | 0.00% | -0.08% | -0.04% | 100% | 100% |
| Class B | 0.04% | 0.06% | -0.02% | 100% | 99% |
| Class C | 0.02% | -0.07% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.01% | 0.01% | 100% | 100% |
| Class D | -0.03% | -0.21% | 0.01% | 100% | 100% |
| Class F (optional) | -0.01% | 0.04% | 0.05% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.09% | -0.38% | 99% | 98% |
| Class C | -0.03% | -0.15% | -0.37% | 100% | 96% |
| Class E | 0.08% | -0.06% | 0.07% | 100% | 98% |
| Overall | 0.02% | -0.03% | -0.27% | 100% | 98% |
| Class D | 0.01% | 0.58% | 0.14% | 101% | 102% |
| Class F (optional) | 0.20% | 0.34% | 0.14% | 100% | 99% |

Test2: The summary results when neighboring MV are selected with the interval of 16 are provided in Table 2, the BD-rate differences are 0.03% in RA, respectively.

Table 2 Coding performance over CE4.4.7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.15% | 0.09% | 100% | 99% |
| Class A2 | -0.02% | -0.11% | -0.07% | 100% | 100% |
| Class B | 0.03% | 0.09% | 0.13% | 100% | 98% |
| Class C | 0.03% | -0.08% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.02% | 0.03% | 100% | 99% |
| Class D | -0.06% | -0.05% | -0.02% | 100% | 100% |
| Class F (optional) | 0.02% | 0.04% | 0.05% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | -0.01% | 99% | 98% |
| Class C | 0.02% | -0.01% | -0.07% | 100% | 95% |
| Class E | 0.04% | 0.43% | 0.57% | 100% | 98% |
| Overall | 0.02% | 0.10% | 0.12% | 100% | 97% |
| Class D | 0.01% | -0.07% | 0.12% | 101% | 100% |
| Class F (optional) | 0.03% | 0.39% | -0.43% | 100% | 99% |

Test3: The summary results when reinitialize HMVP list only at the beginning of a CTU line (current CTU is the first CTU in a CTU line), and neighboring MV are selected with the interval of 8 are provided in Table 3, the BD-rate differences are 0.01% in RA, respectively.

Table 3 Coding performance over CE4.4.7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | 0.00% | 99% | 100% |
| Class A2 | -0.03% | -0.08% | -0.02% | 100% | 100% |
| Class B | 0.02% | 0.08% | 0.13% | 99% | 99% |
| Class C | 0.01% | -0.07% | 0.03% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | 0.05% | 99% | 99% |
| Class D | -0.01% | -0.11% | -0.03% | 100% | 98% |
| Class F (optional) | 0.01% | 0.02% | 0.02% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Test4: The summary results when reinitialize HMVP list only at the beginning of a CTU line (current CTU is the first CTU in a CTU line), and neighboring MV are selected with the interval of 16 are provided in Table 4, the BD-rate differences are 0.01% in RA, respectively.

Table 4 Coding performance over CE4.4.7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.06% | -0.01% | 100% | 99% |
| Class A2 | -0.02% | 0.02% | 0.01% | 100% | 100% |
| Class B | 0.02% | 0.07% | 0.09% | 99% | 99% |
| Class C | 0.01% | 0.00% | -0.01% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.03% | 99% | 99% |
| Class D | 0.02% | -0.19% | 0.03% | 100% | 98% |
| Class F (optional) | 0.01% | 0.00% | 0.03% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Conclusions

This contribution reports the results of CTU-level initialization of history-based motion vector prediction candidate list. Compared to history based MVP, the average BD-rate differences are about 0.01%~0.03% in RA, respectively.

We recommend to study the proposed method in CE.

References

- Li Zhang, et al., “CE4-related: History-based Motion Vector Prediction”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0104, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010, July 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).