| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L00583 |
| --- | --- |

| Title: | Crosscheck of JVET-L0357 (CE14 related: Adaptive colour space clipping filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura | Tel: Email: | +82-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports crosscheck of JVET-L0357 on adaptive colour space clipping filter. The results matched with those provided by the proponent. The software provided by the proponent was reviewed and it is confirmed that the implementation is clearly matched with what the proponent proposed in JVET-L-357.

Simulation results

Test results for RA configuration is shown in Table 1. All the BD-rate results are matched with those provided by the proponent. For encoding/decoding complexity, similar tendency to the provided results by the proponent is observed.

Table 1. BD-rate results of random access configuration

|  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- |
|  |  | psnrY | psnrU | psnrV |
| HDR-A | DayStreet | 0.0% | -0.1% | 0.0% |
|  | FlyingBirds | 0.0% | 0.0% | 0.0% |
|  | PeopleInShoppingCenter | 0.0% | -0.1% | -0.6% |
|  | SunsetBeach | 0.0% | -0.5% | -1.7% |
| HDR-B | Market | 0.0% | 0.0% | 0.0% |
|  | SunRise | 0.0% | 0.0% | 0.0% |
|  | ShowGirl2 | 0.0% | -5.2% | -3.8% |
|  | BalloonFestival | 0.0% | -0.4% | 0.0% |
|  | Hurdles | 0.0% | 0.0% | 0.0% |
|  | Starting | 0.0% | 0.0% | 0.0% |
|  | Cosmos1 | 0.0% | -10.9% | -6.3% |
|  | Average HDR-A | 0.0% | -0.2% | -0.6% |
|  | Average HDR-B | 0.0% | -2.4% | -1.4% |
|  | Average all | 0.0% | -1.6% | -1.1% |
|  | Enc Time[%] | 93% |  |  |
|  | Dec Time[%] | 99% |  |  |

Patent rights declaration(s)

NHK does not have any current or pending patent rights relating to the technology described in this contribution.