| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0590 |
| --- | --- |

| Title: | Crosscheck of CE4.4.12 combined with CE4.4.7a |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jicheng An Wangjing Block A, Alibaba Center No. 9th Wangjing East Garden 4th Area, Chaoyang District, Beijing | Tel: Email: | +86-13439745728 jicheng.ajc@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This contribution presents the crosscheck results of CE4.4.12 combined with CE4.4.7.a. The finished partial results match those provided by the proponents.

Introduction

Test CE4.4.12.c is combined test with CE4.4.12.b and CE4.4.7.a.

Test CE4.4.12.d is combined test with CE4.4.12.b that the pair count reduced from 6 pairs to 3 pairs and CE4.4.7.a.

The Table 1 and Table 2 shows the crosscheck results of CE4.4.12.c and CE4.4.12.d respectively. The MaxNumMergeCand is set to 8. The finished partial results match those provided by the proponent.

Table 1. Crosscheck results of CE4.4.12.c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.77% | -0.70% | -1.02% | 98% | 100% |
| Class A2 | -0.96% | -1.18% | -1.30% | 98% | 98% |
| Class B | -1.18% | -1.04% | -1.22% | 99% | 98% |
| Class C | -0.71% | -0.64% | -0.88% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.93% | -0.89% | -1.11% | 99% | 98% |
| Class D | -0.45% | -0.72% | -0.69% | 99% | 87% |
| Class F (optional) | -0.59% | -0.66% | -0.66% | 98% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over vtm2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.16% | -0.06% | -0.20% | 100% | 96% |
| Class E | -0.06% | 0.21% | 0.14% | 100% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | 0.31% | -0.28% | 101% | 91% |
| Class F (optional) | -0.84% | -0.41% | -0.74% | 99% | 98% |

Table 2. Crosscheck results of CE4.4.12.d

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.78% | -0.74% | -1.04% | 98% | 99% |
| Class A2 | -0.96% | -1.18% | -1.35% | 98% | 99% |
| Class B | -1.17% | -1.09% | -1.22% | 98% | 97% |
| Class C | -0.69% | -0.64% | -0.90% | 99% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.92% | -0.92% | -1.12% | 98% | 97% |
| Class D | -0.46% | -0.75% | -0.72% | 101% | 88% |
| Class F (optional) | -0.59% | -0.62% | -0.65% | 101% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over vtm2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.16% | -0.17% | -0.12% | 103% | 101% |
| Class E | 0.00% | 0.20% | -0.18% | 104% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.03% | 0.47% | -0.06% | 103% | 93% |
| Class F (optional) | -0.86% | -0.92% | -0.50% | 102% | 96% |